3GPP TSG-SA3 Meeting #119	S3-244679

Orlando, US, 11 - 15 November 2024

# Source:	Cisco Systems

# Title:	Address ENs in Solution #8: Supporting all 5G SBA certificate types

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approval of update addressing ENs in Solution 8 of TR 33.776

# 2	References

[1]	3GPP TR 33.776 (V0.5.0), "Study of Automated Certificate Management Environment (ACME) for the Service Based Architecture (SBA)".

[2]	IETF RFC 8555: "Automatic Certificate Management Environment (ACME)".

[3]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF) ".

# 3	Rationale

This contribution proposes updates that address the Editor's Notes (ENs) in Solution #8 in TR 33.776 [1].

The solution details include an EN that whether wildcard support is required or possible with http-01 or alpn-01 challenges is FFS. These challenges are used by some solutions, but solutions exist for all key issues that do not require use of these challenges. Therefore, the EN is removed, and text is added stating solutions that require use of these challenge types will need to assess whether wildcard support is required or possible.

The evaluation includes an EN stating that support for additional certificate profiles is FFS. No additional certificate profiles have been identified; therefore, this EN is removed.

# 4	Detailed proposal

* * * Start of change * * *

## 6.8	Solution #8: Supporting all 5G SBA certificate types

### 6.8.1	Introduction

This solution addresses Key issue #7: Supporting all 5G SBA certificate types.

### 6.8.2	Solution details

ACME [2] was designed for the provisioning and management of TLS/SSL certificates for web servers; however, it was also designed to be easy to extend and extensions beyond web server TLS exist. These extensions can be used to support a wide range of certificate profiles. Additional extensions are possible, as is evident in other solutions in this study.

TS 33.310, clause 6.1.3c [3], profiles the certificates to be used for 5GC SBA. These end entity certificates may be used for the following purposes:

- 	TLS client and server certificates

- 	Signing validation of OAuth 2.0 access tokens

- 	Signing validation of CCA (JWT based authentication) tokens

According to TS 33.310, different end entity certificate profile requirements may be applied to intra-domain and inter-domain SBA for NF producers, NF consumers, NRF instances, Service Communication Proxy (SCP) nodes, and Security Edge Protection Proxy (SEPP) nodes. However, certificate management for the external interface of the SEPP is out of scope for this study.

TS 33.310, clause 6.1.3c.2 [3], lists	general SBA certificate profile requirements. These are limited support for X.509 version 3 certificates according to RFC 5280 [18] and recommended support for ECDSA for end entity certificates. ACME supports the issuance of X.509 version 3 certificates. When using ACME to request a certificate, an ACME client can specify the type of key, including an ECDSA key.

TS 33.310, clause 6.1.3.c.3 [3], covers NF certificate profiles. It states that end entity certificates are directly signed by the CA in the operator domain in which the entity exists. This is true for the solutions in this study that deal with certificate issuance.

NF TLS client and server certificate profiles are described in detail in Table 6.1.3c.3-1 [3], and NF OAuth 2.0 Access Token and CCA Token certificate profiles are described in Table 6.1.3c.3-2 [3]. The solutions in this study that deal with challenge validation and certificate issuance use the NF Instance ID to uniquely identify the NF. This NF Instance ID can be included in the subjectAltName in TLS client and server certificates and in X.509 PKIX certificates used for signing validation of OAuth 2.0 JWT access tokens and CCA tokens, as specified in TS 33.310 [3]. Where an FQDN or IP addresses are part of the NF profile, these will need to be validated too.

Use of http-01 or tls-alpn-01 challenges in a solution (e.g., Solution #1) can address both FQDN and IP address cases; dns-01 cannot be used for IP addresses per RFC 8738 [19].

For certificates requiring multiple identifiers in the subjectAltName extension, these identifiers can be included in the set of NF profile parameters signed by the OAM and included in the Authority Token provided by the OAM to the NF. This includes IP addresses and wildcarded domains, though the latter is not recommended in TS 33.310, clause 6.1.3c.3 [3]. Whether wildcard support is required or possible with these challenges will be addressed within solutions that use those challenges.

TS 33.310, clause 6.1.3c.4 [3], covers SCP certificate profiles. There are no additional requirements beyond those already covered in the previous clauses.

TS 33.310, clause 6.1.3c.5 [3], covers SEPP certificate profiles. For the internal interfaces of the SEPP, there are no additional requirements beyond those already covered in the previous clauses. The external interfaces of the SEPP are out of scope for this study.

### 6.8.3	Evaluation

Any protocol intended for use for automated certificate management for 5GC SBA needs to be capable of being used to support all certificate profiles in 5GC SBA. TS 33.310, clause 6.1.3c [3], defines the certificate profiles for 5GC SBA. X.509 version 3 certificates are used for all entities in 5GC SBA. ACME supports X.509 version 3 certificates and the necessary extensions.

* * * End of change * * * *