3GPP TSG-SA3 Meeting #119	S3-244711

Orlando, US, 11 - 14 November 2024

# Source:	Apple

# Title:	Discussion paper for CT1 LS on SUCI calculation failure

# Document for:	discussion

Agenda Item:	3

# 1	Decision/action requested

It is requested to discuss the discussion paper on SUCI calculation failure

# 2	References

[1] C6-240155 - CR 1030 on TS 31.102 Correction of GET IDENTITY in case of incorrect configuration

[2] C1-245039 - LS on SUCI Calculation Failure

# 3	Rationale

This discussion paper proposed the analysis on the SUCI calculation failure and proposes the way forward.

# 4	Detailed proposal

## 4.1 Background

### 4.1.1 SUCI calculation failure highlighted in CT6 CR- C6-240155 and CT1 LS to SA3

In the CR 1030 C6-240155[1], CT6 highlighted an issue about SUCI calculation failure.

SUCI context shall be supported if "SUCI calculation is to be performed by the USIM" (i.e. Service n°124 and Service n°125 are "available").

For the execution of the command, the following information shall be available in the USIM:

-	Home network identifier (i.e. MCC and MNC when SUPI Type is IMSI or domain name when SUPI Type is Network Specific Identifier, Global Line Identifier or Global Cable Identifier) (see NOTE).

-	Routing indicator (configured in EFRouting_Indicator).

-	Home network public key (see Note).

-	Home network public key identifier (see Note).

-	Protection scheme identifier (see Note).

-	SUPI.

NOTE:	Provision and storage of the information in the USIM when "SUCI calculation is to be performed by the USIM" (i.e. Service n°124 and Service n°125 are "available") is out of the scope of the specification.

When Service n°124 is "available", and Service n°125 is not "available", (SUCI to be calculated in ME), however the information required (listed above) for the SUCI Calculation is not correctly configured, CT6 requested that “In case the information required for the SUCI calculation as listed above is not correctly configured in the USIM, the USIM shall return an error status word as described in Section 7.3.2.”

CT1 could not reach consensus on how to handle this issue, therefore sent a LS to SA3 [2] to discuss this topic and provide feedback how the UE should handle the described SUCI calculation failure.

### 4.1.2 Observation in implementations

This is a rather rare failure case, when SUCI computation failure happens in the 5G USIM, an ‘error status word’ is returned to ME. However, when there is no defined behaviour for the ME when there is no SUCI or necessary parameters returned from USIM.

There are different solutions to address this issue:

- Apply NULL scheme for SUCI generation failure

UE would send SUPI/IMSI in plaintext anyhow in legacy RAT, so it is straightforward that UE just use null-scheme in the SUCI calculation failure case. However, based on the network implementation, it may result in an “overreaction” of the network - sending cause #3 "illegal UE” which deactivates the UE for both PS and CS services until the next power cycle. Besides, 5G is supposed to protect the SUPI especially when Service n°124 is "available".

In TS 33.501, clause 6.12.2, there is requirements on UE/ME calculating null scheme SUCI (plain text SUPI):

--------------------------------------------------------

The UE shall generate a SUCI using "null-scheme" only in the following cases:

- if the UE is making an unauthenticated emergency session and it does not have a 5G-GUTI to the chosen PLMN, or

- if the home network has configured "null-scheme" to be used, or

- if the home network has not provisioned the public key needed to generate a SUCI.

The ME shall calculate the SUCI using the null-scheme if the Home Network Public Key or the priority list are not provisioned in the USIM.

--------------------------------------------------------

Observation 1: Using null-scheme when SUCI calculation failure occurs is neither aligned with current security requirements nor with the 5G privacy preserving feature, and will expose user’s privacy in 5G.

- Consider “USIM as invalid for 5GS services”

This option disables PS Services across all RATs until power down, i.e. the UE could also no longer register in LTE were plain text IMSI is used and could only register to CS domain in 2G/3G. This was considered as too drastic and leads to severe unavailability condition for UE.

- Disable N1 until power down

This option still allows PS services on legacy RATs. In the current 24.501 definition we do not foresee a permanent N1 mode disabling which would be applicable for all PLMNs, however this could be added and is technically feasible.

As an optimization NR might be re-enabled if after a SIM INIT or SIM refresh the UE concludes that SUCI calculation work fine. It is left to UE implementation whether and how the UE should notify the users on disabling N1.

Observation 2: Disabling N1 mode is the recommended approach, without impacting user service and compromising 5G privacy.

# 5	Conclusions and proposals

Based on observations made in this paper, it is proposed that:

Proposal: the option 3) above should be endorsed as the way forward, S3-244712 (draft LS) should be taken as the baseline to reply to CT1.