3GPP TSG-SA3 Meeting #119	S3-244714

Orlando, US, 11 - 14 November 2024

# Source:	Apple

# Title:	Discussion paper on LTM security way forward

# Document for:	discussion

Agenda Item:	4.12

# 1	Decision/action requested

It is requested to discuss the discussion paper on SUCI calculation failure

# 2	References

[1] S3-244324 - Living document on NR mobility enhancement

[2] S3-244316 - LS on security handling for inter-CU LTM

# 3	Rationale

This discussion paper proposed the way forward for LTM security.

# 4	Detailed proposal

## 4.1 Background

SA3 has been working on the LTM (Layer1/Layer 2 Triggered Mobility) security feature for a few meetings, generating a living document [1] with different kind of solutions addressing the key handling for inter-CU LTM based cell switch scenario.

In SA3#118 meeting, after a show of hands, there were majority companies prefer to use RRC based solutions due to the security considerations. However, there are some companies supporting other approaches, e.g. MAC CE based solutions, solutions or handovers do not requiring a PDCP anchor change, sending NCC value per candidate cell via RRC message during LTM preparation phase. SA3 sent LS [2] to RAN2 on those options and request further preference from RAN2.

Proposal 1: SA3 should leave the final decision to RAN2, as they are the leading group for the LTM feature, and they should decide on the balance of security and efficiency.

SA3 proposed to continue studying unsecure MAC CE based solution, which is good for not only R19, but also for future releases. MAC CE is carrying a lot of information (refer TS 38.321), but without any protection until R19. The study on protecting MAC CE is significant and non-neglectable. Considering the impact on the current system, if there will not be agreement to implement MAC CE protection in R19, it still can be the basis for 6G network security.

Proposal 2: SA3 should keep working on the MAC CE protection, even though it may bring impact to the current network, but the study could be taken as the basis for the future releases, e.g. 6G, when considering the enhancement of the whole system.

# 5	Conclusions and proposals

Proposal 1: SA3 should present the security evaluation to RAN, and then leaves the final decision to RAN2, as they are the leading group for the LTM feature, and they should decide on the balance of security and efficiency.

Proposal 2: SA3 should keep working on the MAC CE protection, even it may bring impact to the current network, but the study could be taken as the basis for the future releases, e.g. 6G, when considering the enhancement of the whole system.