3GPP TSG-SA3 Meeting #119	S3-244798

Orlando, US  11 - 15 November 2024

# Source:	Nokia

# Title:	Updates to conclusions for Key Issue #1

# Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

It is required to approve this contribution for updates to conclusions for Key Issue #1.

# 2	References

[1]	3GPP TR 33.745 “Study on security aspects of 5G NR Femto”

# 3	Rationale

The contribution proposes updates for conclusions for key issue #1 to align with LS response from SA2 (S2-2411241) regarding the terminology alignment for Femto ownership. Also, optional inclusion of multi-factor authentication is proposed here.

# 4	Detailed proposal

# * * * * First change* * * *

## 7.1	Key Issue #1: Security of 5G NR Femto Ownership

It is agreed to consider the following principles for the normative work:

-	The security aspects of NEF specified in clause 12 of TS 33.501[4] can be reused.

-	The Femto owner/administrator is authenticated and authorized by the 5GC to operate (add/delete/modify) on the allowed CAG list of the users.

- 	SA2 defined term “NR Femto hosting party” which is similar to “H(e)NB Hosting Party” defined in TS 22.220 will be used during the normative phase. Also, as clarified by SA2, “NR Femto hosting party” plays the role of “CAG owner”.

NOTE: Considering above terminology, the concepts of “H(e)NB Hosting Party Authentication” defined in TS 33.320 [2] can be re-used for “NR Femto hosting party authentication”. Furthermore, multi-factor authentication based solutions may be considered as optional implementation alternatives for “NR Femto hosting party authentication”.

# * * * * End of changes * * * *