3GPP TSG-SA3 Meeting #119	S3-244832

Orlando, US, 11 -15 November 2024

# Source:	Huawei, HiSilicon

# Title:	solution #2 updates to address ENs

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-22

# 2	References

[1]	3GPP TR 33.700-22

# 3	Rationale

This contribution updates the solution #2 to resolve the following ENs.

Editor’s Note: the need for Authentication between a ROF and an AzF in this solution is ffs.

Editor’s Note: whether client certificate can be used here is ffs.

For the first EN, the mutual authentication between the RO Function and the CCF (or AzF) is essential to establish the secure channel providing message integrity/confidentiality/replay protection and resistance to attackers’ tampering.

For the second EN, the solution is revised not to mandate using client certificate to provide flexibility, as the RO can be a human user.

# 4	Detailed proposal

pCR

***  START OF CHANGES  ***

## 6.2	Solution #2: CAPIF-8 reference point security

### 6.2.1	Introduction

This solution addresses "Key Issue #1.1: CAPIF-8 reference point ".

The resource owner function (ROF) interacts with the authorization function (AzF) in the CAPIF core function through the CAPIF-8 reference point. This solution proposes mutual authentication between the ROF and AzF. Besides, the messages exchanged between them are protected with integrity protection, replay protection and confidentiality protection.

### 6.2.2	Solution details

#### 6.2.2.1	Mutual authentication

For authentication between a ROF and an AzF, mutual authentication based on TLS is proposed. The server is authenticated by the server certificates. The certificate profiles follow the TS 33.310 [6], clause 6.1.3a. The ROF authentication can be based on the ROF certificate, the pre-shared key or password etc., which is left for implementation.

NOTE: The structure of the PKI used for the certificate is out of scope of the present document.

#### 6.2.2.2	Protection of messages between ROF – AzF

TLS is used to provide integrity protection, replay protection and confidentiality protection for the CAPIF-8 interface.

The security profiles for TLS implementation and usage follow the provisions given in clause 6.2 of TS 33.210 [7].

### 6.2.3	Evaluation

TBD

***  END OF CHANGES  ***