3GPP TSG-SA3 Meeting #119	S3-244861

Orlando, US  11 – 15 November 2024													   revision of

# Source:	Nokia

# Title:	Discission paper concerning RAN agreements.

# Document for:	Discussion

Agenda Item:	3.1

# 1	Decision/action requested

Discussion concerning RAN agreements – reply concerning feasibility.

# 2	References

[1]				3GPP TR 38.769: "Study on solutions for ambient IoT (Internet of Things)".

# 3	Rationale

This discussion paper aims to describe some of the assumption and agreements drawn by RAN2 influencing the solution principles and assumptions in SA3. It’s based on TR 38.769 [1] and the LS R2-2406150. The discussion paper aims to provide the basis for the feedback requested in LS R2-2406150 and proposed in LS reply S3-244864.

.

Figure 1 Current overall AS procedures between A-IoT device and reader from TR 38.769

As seen from Figure 1, there are three steps included within the RAN2 defined AIoT procedure.

- Step A. AIoT paging, being a single message from the reader to the device triggering the procedure for one or multiple devices. This message may carry identity of a single, multiple, or group of devices.

- Step B. Random access procedure, serving the purpose of providing access to the resources using either contention based random access, or contention free access procedures, as well as providing the full device ID to the AIoTF or any other upper layer data.

  - Note: the size of the full device ID is longer than 96 bits as of SA2 agreements

  - Note: step B may contain “upper layer data” as per RAN2 agreement in the 3rd step of the 3-step contention based random access (CBRA)

- Step C. Command procedure where the reader requests the device to either read or write.

In the following section, we look at the procedures studied in Step B as they relate to connection establishment prior to the command procedures.

## 3.1	Contention based random access (CBRA)

For the RAN2 defined inventory procedure, three access designs is currently studied; 2-step CBRA, 3-step CBRA, and contention free ‘random’ access (CFRA). Either of these procedures are preceded by the AIoT paging which may target one or more devices. In the examples below describing the 2 and 3 step CBRA, the notation of “Full device ID” is an example of “upper layer data”.

### 3.1.1	2-step CBRA

The 2-step CBRA uses two steps to resolve access from the device to the reader. Firstly, the device selects one of the access occasions provided by the paging at random and then in this occasion transmits the full device ID or any other upper layer data. In the first step, potentially a shorter, random identifier is piggy backed to the device ID or any upper layer data to provide the contention resolution i.e. reader responds with the random identifier to indicate which Device ID or upper layer data has been received correctly.

### Observation 1: The device sends the upper layer data in the first message.

### 3.1.2	3-step CBRA

For 3-step CBRA, an extra message is added in order to first provide a short random identifier to the reader which is then used for contention resolution before the full device ID or any upper layer data is transmitted to the Reader.

Observation 2: The procedure sends the upper layer data after the contention resolution leaving room for a two handshake.

### 3.1.3	Contention free Access

The intention of the contention free access is to allow the reader to provide the device with a fixed resource to access, thus eliminating the need for the random resource selection part of the procedure. As of TR 38.769 [1] the current design skips the contention resolution described in either of 3.1.1 or 3.1.2, however an ACK/NACK is still expected. RAN2 has not yet concluded on this topic, but it may be expected that the random identifier can simply be omitted by contention free access.

## 3.2	Data transmission size

As of a recent LS to RAN2 from RAN1 in R2-2409405, RAN1 defines the maximum data block size to be around 1000 bits, depending on various factors such as target coverage/data rate, energy consumption/device availability, etc.

As per RAN2 agreement in RAN2#127 segmentation is supported at least for the D2R direction, but the procedure should be applicable for R2D procedure as well if needed.

Observation 3: The message size is limited to 1000 bits for R2D but in theory unlimited for D2R.

## 3.3	Security aspects

## The 2-step CBRA proposes to send upper layer data as part of the random access procedure in step 1. This requires that the security context has been established such data can be protected. As agreed in KI#4 and KI#5, the system shall provide the means to authenticate, either one way or mutual, confidentiality, integrity and replay protect and protect the device ID, this doesn’t imply that all security primitives shall be enabled, but the design shall provide the means to do it when required by the deployment. As the 2-step approaches sends the upper layer data as part of the first message after paging, it seems not feasible to establish the security context only utilising the paging message. Same applies for the contention free access.

The 3-step CBRA approach enables a 2-way handshake prior to sending upper layer data, which makes it feasible to create a security context fulfilling all the security primitives required by the KI’s prior to sending the upper layer data.

Proposal 1: Inform RAN2 about the security observation concerning the 2-step and 3-step CBRA.

Another observation is the assignment of a random identifier as part of both the 2 and 3-step CBRA to identify the device when sending the contention resolution. As the selection of the identifier might introduce the security treats described in KI#3, RAN2 should be aware of this threat.

Proposal 2: Inform RAN2 that the identifier used during contention resolution shall be selected randomly i.e. not being related to the real identifier.

RAN2 ask SA3 for advice concerning the feasibility of the “command only” use cases, Furthermore, RAN2 assume that the inventory + command is sufficient to support the command-only use cases. As sending a command message securely requires a security establishment, which is currently being proposed as part of the inventory use case, it doesn’t from a security viewpoint make sense to omit the inventory step, as it will omit the security establishment too.

Proposal 3: Inform RAN2 that omitting the inventory step will omit the security establishment and hereby create a vulnerability in the system.

**********   END OF CHANGES