3GPP TSG-SA3 Meeting #119	S3-244926

Orlando, US, 11-15 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | DRAFT | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Public key or cert distribution for Access Token verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_eSBA_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Problem described in S3-244202 is not complete See discussion paper S3-244919 This draft CR illustrates one of the options of the discussion paper. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The public key used to verify the access token signature can be provided by the NRF in an X.509 certificate through a new service or a new operation of an existing service. Clarify that NFp need verify that the access token is issued by the OAuth 2.0 authorization server. The issuer can be the NRF that the NFp registered at, or the NRF that is not registered at but locally configured as authorized OAuth 2.0 authorization server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel.15-18 established mechanims (operator scope / implementation specific) continue to be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF CHANGES

#### 13.4.1.0	General

The authorization framework described in clause 13.4.1 allows NF Service Producers to authorize the requests from NF Service requestors. Subscription requests are also service requests.

The authorization framework uses the OAuth 2.0 framework as specified in RFC 6749 [43]. Grants shall be of the type Client Credentials Grant, as described in clause 4.4 of RFC 6749 [43]. Access tokens shall be JSON Web Tokens as described in RFC 7519 [44] and are secured with digital signatures or Message Authentication Codes (MAC) based on JSON Web Signature (JWS) as described in RFC 7515 [45].

NOTE 1a: Securing the access token using Message Authentication Codes (MAC) based on JSON Web Signature (JWS) as described in RFC 7515 [45] requires a pairwise pre-shared symmetric key between the NRF and the NF Service Producer. The provisioning of such pre-shared symmetric key is outside the scope of this document.

Securing the access token using digital signature based on JSON Web Signature (JWS) as described in RFC 7515 [45] requires that the issuer NRF has a public/private key pair and the NF Service Producer needs to be utilize the public key of the issuer NRF to verify the digital signature of the access token. The provisioning of the NF Service Producer with the public key of the NRF (through an X.509 certificate) to the NF Service Producer can be done by OAM, by using the service operation specified in the clause 14.3.X or any other implementation specific way.

The basic extent provided by the authorization token is at service level (i.e. the "scope" claim includes allowed services per NF type). Depending on the NF Service Producer configuration, higher level of granularity for the authorization token can be defined adding "additional scope" information within the token e.g. to authorize specific service operations and/or resources/data sets within service operations per NF Service Consumer type.

NOTE 1: The additional scope(s) included within the access token add additional security checks at the NF Service Producer that authorizes the services operations, resources and NF Service Consumer type related to the additional scope(s).

The authorization framework described in clause 13.4.1 is mandatory to support for NRF and NF.

The OAuth 2.0 framework does not apply to the notification operation.

Extensions to the authorization framework specific for the security of enablers for Network Automation by 5GS are described in Annex X.

********** NEXT CHANGE

##### 13.4.1.1.1	OAuth 2.0 roles

OAuth 2.0 roles, as defined in clause 1.1 of RFC 6749 [43], are as follows:

a.	The Network Repository Function (NRF) shall be the OAuth 2.0 Authorization server.

b.	The NF Service Consumer shall be the OAuth 2.0 client.

c.	The NF Service Producer shall be the OAuth 2.0 resource server.

OAuth 2.0 client (NF Service Consumer) registration with the OAuth 2.0 authorization server (NRF)

The NF Service registration procedure, as defined in clause 4.17.1 of TS 23.502 [8], may be used to register the OAuth 2.0 client (NF Service Consumer) with the OAuth 2.0 Authorization server (NRF), as described in clause 2.0 of RFC 6749 [43]. The client id, used during OAuth 2.0 registration, shall be the NF Instance Id of the NF. OAuth2.0 clients may also register with the NRF using OAM.

A Network Function that does not implement this option shall be able to get an access token from the NRF as long as the NRF is able to authenticate and authorize the Network Function during the NF access token get service request.

OAuth 2.0 resource server (NF Service Producer) registration with the OAuth 2.0 authorization server (NRF)

The NF Service registration procedure, as defined in clause 4.17.1 of TS 23.502 [8], shall be used to register the OAuth 2.0 resource server (NF Service Producer) with the OAuth 2.0 Authorization server (NRF). The NF Service Producer, as part of its NF profile, may include "additional scope" information related to the allowed service operations and resources per NF Service Consumer type.

Figure 13.4.1.1-1b NF Service Producer registers in NRF

1)	The NF Service Producer registers as OAuth 2.0 resource server in the NRF. The NF profile configuration data of the NF Service Producer may include the "additional scope". The "additional scope" information indicates the resources and the actions (service operations) that are allowed on these resources for the NF Service Consumer. These resources may be per NF type of the NF Service Consumer or per NF instance ID of the NF Service Consumer.

2-3)	After storing the NF Profile, NRF responds successfully.

NOTE: 	Depending on network deployment and operator policy, the OAuth 2.0 authorization server (NRF) that issues an access token can be a different NRF than the NRF that the NF Service Producer is registered at, especially when issuing access token for accessing services of NF Service Producers of a specific NF type.

Authorized OAuth 2.0 authorization servers (NRFs) together with their X.509 certificates or information from where to get these certificates from shall be configured in the NF Service Producer for access token validation.

********** NEXT CHANGE

##### 13.4.1.1.2	Service Request Process

The complete service request is a two-step process including requesting an access token by NF Service Consumer (Step 1, i.e. 1a or 1b), and then verification of the access token by NF Service Producer (Step 2).

NOTE 1a: The service request process regarding the enabler for network automation is specified in Annex X.

Step 1: Access token request

Pre-requisite:

- The NF Service consumer (OAuth2.0 client) is registered with the NRF (Authorization Server).

- The NF Service Producer (OAuth2.0 resource server) is registered with the NRF (Authorization Server) with optionally "additional scope" information per NF type.

- The NRF and NF Service Producer share the required credentials.

- The NRF and NF have mutually authenticated each other – where the NF Service Consumer is identified by the NF Instance ID of the public key certificate of the NF Service Consumer.

1a. Access token request for accessing services of NF Service Producers of a specific NF type

The following procedure describes how the NF Service Consumer obtains an access token before service access to NF Service Producers of a specific NF type.

Figure 13.4.1.1.2-1: NF Service Consumer obtaining access token before NF Service access

1.	The NF Service Consumer shall request an access token from the NRF in the same PLMN using the Nnrf_AccessToken_Get request operation. The message shall include the NF Instance Id(s) of the NF Service Consumer, the requested "scope" including the expected NF Service name(s) and optionally "additional scope" information (i.e. requested resources and requested actions (service operations) on the resources).

The message shall include the NF type of the expected NF Service Producer instance and NF Service Consumer. The NF Service Consumer may also include a list of S-NSSAIs or list of NSI IDs for the expected NF Service Producer instances in the access token request. The message may include the NF Set ID and/or NF Service Set Id of the expected NF Service Producer instances.

The message may include a list of S-NSSAIs of the NF Service Consumer.The message may also include the PLMN ID(s) of the NF Service Consumer.

2.	The NRF shall verify that the input parameters NF Instance ID and NF type as well as PLMN ID(s), if available, in the access token request match with the corresponding ones in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. If the verification of the parameters in the access token request fails, the access token request is not further processed.

The NRF shall additionally verify the S-NSSAIs of the NF Service Consumer and check whether there are restrictions on the NF Service Consumer to access NF Service Producers' services of a specific NF type depending on the slices for which they offer their services. The NRF checks whether the NF Service Consumer is authorized to access the requested service(s). For example, the NRF may verify that the NF Service Consumer can serve a slice which is included in the allowed slices for the NF Service Producer of a specific NF type. If the NF Service Consumer is authorized, the NRF shall then generate an access token with appropriate claims included. The NRF shall digitally sign the generated access token based on a shared secret or private key as described in RFC 7515 [45]. If the NF Service Consumer is not authorized, the NRF shall not issue an access token to the NF Service Consumer.

The claims in the token shall include the NF Instance Id of NRF (issuer), NF Instance Id of the NF Service Consumer (subject), NF type of the NF Service Producer (audience), expected service name(s) (scope), expiration time (expiration) and optionally "additional scope" information (allowed resources and allowed actions (service operations) on the resources). The claims may include a list of S-NSSAIs or NSI IDs for the expected NF Service Producer instances. The claims may include the NF Set ID and/or NF Service Set Id of the expected NF Service Producer instances.

NOTE 1: If the claims do not include a list of NSSAIs or NSI IDs for the target NF type, it implies the token can be used to access expected NF services of all expected NF Service Producers of the NF type based on local configuration and operator policy.

NOTE 2: The expiration time claim (expiration) of the token is to impose time limits on the access token in use. It is carefully chosen based on the operator’s policy to allow flexibility and cost effectiveness, taking into consideration different threat situations and network complexities etc. In the present document, token revocation is not supported.

3.	If the authorization is successful, the NRF shall send access token to the NF Service Consumer in the Nnrf_AccessToken_Get response operation, otherwise it shall reply based on Oauth 2.0 error response defined in RFC 6749 [43]. The other parameters (e.g., the expiration time, allowed scope) sent by NRF in addition to the access token are described in TS 29.510 [68]. The NF Service Consumer may store the received token(s). Stored tokens may be re-used for accessing service(s) from NF Service Producer NF type listed in claims (scope, audience) during their validity time.

1b. Access token request for accessing services of a specific NF Service Producer instance / NF Service Producer service instance

1.	The following steps describes how the NF Service Consumer obtains an access token before service access to a specific NF Service Producer instance / NF Service Producer service instance. 1. The NF Service Consumer shall request an access token from the NRF for a specific NF Service Producer instance / NF Service Producer service instance. The request shall include the NF Instance Id(s) of the requested NF Service Producer, the expected NF Service name, optionally "additional scope" information (allowed resources and allowed actions (service operations) on the resources) and NF Instance Id of the NF Service Consumer. The request may also include the PLMN ID(s) of the NF Service Consumer.

2.	The NRF shall verify that the input parameters in the access token request, i.e. NF Instance ID and, if available, PLMN ID(s) and NF type, match with the corresponding ones in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. If the verification of the parameters in the access token request fails, the access token request is not further processed.

The NRF checks whether the NF Service Consumer is authorized to access the requested services from the NF Service Producer instance/NF Service Producer service instance. The NRF shall additionally verify the S-NSSAIs of the NF Service Consumer and check whether there are restrictions on the NF Service Consumer to access NF Service Producers' services depending on the NF Service Producer's allowed slices for which they offer their services. For example, the NRF may verify that the NF Service Consumer can serve a slice which is included in the allowed slices for the NF Service Producer instance / NF Service Producer service instance. If the NF Service Consumer is authorized,the NRF proceeds to generate an access token with the appropriate claims included. If the NF Service Consumer is not authorized, the NRF shall not issue an access token to the NF Service Consumer.

The claims in the token shall include the NF Instance Id of NRF (issuer), NF Instance Id of the NF Service Consumer (subject), NF Instance Id or several NF Instance Id(s) of the requested NF Service Producer (audience), expected service name(s) (scope), optionally "additional scope" information (allowed resources and allowed actions (service operations) on the resources), and expiration time (expiration).

3.	The token shall be included in the Nnrf_AccessToken_Get response sent to the NF Service Consumer. The NF Service Consumer may store the received token(s). Stored tokens may be re-used for accessing service(s) from NF Instance Id or several NF Instance Id(s) of the requested NF Service Producer instance listed in claims (scope, audience) during their validity time.

Step 2: Service access request based on token verification

The following figure and procedure describe how authorization is performed during Service request of the NF Service Consumer. Prior to the request, the NF Service Consumer may perform Nnrf_NFDiscovery_Request operation with the requested additional scopes to select a suitable NF Service Producer (resource server) which is able to authorize the Service Access request.

Figure 13.4.1.1.2-2: NF Service Consumer requesting service access with an access token

Pre-requisite: The NF Service Consumer is in possession of a valid access token before requesting service access from the NF Service Producer.

1.	The NF Service Consumer requests service from the NF Service Producer. The NF Service Consumer shall include the access token.

The NF Service Consumer and NF Service Producer shall authenticate each other following clause 13.3.

2.	The NF Service Producer shall verify the token as follows:

The NF Service Producer shall check that the issuer claim in the access token matches the identity of the OAuth 2.0 authorization server (NRF)   and that the NRF in general is allowed to issue an access token for services provided by this NF Service Producer based on the information available to the NF Service Producer at configuration time (either a URI where to retrieve these information or the information is stored). If they do not match, the NF Service Producer may provide error cause and logging information to the NRF where the NF Service Prodcuer is registered or any other NRF or   OAM.

-	The NF Service Producer ensures the integrity of the token by verifying the signature using NRF’s public key or checking the MAC value using the shared secret.

NOTE X: 	The NRF’s public key can be configured at the NF Service Producer, or the NF Service Producer can retrieve the NRF’s public key through the service operation specified in clause 14.3.X. The NF Service Producer is pre-configured with a root certificate. The NF Service Producer can also be preconfigured with any intermediary CA certificates that can be used to verify the NRF certificate that is associated with the private key the NRF used for signing the access token. To ensure that the NRF’s certificate is not revoked, the NF Service Producer can retrieve the status of the NRF’s certificate through the service operation specified in clause 14.3.X or corresponding certificate authority, or NF Service Producer can subscribe the status change of the NRF’s certificate through the service operation specified in clause 14.3.X. NF Service Producer can report any suspicious behaviour by error logging through the service operation specified in clause 14.3.X.

-	 If integrity check is successful, the NF Service Producer shall verify the claims in the token as follows: -

-	In the direct communication case, it checks that the NF Instance ID in the subject claim within the access token matches the NF Instance ID in the subjectAltName in the NF Service Consumer's TLS client certificate.

NOTE 3: Void.

-	It checks that the audience claim in the access token matches its own identity or the NF type of NF Service Producer. If a list of S-NSSAIs or list of NSI IDs i of the NF type of the NF Service Producer s present, in the access token the NF Service Producer shall check that at least one of the S-NSSAIs or NSI IDs served by the NF Service Producer is included in the list. If applicable (e.g., when the request is for information related to a specific UE), the NF Service Producer may check that the NF Service Consumer is allowed to access (as indicated by the NF Service Producer’s S-NSSAIs in the access token presented by the NF Service Consumer) at least one  slice(s) that the UE is currently registered to, e.g., by verifying that the UE’s allowed NSSAI(s) intersect with the NF Service Producer's S-NSSAIs in the access token.

-	If an NF Set ID present, the NF Service Producer shall check the NF Set ID in the claim matches its own NF Set ID.

If an NF Service Set ID present, the NF Service Producer shall check if the NF Service Consumer is authorized to access the requested service according to NF Service Producer Service Set ID in the access token claim.

-	If scope is present, it checks that the scope matches the requested service operation.

- 	If the access token contains "additional scope" information (i.e. allowed resources and allowed actions (service operations) on the resources), it checks that the additional scope matches the requested service operation.

-	It checks that the access token has not expired by verifying the expiration time in the access token against the current data/time.

-	If the CCA is present in the service request, it may verify the CCA as specified in clause 13.3.8.3 and that the subject claim (i.e., the NF Instance Id of the NF Service Consumer) in the access token matches the subject claim in the CCA.

3.	If the verification is successful, the NF Service Producer shall execute the requested service and responds back to the NF Service Consumer. Otherwise, it shall reply based on Oauth 2.0 error response defined in RFC 6749 [43].

********** NEXT CHANGE

### 14.3.1	General

The following table illustrates the security related services for OAuth 2.0 that NRF provides. OAuth 2.0 based authorization is described in clause 13.4.1.

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnrf_AccessToken | Get | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF |
|  | CertStatus |  |  |
|  | GetCert | Request/Response | NRF, AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF |
|  | RetrieveCert |  |  |
|  | SubscribeCertStatus |  |  |
|  | NotifyCertStatus |  |  |
|  | ErrorReport |  |  |
|  | Logging |  |  |

The complete list of NRF services is defined in TS 23.501 [2], clause 7.2.6, and further refined in TS 23.502 [8], clause 5.2.7.

********** NEXT CHANGE

### 14.3.X	Nnrf_AccessToken CertStatus Service Operation

Service Operation name: Nnrf_AccessToken_CertStatus.

Description: NF Service Producer requests NRF to provide status of a specific certificate or public key cached.

Inputs, Required: the NF Instance Id of the token issuer NRF, (list of) key identifier(s).

Inputs, Optional: UUID of NF Service Producer.

Outputs, Required: status information (active, revoked, expiry) for one or several certificates.

Outputs, Optional: None.

### 14.3.X	Nnrf_AccessToken_GetCert Service Operation

Service Operation name: Nnrf_AccessToken_GetCert.

Description: NF Service producer requests NRF to provide public key certificate to validate the signature of the Access Token.

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier (Key ID from the Header Parameter of the JOSE header of the access token).

Inputs, Optional: UUID of NF Service Producer.

Outputs, Required: X.509 public key certificate(s) or certificate(s) chain used for signing the access token.

Outputs, Optional: None.

### 14.3.X	Nnrf_AccessToken_RetrieveCert Service Operation

Service Operation name: Nnrf_AccessToken_RetrieveCert.

Description: NF Service producer retrieves from NRF the public key certificate or to validate the signature of the Access Token.

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier , UUID of the NF Service producer issuing the request (since certain certificates may be applicable only for specific NF Service Producers.

Inputs, Optional: None.

Outputs, Required: X.509 public key certificate(s) or certificate chain(s) used for signing the access token.

Outputs, Optional: None.

### 14.3.X	Nnrf_AccessToken_SubscribeCertStatus Service Operation

Service Operation name: Nnrf_AccessToken_SubscribeCertStatus.

Description: NF Service producer subscribes notification of certificate status change from NRF .

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier ,  callback URI.

Inputs, Optional: UUID of the NF Service producer issuing the request,.

Outputs, Required: list of updated certificates, or issuer id and key id, which allows to use GetCert or RetrieveCert operation.

Outputs, Optional: None.

### 14.3.X	Nnrf_AccessToken_NotifyCertStatus Service Operation

Service Operation name: Nnrf_AccessToken_NotifyCertStatus.

Description: NRF sends notification of certificate status change to the NF producer subscribed to change of the certificate status.

Inputs, Required: original and new status of the certificate(s) with certificate id, e.g. issue id + kid.

Inputs, Optional: None.

Outputs, Required:  None.

Outputs, Optional: None.

### 14.3.X	Nnrf_AccessToken_ErrorReport Service Operation

Service Operation name: Nnrf_AccessToken_ErrorReport.

Description: NF Service Consumer requests NRF to take notice of the cause of error, when it is not possible to validate the signature of the Access Token.

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier, UUID of the NF Service producer issuing the request, callback URI, error cause.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

### 14.3.X	Nnrf_AccessToken_Logging Service Operation

Service Operation name: Nnrf_AccessToken_Logging.

Description: NF Service Consumer povides NRF with any type of logging information related to the service operations.

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier, UUID of the NF Service producer issuing the request, callback URI, logging information.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

********** END OF CHANGES