3GPP TSG-SA3 Meeting #119	S3-245096

# Orlando, US, 11 -15 November 2024

# Source:	Ericsson, Nokia, Nokia Shanghai Bell

# Title:	New WID on Protection of XRM Metadata

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Protection of XRM Metadata

Acronym: XRM_Ph2-Sec

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| XRM_Ph2 | SA WG2 | 1040032 | Extended Reality and Media service (XRM) Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: YES

# 3	Justification

The topic of Extended Reality and Media service (XRM) has been studied by SA2 in TR 23.700-70.

KI#2 of that TR is about the UPF being able to identify certain XRM Metadata sent from an external Application Server (AS) to the UE together with end-to-end (e2e) encrypted XRM data streams. It was concluded (clause 8.2. of TR 23.700-70) that the XRM metadata shall be encrypted and integrity protected between the UPF and the AS.

It was further concluded by SA2 to support multiple methods for this in normative work:

- Using Media over QUIC (MoQ)

- Using Proxy-UDP-in-HTTP/3 together with QUIC-Aware Proxying,

- Using UDP options

The requirement (as stated in the conclusion of TR 23.700-70) that XRM metadata shall be encrypted and integrity protected implies that further work is needed by SA3 to specify the details of the security solutions for the three selected methods above.

# 4	Objective

This work aims to specify required enhancements on 5GS to support protection of XRM metadata sent between an external Application Server and the UPF.

The following work-items are included based on the conclusions in the TR 23.700-70:

- WT#1 Media over QUIC (MoQ): Specify security details (if any) for the case when MoQ is used to transfer the XRM Metadata

-WT#2 Proxy-UDP-in-HTTP/3 + QUIC-Aware Proxying: Specify the security details for the case when QUIC-aware-proxying is used to transfer the XRM Metadata

-WT#3 UDP-option: Specify the security details for the case when UDP-options are used to transfer the XRM Metadata

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | Dependencies |
| --- | --- | --- | --- |
| WT#1 | 0 | 0.25 | WT is self-contained |
| WT#2 | 0 | 0.25 | WT is self-contained |
| WT#3 | 0 | 0.25 | WT is self-contained |

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.501 | New Clause/Annex for XRM | TSG#107 (March 2025) |  |

# 6	Work item Rapporteur(s)

TBD: {Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

Stage 3 aspects covered by CT WGs.

# 9	Supporting Individual Members

Supporting IM name

Ericsson

Nokia

Nokia Shanghai Bell