3GPP TSG-SA3 Meeting #119	S3-245188

# Orlando, USA  11 - 15 November 2024

# Source:	Nokia, Nokia Shanghai Bell, Ericsson

# Title:	Conclusion for KI#1 - Third party specific user identities

# Document for:	Approval

Agenda Item:	5.2

# 1	Decision/action requested

It is required to approve the conclusion proposed for KI#1 in TR 33.790

# 2	References

[1]	3GPP TR 23.700-77: "Study on system architecture for next generation real time communication services; Phase 2"

[2]	3GPP TR 33.790: "Study on the security support for the next generation real time communication services phase 2"

[3]	S2-2411009:  CR for KI#4 to Support of third party user identity information in IMS

# 3	Rationale

KI#4 on "Extensible IMS framework to support authorization and authentication of third-party identities in IMS sessions" was concluded in TR 23.700-77, and CR to TS 23.228 for KI#4 to support third party user identity information in IMS was agreed.

The CR defined overall system architecture and functional entities to support authorization, authentication, and verification of third party user identity information in IMS (Annex XX.2) as below:

The CR also described procedures to support authorization and verification of third party user identity information in IMS (Annex XX.3).

Potential security requirements of Key issue #1 (Third party specific user identities) are defined in 5.1.3 of TR 33.790 as follows:

-  The IMS system shall be able to coordinate with the third party to verify and authorize the third-party specific user identities.

-  The IMS network shall be able to support the integrity protection of the third-party specific user identities on the originating side and terminating side.

According to the agreed CR to TS 23.228, the first requirement on the third-party specific user identities verification and authorization is supported in the System architecture to support authorization, authentication, and verification of third party user identity information in IMS, see Annex XX of TS 23.228.  The integrity protection of the third-party specific user identities on the originating side and terminating side is protected also in case the RCD information is signed and included in the PASSporT.

As a result the normative work for KI#1 in the TR 33.790 [2] is proposed to be a new clause in TS 33.501 [4] or TS 33.203 [5].

# 4	Detailed proposal

******************START of CHANGES****************************************

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

For KI#1 the normative work principles from the conclusions in TR 23.700-77 [2], clause 8.4 (Conclusions for KI#4) are followed. Moreover there is an agreed CR for TS 23.228 [3] which specifies in sufficient detail the support for authorization, signing, and verification of third party user identity information in IMS.

As a result the normative work for KI#1 in this document is proposed to be a new clause in TS 33.501 [11] or TS 33.203 [XX] which will provide a summary of the new feature, list of security requirements and a reference to the related clauses in TS 23.228 [7].

******************START of the 2nd CHANGES********************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-77: ""Study on system architecture for next generation real time communication services; Phase 2".

[3]	3GPP TR 33.890: "Study on security support for next generation real time communication services".

[4]	3GPP TR 24.229: " IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP);Stage 3".

[5]	ATIS-1000074: "Signature-based Handling of Asserted information using Tokens (SHAKEN)|

[6]	IETF draft-ietf-stir-passport-rcd-26: "PASSporT Extension for Rich Call Data"

Editor's Note:	The above document cannot be formally referenced until it is published as an RFC.

[7]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2"

[8]	IETF draft-ietf-sipcore-callinfo-rcd-09: "SIP Call-Info Parameters for Rich Call Data".

Editor's Note:	The above document cannot be formally referenced until it is published as an RFC.

[9]	draft-ietf-sipcore-callinfo-rcd-03: "SIP Call-Info Parameters for Rich Call Data".

[10]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[11]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[12]	IETF RFC 7515: "JSON Web Signature (JWS)"

[13]	3GPP TS 33.122: " Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs "

[14]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System architecture for the 5G System (5GS); Stage 2"

[XX]	3GPP TS 33.203: "3G security; Access security for IP-based services"

******************END of CHANGES****************************************