3GPP TSG-SA3 Meeting #119	S3-245275

Orlando, USA, 11 – 15 November 2024

# revision of S3-244792

# Source:	BSI (Federal Office for Information Security)

# Title:	Add specific UDR SCAS test cases for TS 33.530

# Document for:	Approval

Agenda Item:	4.2

# 1	Decision/action requested

This contribution proposes UDR-specific SCAS test case.

# 2	References

[1]	3GPP TS 33.530 Security Assurance Specification (SCAS) for the Unified Data Repository (UDR) v0.1.0

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements"

# 3	Rationale

This contribution proposes UDR-specific test cases related to security functional requirements of the UDR to draft TS [1] chapter 4.

This test case is derived from the requirements described in Rel-19 TS 33.501, 5.8b Requirements on the UDR and the threat references as part of the Rel-19 TR 33.926 Annex AA. The UDM can use the UDR as a repository function for storing data.

# 4	Detailed proposal

It is proposed that SA3 approve the below changes for inclusion in the TS [1].

********** START OF 1st CHANGE **********

[4]		3GPP TS 33.501: "Security architecture and procedures for 5G system"

[5]		3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP

network product classes".

[6]	3GPP TS 29.504: “5G; 5G System; Unified Data Repository Services; Stage 3”

********** START OF 2nd CHANGE **********

## 4.2	UDR-specific adaptations of security functional requirements and related test cases

### 4.2.1	Introduction

The present clause describes the security functional requirements and the corresponding test cases for UDR network product class. The proposed security requirements are classified in two groups:

-	Security functional requirements derived from TS 33.501 [2] and detailed in clause 4.2.2.

-	General security functional requirements which include requirements not already addressed in TS 33.501 [2] but whose support is also important to ensure that UDR conforms to a common security baseline detailed in clause 4.2.2.0.

### 4.2.2	Security functional requirements on the UDR deriving from 3GPP specifications and related test cases

#### 4.2.2.1	General

The general approach in TS 33.117 [3] clause 4.2.2.1 and all the requirements and test cases in TS 33.117 [3] clause 4.2.2.2 related to SBA/SBI aspect apply to the UDR network product class.

#### 4.2.2.2 	Specific UE security-related information update in the UDR

Requirement Name: Ability of NFs to update UE security-related information in the UDR

Requirement Reference: TS 33.501 [4], clause 5.8b Requirements on the UDR

Threat References: TR 33.926 [5], clause AA.2 Assets and threats specific to the UDR

Requirement Description: Only the UDM is able to update security-related information in the UDR.

Test Case:

Test Name: TC_ONLY_UDM_CAN_UPDATE_UDR

Purpose:

- Verify that only the UDM is able to update the security related information in the UDR.

Procedure and execution steps:

Pre-Condition:

- The test case is applicable if UDR and UDM are implemented as separate network functions (non-colocated)

- The network environment, such as the UDM, can be simulated.

- The network environment, such as the non-UDM, can be simulated.

Execution Steps:

Test Case 1:

- The tester correctly calculates an access token with valid audience and scope claims reflecting the UDM network function to access the UDR resources.

- The tester includes the access token into an update request to the UDR, e.g., update the SQN or the Authentication Status.

Test Case 2:

- The tester correctly calculates an access token with valid audience and scope claims reflecting a non-UDM network function to access the UDR resources.

- The tester includes the access token into an update request to the UDR, e.g., update the SQN or the Authentication Status.

Expected Results:

For test case 1: The UDR accepts the service request.

For test case 2: The UDR rejects the service request.

Expected format of evidence:

Evidence suitable for the interface, e.g., pcap file or screenshot containing the operational results.

********** END OF CHANGE **********