# 3GPP TSG-SA3 Meeting #119 	S3-245304

# November 11 –15, 2024  Orlando, USA

# Source: 	Interdigital

# Title: 	Resolution of ENs in Solution #21

# Document for: 	Approval

Agenda Item: 	5.9

# 1	Decision/action requested

Approve the pCR to TR 33.713

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[3]	TS 22.369 Service requirements for Ambient power-enabled IoT

# 3	Rationale

This contribution resolves Editor’s Notes in Solutions #21 by adding content and expanding the Evaluation.

*** Start of 1st Change ***

### 6.21.2			Details

The simplified call flow based on the AIOT Random Access framework is presented below. This procedure is based on the security properties of cryptographic puzzles.

Figure 6.21.2-1: Modified AIoT Random Access Procedure for establishing ephemeral security between AIOT Device and Reader

The steps in Figure 6.21.2-1 are described below.

1. The Reader determines to prepare a set of cryptographic puzzles and the Reader will prepare a set of N tuples. Each tuple is comprised of a K-MACi (key) and corresponding K-MACi-IND (key index corresponding to key).

2. The Reader composes a set of N cryptographic puzzles each hiding a tuple comprising of the Ephemeral Key K-MACi, corresponding Ephemeral Key Index K-MACi-IND , and either partial key or partial hash function argument.

3. The reader transmits a paging message and a set of occasion synchronization messages. The combination of the paging message and synchronization message identifies which AIoT devices should respond to the paging message. In other words, the combination of the paging message and synchronization messages identifies which AIoT devices should perform a random access procedure. The paging message includes one or more of the puzzles that were constructed in step 2.

4. The AIoT Device uses the information in the paging message and synchronization messages to determine that the AIoT Device needs to respond to the paging message. In other words, the AIoT Device determines that it needs to perform a RACH procedure.  If the AIoT Device determines that it needs to perform a RACH procedure, then, a the AIOT Device randomly selects one puzzle from the set of N puzzles that were received in the paging message. If the AIoT Device does not determine that it needs to perform a RACH procedure, or the AIoT device determines that the strength of the puzzle, as determined by the puzzle number, does not satisfy certain requirements, then the procedure will stop in this step and the AIoT will not perform a RACH procedure. Determining that the strength of the puzzle does not satisfy the AIoT security requirements for the application means that the selected puzzle strength may compromise information used by the application after the security context is established with a puzzle of a certain strength.

5. The AIoT Device solves the puzzle that was selected in step 4 and recovers the security parameters.

6. An AIOT device selects an occasion (using at least slotted ALOHA as the baseline), and transmits a random device ID in a message, MSG1. The message also includes the K-MACi-IND that was recovered in step 5.

7. The Reader performs a lookup for the K-MACi from the corresponding K-MACi-IND received in step 6.

8. The AIOT Device and the Reader enter a state where they have established an ephemeral security context using K-MACi. The subsequent messages of this procedure may now be confidentiality and integrity protected using the ephemeral security context based on K-MACi. In other words, the AIOT Device and the Reader have established an ephemeral security context. The ephemeral security context is based on K-MACi. After this step, the AIoT Device may use the ephemeral security context to encrypt data that it sends to the reader and the reader may use the ephemeral security context to encrypt data that it sends to the AIoT Device. Thus, information can be sent more securely between the AIoT Device and the Reader. In addition, the ephemeral security context obtained in this step can be used to bootstrap more persistent security associations between AIOT functional entities.

9. Upon successful reception of MSG1, the reader transmits MSG2 by including the received random device ID in MSG2. The Reader may use K-MACi to encrypt some or all of the information in MSG2.

10. The AIoT Device uses K-MACi to decrypt some or all of the information in MSG2. A random device ID is an example of information that is carried in MSG2. If the AIoT Device determines that the random device ID that it transmitted in step 6 is included in MSG2, then the AIoT Device transmits MSG3 which contains upper-layer data (e.g., an application layer device ID). The AIoT Device may use K-MACi to encrypt some or all of the information in MSG3.

11. The Reader uses K-MACi to decrypt some or all of the information in MSG3. The reader may then Transmit MSG4 (e.g., for subsequent command transmission), but the understanding is that contention is already resolved at MSG2 transmission. The reader may use K-MACi to encrypt some or all of the information in MSG4.

### 6.21.3			Evaluation

This solution addresses the requirement of Key issue #3.

This solution proposes a method for the establishment of the ephemeral security context that can be used for privacy protection for the AIoT Device identity.

A limited paging message link budget may preclude using multiple puzzles in paging messages.

The use of ephemeral security association before the AIOT command triggering may cause an additional resource consumption for all AIOT devises in range.

Power and processing-constrained AIOT devices might not be capable of using puzzle-based methods. In addition, puzzle-based methods rely on the comparative abilities of AIOT devices vs. adversaries that may be not power and processing-constrained. A security analysis is needed to determine the applicability of puzzle-based methods in AIOT security.

Editor’s Note: Further evaluation is FFS.

*** End of 1st Change ***