| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | - | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifying multiplexed media stream data flows using PDU Set Marking information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital France R&D, SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 20th May 2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This pCR proposes a solution to a new key issue related to traffic detection in multiplexed media stream data flows described in S4-241087 by introducing the stream identifier in PDU Set Marking RTP HE. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Update to clauses 5 and 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Background

The new SA4 Rel-19 study item on “5G Real-time Transport Protocol Configurations, Phase 2” (5G_RTP_Ph2) has been approved in SP-240523. The study item lists thirteen distinct key issues to improve 5G RTP as defined in TS 26.522. In contribution S4-241087, we discussed a new key issue #14 on traffic detection in multiplexed media stream data flows. In this contribution we propose solution to the above new key issue #14 by introducing the stream identifier in PDU Set Marking RTP HE.

# Changes

First Change

# Solutions

## 6.X Solution #X: Traffic detection and QoS flow mapping for multiplexed media stream data flows

### 6.X.1 Key Issue mapping

This solution maps to Key Issue #14.

### 6.X.2 Description

XR Media services may include multiple types of media data flows, e.g., video streams, audio streams, metadata, or sensor data for more immersive experience. To enable these immersive services, different types of media data can be multiplexed into a single data flow or multiple data flows. Each media stream may have its own PDU set properties. PDU sets arriving at the UPF and RAN nodes are from different streams, and the RAN nodes needs to identify the respective media streams to which they belong.

Each PDU set may have an identifier to help UPF and RAN nodes to identify the media stream it belongs to using a stream identifier (SID). An Application Server (AS) or an Application Function (AF) may generate a unique SID for a media stream and add the SID information into the PDU Set marking RTP HE. The presence of media stream identifier information in a PDU Set Marking RTP HE may be signalled via control plane signalling, for example as an extension to the Protocol Description provided by the AF.

### 6.X.2.1 One-byte RTP Header Extension Formats

The one-byte PDU Set Marking RTP HE with the stream identifier information is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |E| R |D|  PSI  |      PSSN         |     PSN   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     PSSize                    |     NPDS

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|               |        SID                    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 6.X.2.2 Two-byte RTP Header Extension Formats

The two-byte PDU Set Marking RTP HE with the stream identifier information is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         0x100         |appbits|           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      ID       |      len      |E| R |D|  PSI  |      PSSN

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    PSN    |                   PSSize                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             NPDS              |       SID                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 6.X.2.3 Semantics

The semantics of the stream ID field of the PDU Set Marking RTP Header Extension is defined as follows:

- stream ID [SID] (16 bits): This field indicates a unique identifier for a media stream present in an RTP session. It is used for identifying media streams within an RTP session or across RTP sessions. This field is optional and may subject to signaling negotiation, where the RTP sender may indicate whether it will provide the media stream identifier value in the PDU Set RTP HE or not. If not enabled, the field shall not be present within the RTP Header Extension. If enabled, but the RTP sender is unable to provide a unique media stream identifier, it shall set the value to 0 in all PDUs of that PDU Set. The unique stream identifier value for a media stream may be generated by the Application server or the Application Function.

End of First Change

# Proposal

We propose to agree the proposed changes in clause 2 into TR 26.822.