3GPP TSG-SA WG4 Meeting #130						S4-241840

USA, Orlando, 18 – 22 November 2024

Source:	HEAD acoustics GmbH

Title:	Test methods for UE acoustic echo control

Agenda item:	7.8

Document for:	Discussion

# Introduction

One important objective of the new work item ATIAS_Ph2 [1] is to introduce test methods for the performance evaluation of acoustic echo control functionality integrated in the UE. The most relevant aspect for traditional as well as for immersive speech communication is that the generation and insertion of echo signals in send direction should be avoided wherever possible. For mono telephony, several test methods and requirements are specified in 3GPP TS 26.131/132 [2] [3] for this purpose.

In order to investigate if/how such test methods and related requirements could be applied/reused/adapted for immersive communication terminals and corresponding specifications 3GPP TS 26.260/261 [4] [5], the present document highlights differences and similarities to mono telephony and identifies possible gaps that need to be defined.

# Communication scenario and terminology

Figure 1 illustrates an immersive, IVAS-based communication setup between a near-end and far-end user. Similar to traditional mono telephony, the actual UE under test is depicted on the left side as the near-end side. Audio is transmitted from both directions and received by the opposite end ("full duplex").

Figure 1: Full duplex speech communication with near-end and far-end

In immersive communication scenarios, two possible degradations may occur:

1)	In receiving direction, the signal from the far-end is decoded/rendered into a coded format supported by the UE.

-	An additional signal processing block prepares the output of M playback channels (i.e., loudspeakers or headphones), over which the signal is presented to the listener's ears.

-	The acoustic output is also fed back to the UE over the N input microphones. This transmission path is typically denoted as an "echo path".

-	Without further processing in sending direction, the user at the far-end would perceive his/her own voice as an echo. To estimate and compensate the echo path signal, typically the receive signal is utilized in the signal processing in sending direction.

2)	In sending direction, the user's voice at the near-end is captured by N input microphones.

-	Several parallel or serial signal processing blocks like e.g., noise reduction, echo cancellation, etc. are applied.

-	From the captured and processed inputs, the UE performs a rendering to a certain IVAS coded format. The audio is then IVAS-encoded, transmitted and finally decoded/rendered/played back over the far-end device.

-	In double talk scenarios, concurrent near-end and far-end speech is active. The user's voice in sending direction might be degraded due to the simultaneous echo cancellation processing.

NOTE 1:	The M playback channels do not necessarily correspond to the coded format that was produced by the decoder.

NOTE 2:	The N input microphones/channels do not necessarily correspond to a certain IVAS coded format.

NOTE 3:	The description above and the illustration in Figure 1 are intended to be rather generic and do not consider any specific UE type according to clause 5.4.2 in 3GPP TS 26.260 [4] (like e.g., handheld hands-free UE in RCV, headset UE in SND, etc.).

NOTE 4:	The description above and the illustration in Figure 1 exemplarily assume a scenario in which send and receive direction correspond to acoustic signal capture and playback. An equivalent description and following considerations made in the present document apply for electrical interface UEs as well, but both are left out here for sake of simplicity.

In test specifications for terminals like 3GPP TS 26.131/132, both aspects are typically handled by separate test cases and requirements (with and without active far-end speech), which should also be considered for possible new test methods in 3GPP TS 26.260 [4].

# Test setup

## Overview

Based on the definitions of the previous clause, Figure 2 illustrates a simplified scenario that is applicable for testing purposes. Instead of a far-end device/talker, a reference client and corresponding test equipment are used to evaluate the performance of echo cancellation and double talk capabilities of the UE by inserting/capturing signal at the point of interconnect (POI).

Figure 2: Full duplex speech communication with reference client and test equipment

It can be noticed that both test cases on acoustic echo control (echo-only and double talk) are …

-	always evaluated in sending direction, and

-	are independent of the coded-format and type of audio playback (headphones, loudspeaker, electrical) used in receiving direction.

These two observations have impacts on drafting possible new test methods (and requirements), which are discussed in the following sub-clauses.

## Test Signals

### Echo cancellation

When testing echo cancellation performance without active far-end signal, a commonly used approach is to test a worst-case scenario, i.e., with maximum possible acoustic/electrical output. This is usually achieved by using the maximum volume control setting (if available) and a test signal providing …

-	high level, close to digital overload,

-	low crest-factor (to "squeeze" as much energy as possible into digital signal representation)

For this purpose, the "compressed speech signal" from clause 7.3 of Recommendation ITU-T P.501 [6] is used in 3GPP TS 26.131/132 for such tests:

-	Level: -10 dBm0 (nominal: -16 dBm0)

-	Crest factor: 15 dB (without compression: 23 dB)

To adapt the test signal for UEs supporting immersive audio, several aspects have to be considered/discussed:

-	The compressed speech signal from ITU-T P.501 is a mono signal. If it should also be used for testing immersive UEs, test signals for the receive direction need to be specified for each coded format.

-	Instead of "designing" test signals for each of the various audio formats manually, a similar approach as described in clause 5.5.2 of 3GPP TS 26.260 could be chosen, which is used for e.g., the generation of all test signals in receiving direction and for electrical interface UE in sending direction.

-	Here a certain "virtual far-end reference scenario" could be defined (e.g., a talker at 1 m distance in front of a device under free-field or slightly reverberant conditions).

-	The immersive audio test signals per coded-format are then generated by applying the IVAS external renderer according to 3GPP TS 26.254 [7] on a mono source signal with metadata (ISM1). For MASA, the metadata could also be used directly, in a similar way as already specified in clause 5.5.2 in 3GPP TS 26.260 [4].

-	However, due to different rendering methods, it should be checked that comparable crest factors are achieved for each coded format. The signal level calibration based on ITU-R BS.1770 (see clause 5.5.1 in 3GPP TS 26.260 [4]) might also have different impacts per coded format, since it includes an additional rendering step to 7.1+4 format.

-	Consider a completely different approach (instead of adapting the existing test signal)?

### Double talk capability

When testing performance of double talk capabilities, a commonly used approach is to use a simulated (pseudo)conversation with partly/time-variant overlapping between far-end and near-end talker. 3GPP TS 26.131/132 specifies a test signal based on clause 7.3.5 from Recommendation ITU-T P.501 [6], which utilizes several sequences according to Table 1. The main part of the signal (without conditioning) is depicted in Figure 3.

Table 1: Test stimuli for recording of Echo Canceller operation
(from clauses 6/7/8.11.1 of 3GPP TS 26.132)

|  | Conditioning | Single words (segment 1) and full sentence (segment 2) double talk |
| --- | --- | --- |
| Far-end signal | FB_female_conditioning_seq_long.wav | FB_male_female_single-talk_seq.wav |
| Artificial mouth signal | FB_male_conditioning_seq_long.wav | FB_male_female_double-talk_seq.wav |

Figure 3: Double talk test sequence (from clause 7.3.5 of ITU-T P.501)

Since the focus of double talk test methods is on the speech signal transmitted in sending direction, the signal in receiving direction is not pre-distorted/compressed in any way and the default level (-16 dBm0) is used.

To adapt the test signal for UEs supporting immersive audio, the same aspects as for echo cancellation tests in clause 3.2.1 have to be considered/discussed.

## Audio formats

3GPP TS 26.260 currently specifies that by default the IVAS decoder of the reference client has to reproduce the coded format used by the IVAS encoder of the UE. These output signals (and metadata) could in general be used for all kinds of analyses like e.g., level versus time, frequency response, loudness, etc. However, due to the large amount of possible IVAS coded-formats, multiple separate analysis methods would be required. In 3GPP TS 26.260, this is intentionally the case for certain test methods in sending direction (clause 5.6 of [4]), where it is beneficial to directly analyze the IVAS decoder output of the reference client.

On the other hand, other existing test methods in 3GPP TS 26.260 in sending direction apply an additional mono rendering step, which has two big advantages for the following analysis:

1)	Separate analyses for each coded format are avoided,

2)	existing common analysis methods can be applied and allow comparison to mono telephony (works for basic parameters like delay or level/loudness rating).

A common coded format for the analysis seems beneficial and should also be applied for immersive UEs.

Rendering to mono could also be useful for the analysis of echo signals, since any available/imaginable evaluation metric will always obtain better results for reducing residual echo components.

For the assessment of double talk capabilities, a possible common coded format could be also binaural or stereo instead of mono (see next clause).

## Analysis methods

### Echo cancellation

In 3GPP TS 26.131/132, the echo loss according to Recommendation G.122 [8] is calculated, which performs a simple weighted sum across the 1/3rd octave bands ratio between the signal inserted at POI into receiving direction and the signal captured at POI in sending direction. The aggregated "terminal coupling loss" (TCL) is evaluated in dB and used in performance requirements.

For a possible adaptation in TS 26.260, the rendered mono signal as described in the previous clause 3.3 could be used. Note that also the signal in receiving direction is used as a reference and also requires an additional mono rendering step.

Besides these basic measurements, other evaluation metrics could be considered:

-	In related test specifications for speech terminals, like e.g., ETSI ES 202 740 [9], an additional analysis performed on the same recording as for TCL. In order to consider not only average echo level versus frequency but also short-time echo components, a level-vs-time analysis is used.

-	A more advanced method for evaluating residual echoes is specified in ETSI TS 103 802 [10]. This prediction model provides an estimate in terms of MOS according to a specified listening test design. It performs a more "holistic" analysis of the (simulated) end-to-end conversation as illustrated in Figure 1, by e.g., considering the self-masking of the far-end talker's own voice.

### Double talk capabilities

In 3GPP TS 26.131/132, the double talk capabilities are evaluated with a metric that is specified in Annex B of 3GPP TS 26.132 [2]. The measurement is conducted in two separate runs (with and without active receiving direction, near-end signal is always active), for which the corresponding level-vs-time representations are calculated. From the level difference versus time, a kind of 2D-histogram is calculated, as shown in Figure 4. It provides information about the degree and durations of identified level differences by means of discrete categories shown in Table 2.

Figure 4: Classification of level differences versus durations (from 3GPP TS 26.132)

Table 2: Categories for echo canceller performance classification

| Category | Level difference (ΔL) | Duration (D) | Description |
| --- | --- | --- | --- |
| A1 | -4 dB ≤ ΔL < 4 dB |  | Full-duplex and full transparency |
| A2 | -15 dB ≤ ΔL < -4 dB |  | Full-duplex with level loss in Tx |
| B | ΔL < -15 dB | D < 25 ms | Very short clipping |
| C | ΔL < -15 dB | 25 ms ≤ D < 150 ms | Short clipping resulting in loss of syllables |
| D | ΔL < -15 dB | D ≥ 150 ms | Clipping resulting in loss of words |
| E | ΔL ≥ 4 dB | D < 25 ms | Very short residual echo |
| F | ΔL ≥ 4 dB | 25 ms ≤ D < 150 ms | Echo bursts |
| G | ΔL ≥ 4 dB | D ≥ 150 ms | Continuous echo |

However, so far, no suitable performance requirements could be identified in 3GPP TS 26.131 [3] and only characterization is required - even though (or: because?) this methodology provides a lot of results from a single measurement.

Besides this measurement, other evaluation metrics and/or modifications could be considered:

-	Recommendation ITU-T P.502 [11] applies a similar approach as the aforementioned methodology (conduct two-run measurements and analyzing the level difference versus time), but simply provides an estimate of the overall double talk attenuation in dB. The method is referenced in several test specifications for speech terminals, like e.g., ETSI ES 202 740 [9], in which often also performance requirements are defined and could be adapted.

-	Independent of the analysis method itself, it should be considered to repeat double talk tests with certain level offsets. In ETSI ES 202 740 [9] for example, the following offsets (relative to nominal levels) are additionally tested in order to ensure that louder/softer talkers or an increased signal level in receiving direction have an impact on double talk capabilities:

-	SND: 0 dB / RCV: 0 dB (default levels)

-	SND: -6 dB / RCV: +6 dB (soft talker, increased receive signal level)

-	SND: +6 dB / RCV: +6 dB (loud talker, increased receive signal level)

-	Similar to the aforementioned analysis of echo cancellation performance, the captured signal in sending direction should be rendered to a common output format. Even though mono output would in general work here as well but rendering to stereo or binaural might provide some additional information (e.g., if spatial cues introduced at the near-end by rotation/movements of the talker/device are maintained in the transmitted signal).

## Immersive testing aspects

All considerations/thoughts in the present document so far address the question of whether existing test/analysis methods and signals from traditional mono telephony can be adapted/modified for testing immersive UEs at all. In addition, new test methods should also feature aspects that are relevant for immersive audio:

-	Evaluate different (virtual) source positions (or even a trajectory) in the signal inserted into receive directions (either instead of or in addition to a static scenario)

-	Evaluate multiple (physical) orientations/positions of the RCV-UE-type relative to the SND-UE-type (and/or vice versa) – if applicable, which may depend on the involved SND/RCV-UE types.

-	Besides evaluating different static orientations/positions (changes between measurements), tests should also contain dynamic changes of orientations/positions (during measurement) to vary the echo path versus time.

-	Alternatively, tools like the "rotating surface/panel" could be used to reproduce a time-variant echo path, which is already used in multiple test specifications for speech communication terminals (see for example clause 5.2.3 of ETSI TS 103 607 [12]).

# Conclusion

The present document introduced a general view on performance assessment methods for acoustic echo control functionality in IVAS-enabled UEs. Test setups, signals, methods and requirements from existing specifications like 3GPP TS 26.131/132 [2] [3] and others were investigated with regard to if and up to which degree they are also suitable for immersive terminals. In general, the adaptations of several existing test methods are possible and could be considered in the scope of ATIAS_Ph2.

Although quality and performance indicators for the QoE of immersive communication are still a subject of research, full-duplex communication is certainly a highly relevant factor. Thus, new test methods and requirements in 3GPP TS 26.260/261 [4] [5] should include dynamic changes (i.e., during the measurement) of the echo path, which can be realized either virtually (e.g., by a trajectory in the source signal) or physically (e.g., by rotating UE, talker or a panel).

# References

| [1] | 3GPP SP-241314, “WID on Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2,” HEAD acoustics GmbH, Qualcomm Incorporated, Nokia Corporation, Dolby Sweden AB, Orange, Xiaomi, Melbourne, 2024. |
| --- | --- |
| [2] | 3GPP TS 26.132, "Speech and video telephony terminal acoustic test specification," Release-18. |
| [3] | 3GPP TS 26.131, “Terminal acoustic characteristics for telephony; Requirements,” Release-18. |
| [4] | 3GPP TS 26.260, “Objective test methodologies for the evaluation of immersive audio systems,” Release-18. |
| [5] | 3GPP TS 26.261, “Terminal audio quality performance requirements for immersive audio services,” Release-18. |
| [6] | ITU-T Recommendation P.501, “Test signals for use in telephony and other speech-based applications,” 06/2022. |
| [7] | 3GPP TS 26.254, “Codec for Immersive Voice and Audio Services (IVAS); Rendering,” Release-18.1.0. |
| [8] | Recommendation ITU-T G.122, “Influence of national systems on stability and talker echo in international connections,” 03/1993. |
| [9] | ETSI ES 202 740 v1.8.2, Transmission requirements for wideband VoIP loudspeaking and handsfree terminals from a QoS perspective as perceived by the user, 2022-05. |
| [10] | ETSI TS 103 802 v1.1.1, “Speech and multimedia Transmission Quality (STQ); Objective test method for the evaluation of echo impairments,” 2023-07. |
| [11] | Recommendation ITU-T P.502, “Objective test methods for speech communication systems using complex test signals - New,” in Updated Appendix III - Automated double talk analysis procedure, 09/2014. |
| [12] | ETSI TS 103 607 v1.1.1, Speech and multimedia Transmission Quality (STQ); Transmission requirements for wearable wireless wideband terminals from a QoS perspective as perceived by the user, 2024-07. |