3GPP TSG-SA WG4 Meeting #130	S4-241856

# USA, Orlando, 18 – 22 November 2024

Source:	InterDigital Europe

Title:	haptic media traffic characteristic

Spec:	26.854

Agenda item:	10.8

Document for:	Agreement

1. Introduction

This contribution builds upon S4-241855 and proposes to document the traffic characteristics of haptic media streams, in a manner inspired from 26.925.

2. Reason for Change

Adding initial text related to haptics media traffic characteristic in the TR, to provide evidence on how the traffic in 3GPP haptic enhanced media services may be impacted.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change  all new * * * *

## X Typical haptics media traffic characteristics

### X.1 Typical traffic characteristics summarized per use cases and formats.

In general, haptics media PCM coded bitstreams require substantially more bandwidth than parametric coded bitstreams, this is due to the capability of having silent units in a parametric coded bitstream.

When coding repetitive haptics media effect, a key difference between a parametric coded bitstream and a pcm coded bitstream on the traffic characteristic is the following:

- 	In a coded parametric bitstreams, identical or similar effects can be coded and sent a single time and then referenced, rather than coding multiple time the same or similar haptics media effect

- 	In a pcm coded bitstream, similarly to audio and video, coding performance improves for these repeated consecutive effects but provide a much higher overhead compared to the parametric coded bitstream.

The following table summarise the expected characteristics and average bitrates per channel applicable for the use cases of section 5. In this table the density represents the quantity of haptics effects over the duration of the sequence, it is not related to the intensity of the haptics effect. Three parameters impact the bandwidth requirements: the number of channels, the media format and the density.

Table X.1-1: summary of typical haptics media traffic characteristic.

| Use case | Nb of channels | Media Format | Density | Average bitrate per channel |
| --- | --- | --- | --- | --- |
| Haptic enhanced media distribution (5.2) | 1 to 32 | Parametric | Light Medium High | 0.25 to 0.75 kbps 0. 5 to 1.5 kbps 1 to 5 kbps |
|  | 1 to 32 | Time sampled | Light Medium High | From 6 to 64kbps depending on the density and the quality of the desired signal. 8-16 kbps for good quality at medium and high density. 32-64 kbps for very high quality at medium and high density. |
| Haptic enhanced communication (5.3) | 1 to 4 | Parametric | Light Medium | 0.25 to 0.75 kbps 0.5 to 2 kbps |
| Immersive Entertainment (5.4) | 1 to 32 | Parametric | Light Medium High | 0.25 to 0.75 kbps 0. 5 to 1.5 kbps 1 to 5 kbps |
|  | 1 to 32 | Time sampled | Light Medium High | From 6 to 64kbps depending on the density and the quality of the desired signal. 8-16 kbps for good quality at medium and high density 32-64 kbps for very high quality at medium and high density |
| Immersive multi-modal XR and metaverse (5.5) | 6 to 32 | Parametric | Light Medium High | 0.25 to 0.75 kbps 0. 5 to 1.5 kbps 1 to 5 kbps |
|  | 6 to 32 | Time sampled | Light Medium High | From 6 to 64kbps depending on the density and the quality of the desired signal. 8-16 kbps for good quality at medium and high density 32-64 kbps for very high quality at medium and high density* |

* Note1: TR 22.847, mentioned a bitrate of up to 200kpbs, which may be related to use cases for close-loop tactile internet, robotics etc.

Note 2: values in the table may be re-evaluated if more tests are done.

* * * Next Changes all new * * * *

# Y QoS

## Y.1 Introduction

TBD

## Y.2 A-synchronicity between haptics and other media

In Haptics enhanced media services, the quality of experience may be impacted by the a-synchronicity of the haptics media with either audio, or visual information. This a-synchronicity may be due to various QoS factors which are FFS.

Studies demonstrate (such as [111] and [222]) that haptics effects perceived prior visual cues is less detrimental to the experience than the reverse and that a-synchronicity is more tolerable in passive than active user involvement.

The suitable threshold for a-synchronicity [222], [111], [22.847], [TR 22.856] is listed in table Y.2-1

Table Y.2-1: Tolerable a-synchronicity thresholds per use-cases

| Use case | Media | Tolerable a-synchronicity threshold (note 1) |  |
| --- | --- | --- | --- |
| Haptic enhanced media distribution (5.2) | audio-haptics | audio delay: TBD | haptic delay: TBD |
|  | visual-haptics | visual delay: TBD | Haptic delay: TBD |
| Haptic enhanced communication (5.3) | audio-haptics | audio delay: TBD | haptic delay: TBD |
|  | Visual-haptics | Visual delay: TBD | Haptic delay: TBD |
| Immersive games (5.4 and 5.5) [22.847] | audio-haptics | audio threshold: 50 ms | haptic threshold: 25 ms |
|  | visual-haptics | visual delay: 15 ms | Haptic delay: 50 ms |
| Immersive entertainment (5.4) [TR 22.856] | audio-haptics | audio delay: 25 ms | haptic delay: 12 ms |
|  | visual-haptics | visual delay: 20 ms | Haptic delay: 30ms |
| Immersive multimodal XR and metaverse (5.5) | audio-haptics | audio delay: TBD | haptic delay: TBD |
|  | visual-haptics | Visual delay: TBD | Haptic delay: TBD |
|  | NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

Note: Further work needed to complete and refine these values.

* * * next Change add to reference * * * *

[222]	I. M. Vogels, “Detection of temporal delays in visual-haptic interfaces,” Human Factors, vol. 46, no. 1, pp. 118–134, 2004

[111] 			https://dl.acm.org/doi/pdf/10.1145/3625468.3647626 ; retrieved nov.11 2024.

[22.856]		3GPP TR 22.856, “Study on Localized Mobile Metaverse Services.”

* * * End of Changes * * * *