3GPP TSG-SA WG4 Meeting #130	S4-241858

# USA, Orlando, 18 – 22 November 2024

Source:	InterDigital Europe, Tencent

Title:	Pseudo-CR on Haptics media in 3GPP services

Spec:	26.854

Agenda item:	10.8

Document for:	Agreement

1. Introduction

This contribution addresses part of item (5) of the sid:

“Identify 3GPP streaming and communication services that may benefit from the haptics media type and identify the gaps and requirements need to integrate haptics experiences.”

2. Reason for Change

The contribution addresses one main objective of the SID, the work would be incomplete without it.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change  all new* * * *

## 8.X		Haptics media integration in 3GPP services

### 8.X.1	5G Media Streaming services

The support of Haptic media streaming over 5G would enable the use-cases defined in section 5, particularly those for on demand and live streaming.

The 5GMS architecture [26.501] defines in table 4.0.1-1 the list of main features of 5GMS, highlighting whether they are specified for the uplink and/or downlink. These features are equally applicable to Haptics media as illustrated in Table 8.X.1-1. Priority for haptics media integration in 5G streaming services is on the downlink path.

Table 8.X.1-1applicability of the 5GMS features to haptics enhanced media streaming services.

| Feature | Procedures applicable for haptics media |  |
| --- | --- | --- |
|  | Downlink media streaming | Uplink media streaming |
| Content hosting | Yes | Not applicable |
| Content publishing | Not applicable | Yes /lower priority |
| Content preparation | Not defined | Not defined |
| Network assistance | Yes | Yes |
| Dynamic policies | Yes | Yes |
| Remote control | Not applicable | Yes/ lower priority |
| Consumption reporting | Yes | Not applicable |
| QoE metrics reporting | Yes | Not applicable |
| Edge processing | Yes |  |
| eMBMS delivery | Yes | Not applicable |
| Data collection, reporting and exposure | Yes | Yes |
| Service URL handling | Yes | Yes |

In addition to the generalized architecture described in section 8.2 of this document, the 5GMS specification [26.501] details the UE functions of the 5GMSd client for downlink, in section 4.2.2 and uplink in section 4.2.3.

For handling haptic media, no new UE subfunctions are needed. The following highlights some requirements needed within some of the subfunctions described in [26.501] to support haptic media:

-	Media Access Client: Accesses media content, such as DASH-formatted media segments, for immediate or delayed consumption. To access haptic media content, the haptic media content needs to be provided in suitable formatted media segments (e.g. Dash segments)

- 	Media encoder and decoder: Decodes or compress the media, such as audio, video or haptics media.

- 	Media Capturing: Devices such as video cameras or microphones that transform an analogue media signal into digital media data. For haptic media, capturing devices, sensors are described in section 6.x of this document

- 	Metrics Collection and Reporting and Consumption Collection and Reporting: Information specific to haptic media, if any are FFS.

### 8.X.2	Broadcast services

This section is FFS

### 8.X.3	Real time media communication

#### 8.X.3.1 WebRTC services

In addition to the generalized architecture described in section 8.2 of this document, the GA4RTAR specification in section 4.2 [26.506] details the UE functions of the RTC endpoint. For handling haptic media, no new UE subfunctions are needed. The RTC Access Function is extended to support accessing, coding, and transmitting haptic media.

The protocol stack for a basic RTC endpoint is specified in section 13.1 of [26.113].

Figure 8.X.3.1-1 illustrates the integration of haptics media support in this protocol stack.

Haptics Media

Figure 8.X.3.1-1 Haptics media support in the protocol stack for a basic RTC endpoint

Haptics media is represented at the same level as the other Video and Audio media. While there is no change to the structure of the protocol stacks, RTP payload for Haptics media would be handled similarly to those necessary for Audio and Video.

#### 8.X.3.2 IMS based RTC services

IMS based RTC services include IBACS [26.264] and MTSI [26.114], in which Haptics media can be integrated to satisfy the use case of section 5. Figure 4.3 of [26.114] specifies the user plane protocol stack for a basic MTSI client. It also applies to IBACS.

Figurr 8.X.32-1 illustrates the integration of haptics media support in this protocol stack.

Haptics media

Figure 8.X.3.2-1 Haptics media support in the user plane protocol stack for a basic MTSI endpoint

Haptics media is represented at the same level as the other media types. While there is no change to the structure of the protocol stacks, Payload formats and RTP payload for Haptics media would be handled similarly to those necessary for Audio and Video.

## 8.X Identified gaps.

This section is TBD and will summarize the gaps identified in previous sections.

* * * End of Changes * * * *