TSG-SA WG4 Meeting #130	S4-241859

# USA, Orlando, 18 – 22 November 2024

Source:	InterDigital Europe

Title:	Pseudo-CR on Haptics technologies

Spec:	3GPP TR 26.854

Agenda item:	10.8

Document for:	Agreement

1. Introduction

In accordance with the SID and work plan, this contribution provides initial text for technologies that are suitable to address the use case, and 3GPP services when haptic media may be considered.

2. Reason for Change

Provides text for haptic media technologies.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854.

* * * First Change * * * *

# 7 	Candidate technologies

Editor’s note:  existing standards/work done in other SDOs

## 7.1	Codecs

### 7.1.1	MPEG Haptics Coding

#### 7.1.1.1	Overview

JTC 1/SC 29/WG 7 (MPEG-3DGH) has completed the development of the MPEG Haptics Coding standard at the April 2023 meeting and is pending publication as [ISO/IEC 23090-31].

In addition to a codec and bitstream format, this MPEG Haptic coding standard supports multiple haptic modalities and specifies a human readable JSON coded representation format (HJIF) for descriptive and quantized media formats. Haptic primitive from AHAP and IVS parametric signals can be losslessly converted to and from HJIF. The HJIF data model is a flexible hierarchical structure that can describe one or more channels, allowing for mono, stereo or multi-channel haptics media experiences, targeting one or more actuators or devices. Application areas especially targeted for the use of MPEG Haptics coding include media streaming and broadcast, immersive applications and XR services, real time communications, as well as tactile communication, the latter being out of scope of this study.

During the standardization process, MPEG also worked on a reference and conformance software [ISO/IEC 23090-33] that was publicly released. An in-depth evaluation of the performances of this reference codec was conducted with both objective metrics (PSNR) and subjective tests (MUSHRA) for different target bitrates and different input test streams. The encoding of parametric input content is usually around 1-4 kbps with lossless coding. The results on PCM input data show that signals encoded with the 2 kbps target bitrate present perceptible distortions with an average PSNR of 24db. Signals encoded with a 8 kbps target bitrates show some distortions, but not annoying, with an average PSNR of 39.17db. Finally signals encoded with a 16kpbs bitrate have no perceptible distortion with an average PSNR of 45.62db

#### 7.1.1.2	MPEG Haptics Codec Architecture

Figure 7.1.1.2-1 represents the MPEG haptics codec architecture. Media formats supported by the MPEG Haptics representation and coding includes AHAP, HJIF, IVS parametric media formats, as well as the WAV time sampled media format.

The synthesizer is not defined by the standard and is illustrated in this figure to highlight how the MPEG Haptics codec can be integrated with a renderer.

Fig 7.1.1.2-1 – MPEG Haptics codec architecture.

#### 7.1.1.3 Integration in MPEG-SD

MPEG Scene Description [MPEG SD] is used in IBACS [IBACS] and in [SR-MSE] via MeCAR [MECAR]. To address use-cases in section 5.4 and 5.5 and these 3GPP services, Haptics media may be integrated with a scene description.

MPEG Scene Description provides extensions for the support of haptics media.

Note: further details are TBD.

### 7.1.2	IEEE

#### 7.1.2.1 Overview

The [IEEE 1918.1.1-2024], IEEE Standard for Haptic Codecs for the Tactile Internet was developed in 2016 and approved in 2024. This standard specifies 3 different codecs (no-delay kinesthetic codec, delay-robust kinesthetic codec, and a tactile codec) supporting only a time-sampled media format (PCM) and no parametric media formats.

## 7.2	Storage format

A specification for the storage and delivery signaling for haptics media as defined in [ISO/IEC 23090-31]  is currently under development by MPEG and has reached FDIS stage. ISO/IEC 23090-32 [yyyy] defines how a haptics MIHS bitstream can be encapsulated in ISOBMFF media containers. The specification supports single and multi-track encapsulation, where different channels or bands of the haptics media can be stored in separate tracks to enable selective access.

MPEG is also working on a reference software implementation for the storage and delivery aspects defined in ISO/IEC 23090-32 as part of a new specification (ISO/IEC 23090-37 [zzzz]) that has recently been initiated and is expected to be completed by the end of 2025.

## 7.3	Transport protocols

### 7.3.1	Haptics media delivery over DASH

ISO/IEC 23090-32 also specifies how haptics media is signaled in an MPEG-DASH manifest (MPD) for adaptive media delivery and defines descriptors to signal information pertaining to the haptics experience to allow a streaming client to select the parts of the haptics media to stream based on playback timeline, network conditions, and/or user interaction.

### 7.3.2	Haptics top-level media type and subtypes

The 'haptics' media type has been documented and registered by IANA as a top-level media type, along with 'audio', 'video', 'application', and others. [IANA].

Under this top-level media type, the following haptics subtypes are currently registered:

- ivs, haptics/ivs

- hjif, haptics/hjif

- hmpg, haptics/hmpg

The justification for the 'haptics' top-level media type is found in [RFC-haptics-type]

### 7.3.3	Haptics media RTP payload

An RTP payload format for haptics media is under development in IETF [RTP-haptics] and has reached the working group last call. The draft describes how the haptic data, in MIHS units defined in [ISO/IEC 23090-31] can be transmitted using the RTP protocol. This RTP payload format enables the transport of the “hmpg” media subtype defined in [RFC-haptics-type].

Some of the characteristics of the payload formats include:

- The 4 types of MIHS units are indicated in the payload header: initialization, time dependent (temporal), time independent (spatial), and silent units.

- A MIHS unit can be marked as independent or dependent. An independent unit resets the previous haptics effect and correspond to a “sync” MIHS unit as defined in [ISO/IEC 23090-31].

- 3 payload structures are defined and can be used within a same stream: a single unit payload structure for a single MIHS unit per packet, a fragmented unit payload structure for MIHS units which are too large to be transmitted in a single packet, and an aggregation packet payload structure to transport multiple MIHS units in a single RTP packet.

- The aggregation packet payload structure can be used to transport multiple MISH units that correspond to the same timestamp, single-time aggregation packet (STAP), or to transport multiple MISH units that correspond to different timestamps, multi-time aggregation packet (MTAP).

- In case of congestion control, the draft recommends prioritizing initialization units, to treat silent units as less important, and to use the MIHS unit layer information present in the RTP payload header to prioritize packets.

- The draft also describes SDP considerations, to define additional optional parameters that can be used in the SDP exchange.

### 7.3.4	OpenXR APIs

Details are TBD.

* * * Next Change * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR22.847: “Study on supporting tactile and multi-modality communication services”

[3]	3GPP TR26.813: “Study of Avatars in Real-Time Communication Services”

[4]	3GPP TR22.856: “Study on Localized Mobile Metaverse Services”

[HFX]	Haptic editor for full-body immersive experiences, In Proceedings of the 24th ACM Symposium on Virtual Reality Software and Technology 2018

[OPENXR]	Khronos: “OpenXR 1.1.40 Specification (with all registered extensions)”

[ISO-90-31] 	ISO/IEC 23090-31 “Information technology — Coded representation of immersive media Part 31: Haptics coding”

[bHaptics]	bHaptics Designer: https://docs.bhaptics.com/designer/ retrieved Sept 15th 2024.

[MetaStudio]	Meta Haptics Studio : https://www.meta.com/experiences/meta-haptics-studio/6759764157450104/ retrieved Sept 15th 2024.

[interHC]	Interhaptics Haptic Composer: https://www.interhaptics.com/tech/haptic-composer retrieved Sept 15th 2024.

[metaUty]			Meta Haptics SDK for Unity: https://developers.meta.com/horizon/documentation/unity/unity-haptics-sdk retrieved Sept 15th 2024.

[metaUreal]	Meta Haptics SDK for Unreal: https://developers.meta.com/horizon/documentation/unreal/unreal-haptics-sdk, retrieved Sept 15th 2024.

[bhapSDK]		bHaptics SDK: https://docs.bhaptics.com/sdk/ , retrieved Sept 15th 2024.

[ACHSDK]	Apple Core Haptics SDK: https://developer.apple.com/documentation/corehaptics, retrieved Sept 15th 2024.

[interPlat] 		Interhaptics Platform: https://www.interhaptics.com/platform-partner/ , retrieved Sept 15th 2024.

[G3X gen2]	Qualcomm  Snapdragon G3X gen2: https://docs.qualcomm.com/bundle/publicresource/87-69021-1_REV_A_Snapdragon_G3x_Gen_2_Gaming_Platform_Product_Brief.pdf, retrieved Sept 15th 2024.

[yyyy] 	ISO/IEC 23090-32, “Information technology — Coded representation of immersive media — Part 32: Carriage of Haptics Data”

[zzzz] 		ISO/IEC 23090-37, “Information technology — Coded representation of immersive media — Part 37: Conformance and Reference Software for Carriage of Haptics Data”

[IANA]	https://www.iana.org/assignments/media-types/media-types.xhtml#haptics retrieved Oct 24th.2024

[RFC-haptics-type]		https://datatracker.ietf.org/doc/draft-ietf-mediaman-haptics/, retrieved Oct 24th, 2024.

[RTP-haptics]		https://datatracker.ietf.org/doc/html/draft-ietf-avtcore-rtp-haptics, retrieved Oct 24th, 2024.

[IEEE 1918.1.1-2024] 	1918.1.1-2024, “IEEE Standard for Haptic Codecs for the Tactile Internet”

[MPEG SD]		ISO/IEC 23090-14, “Information technology — Coded representation of immersive media Part 14: Scene description”

[IBACS]	3GPP TS 26.264, “IMS-based AR Real-Time Communication.”

[SR-MSE]		3GPP TS 26.565, “Split Rendering Media Service Enabler”

[MECAR]	3GPP TS 26.119, “Media Capabilities for Augmented Reality”

* * * Next Change * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

AI	Artificial Intelligence

AVH	Audio Video Haptics

HJIF	Haptic JSON Interchanged Format

HMD	Head Mounted Display

XR	Extended Reality

MIHS		MPEG-I Haptic Stream

MTAP 		multi-time aggregation packet

STAP		single-time aggregation packet

* * * End of Changes * * * *