3GPP TSG SA WG4 Meeting #130	S4-241914

Orlando, US, 18-22 November 2024

# Title:	DRAFT Reply LS on Introduction of Extensions to IP Packet Filters for Differentiated QoS Handling for Multiplexed Media Flows

# Response to:	S2-2411001 (S4-241812)

# Release:	Rel-19

# Work Item:	XRM_Ph2

Source:	L.M. Ericsson Limited (to be: SA WG4)

To:	SA WG2

CC:	CT3

Contact person:	Bo Burman

bo (dot) burman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

# 1	Overall description

SA4 thanks SA2 for their liaison on Extensions to IP Packet Filters.

Question 1:

SA2 would like to know if SA4 believes the chosen IP Packet Filter Set extension will suffice to handle needed differentiation for the intended, multiplexed IP traffic flows, as described above and if, additional information is needed, for example Media Identification (MID).

Answer 1:

SA4 believes that the (S)RTP header fields SSRC and PT are sufficient to identify and differentiate separate (S)RTP streams in a multiplexed IP traffic flow.

As defined in and described by IETF RFC 3550, SSRC is an (S)RTP session-unique identifier that identifies the (S)RTP stream. PT is a dynamically allocated identifier used as reference to media signaling (typically SDP), where the encoding format configuration for the (S)RTP payload is defined. Depending on application use cases, a single (S)RTP stream (SSRC) may use one single (S)RTP payload encoding format (PT) for the entire RTP stream, or use multiple different encoding formats that may thus differ between (S)RTP packets. In other application use cases, multiple (S)RTP streams (SSRC) may reuse the same (S)RTP payload encoding format (PT value), or each (S)RTP stream may use a separate RTP payload encoding format.

SA4 would like to highlight the benefit of both SSRC and PT being mandatory (S)RTP header fields that are present in all (S)RTP packets and all implementations. However, SA4 is aware that for some (S)RTP/(S)RTCP implementations, the SSRC field value might not be available to the signaling plane before the (S)RTP session is started.

Regarding potential use of (S)RTP/(S)RTCP SDES MID as described by IETF RFC 9143, SA4 considers this to be a simplified (S)RTP stream identification for the cases where SSRC is not known before the (S)RTP session starts. Use of MID information is required by implementations of W3C WebRTC 1.0, in both SDP signaling, in (S)RTP Header Extension, and in (S)RTCP SDES packets, as described by IETF RFC 8834. SA4 is not aware of any other (S)RTP/(S)RTCP implementations beyond WebRTC 1.0 and Gstreamer RTP stack that make use of SDES MID. Use of RTP Header Extension encryption according to RFC 6904 would allow for MID to be unencrypted even if other RTP Header Extensions are encrypted. Use of RTP Header Extension encryption according to RFC 9335 encrypts all RTP Header information and would make MID inaccessible for Packet Filters. There may thus exist (S)RTP/(S)RTCP implementations that neither have SSRC value available before (S)RTP session start, nor can make use of SDES MID either because it is not implemented or is encrypted.

Question 2:

SA2 would also like to ask if, when differentiated mapping of (S)RTP media streams to QoS Flows is applied, there may be also a need to map certain (S)RTCP packet types to the same QoS Flow as the (S)RTP media streams they control?

Therefore, SA2 would also like to ask SA4 to guide SA2 on differentiated handling of (S)RTCP packets, and also whether Packet Type and SSRC value would suffice as extended packet filters.

Answer 2:

Regarding guidance of potential differentiated handling of (S)RTCP packets, SA4 believes that providing the same handling for both (S)RTP and (S)RTCP packets will be desirable in the absolute majority of cases. Any other approach would need explicit motivation and study of potential benefits and drawbacks.

SA4 believes that (S)RTCP Packet Type and SSRC value would suffice as extended (S)RTCP packet filters, in case it would be motivated to separate out specific (S)RTCP packets or Packet Types.

To summarize, SA4 suggests the following: (S)RTP multiplexed media Packet Filter information should, depending on availability, be including at least either:

- Synchronization Source (SSRC) and / or;

- Payload Type or Packet Type (depending on whether the Packet Filter is for (S)RTP or (S)RTCP)

or,

- Media Identification (MID)

SA4 would like to add that depending on which of the cases above applies, either multiple (SSRC, PT) or multiple MID values can be included in the Packet Filter information.

# 2	Actions

To SA WG2

ACTION: 	SA4 kindly asks SA2 to take the above provided answers into consideration.

# 3	Dates of next SA WG4 meetings

SA4#131:	17 - 21 February, 2025	Geneva, Switzerland

SA4#131bis-e:	7 - 11 April, 2025		Electronic meeting

SA4#132:	19 - 23 May, 2025		Japan