3GPP TSG-SA4 Meeting #130	S4-241961

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0006 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further corrections to Annex A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Sweden AB, Ericsson LM, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The IVAS codec specification main part specifies certain features for which Annex A does not specify the RTP/SDP handling correctly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct RTP/SDP handling of these features is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementers may become confused to find IVAS codec features described in the specification main part but left without proper description of how the features can be used in an IVAS media session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

# A.3	Payload Format

## A.3.1	Format Overview

The RTP Payload Format described in this document addresses the specific requirements of the IVAS codec. The format supports the transmission of IVAS Immersive mode frames or EVS coded frames with the following features:

- 	IVAS Immersive mode operation

-	WB, SWB and FB audio bandwidths, respectively 16, 32, and 48 kHz sampling rates

-	all immersive formats of the IVAS codec

-	1-4 independent (mono) streams with meta data (ISM)

-	stereo (including binaural audio)

-	multi-channel in 5.1, 7.1, 5.1+2, 5.1+4, 7.1+4

-	scene-based audio (Ambisonics) up to order 3 (SBA)

-	metadata-assisted spatial audio (MASA)

-	combinations of ISM+MASA (OMASA) and ISM+SBA (OSBA)

-	bitrates ranging from 13.2 kbps to 512 kbps

-	EVS operation

-	supporting all EVS operation modes (mono) of the IVAS codec, including the EVS Primary and AMR-WB IO modes, using a payload syntax compatible to the header-full format defined in Annex A of [3] (with some limitations)

NOTE: The format does not support the compact format, present in Annex A of [3].

-	NB, WB, SWB and FB audio, respectively 8, 16, 32, and 48 kHz sampling rates

-	bitrates ranging from 5.9 (VBR) to 128 kbps

-	20 ms frame duration

-	multiple frames per RTP payload

-	Discontinuous Transmission (DTX)

-	transmission of Processing Information (PI), i.e. PI data, in forward and reverse direction to support the rendering, send requests and feedback

-	switching between EVS (mono) and IVAS (stereo and immersive) operation in the same payload type

## A.3.2	RTP Header Usage

The format of the RTP header is specified in [34]. This IVAS RTP payload format uses the fields of the RTP header in a manner consistent with the usages in [34].

The assignment of the RTP payload type for IVAS is out of scope of this document. In most cases SDP would be used to signal the payload type for dynamic assignment.

The RTP clock rate for IVAS is 16000, regardless of the audio bandwidth. A clock rate of 16000 is also used for the AMR-WB [36] and EVS codecs [3]; having a unique clock rate across all payload types of one media avoids the issues described in [37].

The RTP timestamp defines the sampling instant (media time) of the first sample of the first IVAS frame in an RTP packet. The duration of one IVAS frame is 20 ms. Thus, the media time is increased for each successive IVAS frame of an RTP packet by 320 ticks. The RTP timestamp of a packet is used for the first PI data in the IVAS RTP payload. The timing of PI frames during  DTX is explained in clause A.3.5.4.

The RTP header marker bit (M) shall be set to 1 for the first packet of a talk spurt, i.e. if the first frame-block carried in the RTP packet contains the frame first in a talkspurt. For all other RTP packets the marker bit shall be set to zero (M=0). This is the same usage as described in [35].

## A.3.3	Packet Payload Structure

### A.3.3.1	General

The IVAS encoder generates encoded frames representing 20 ms of speech or audio data. The IVAS payload contains:

-	(optional) E-bytes (including the CMR) for adaptation and indication of optional PI data section;

-	one or more ToC(s) describing the IVAS audio frame(s) included in the payload;

-	IVAS frame data block(s), representing 20 ms of speech or audio data (depending on ToC signaling), and;

-	optional PI data section;

### A.3.3.2	Format Description

An RTP payload comprises the IVAS payload, which consist of the IVAS-specific payload header followed by the frame data and optional PI data as shown in Figure A.3.3.2-1. The frame data consists of one or more IVAS or EVS coded frames (including NO_DATA, see A.3.3.3.2). The optional PI data section can be considered as additional metadata to support the rendering, send requests and feedback. There may be zero-padding bits in addition at the end of the payload. Padding bits shall be discarded by the receiver.

NOTE:	The purpose of padding is that in the case of EVS AMR-WB IO frames, payload data may need to be octet-aligned using zero-padding bits at the end of the payload. EVS Primary frames are by definition octet-aligned (see clause A.2.2.1.4.1 of [3]).

+-----------------------+---------------------+--------------------+----------+
| RTP Header (+ HDREXT) |    payload header   |     frame data     |  PI data |
+-----------------------+---------------------+--------------------+----------+

                        \--------------------\ /------------------------------/
                                         IVAS payload

Figure A.3.3.2-1: RTP Header with IVAS payload structure

### A.3.3.3	Payload Header

#### A.3.3.3.1	General

The IVAS payload header consists of Table of Contents (ToC) bytes and Extra (E) bytes, defined in clauses A.3.3.3.2 and A.3.3.3.3, respectively. The first bit of each as header byte is the Header Type identification bit (H) to identify whether a header byte is a ToC or E byte. If the H bit is set to 0, the corresponding byte is a ToC byte, and if set to 1, the corresponding byte is an E byte. The second bit of a ToC byte is the Following (F) bit (see clause A.3.3.3.2), which if set to 1 indicates that another header byte is following. The last header byte shall be a ToC byte and have the F bit set to 0.

The general structure of a header byte is shown in figure A.3.3.3.1-1.

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|H|   ToC / E   |
+-+-+-+-+-+-+-+-+

Figure A.3.3.3.1-1: Generic structure of a payload header byte.

H (1 bit):	Header Type identification bit. For a ToC byte this is set to 0, for an E byte this is set to 1.

#### A.3.3.3.2	ToC byte

The ToC bytes define the content of the frame data in the IVAS payload following the IVAS payload header. For each IVAS or EVS frame and for each NO_DATA frame (i.e. a frame that has zero size frame data) in the payload there shall be one ToC byte to signal the IVAS mode and bit rate. ToC bytes and the respective frame data shall be in the same order.

The Table of Content (ToC) byte structure is an extension of the ToC byte structure defined in clause A.2.2.1.2 in [3]. In the EVS payload format in [3] a code point in the ToC byte (see Figure A.5 in [3]) for extensions has been reserved, the "Unused" bit. In the present document this "Unused" bit of the Frame type index bits is activated and called "IVAS indicator" to distinguish EVS and IVAS frame data. The specific ToC structure for an IVAS frame is shown in Figure A.3.3.3.2-1.

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|0|F|0|1|  BR   |
+-+-+-+-+-+-+-+-+

Figure A.3.3.3.2-1: Table of Content (ToC) byte structure for an IVAS frame.

F (1 bit):	If set to 1, the bit indicates that the header byte is followed by another header byte. If set to 0, the bit indicates that this header byte is the last one in this payload and no further header bytes follows this entry.

BR (4 bits):	Bit rate index as defined in Table A.3.3.3.2-1 .

Table A.3.3.3.2-1: Frame Type index when EVS mode bit = 0 and "Unused"/IVAS indicator bit = 1

| EVS/IVAS mode bit (1 bit) | IVAS indicator (1 bit) | IVAS bit rate | Indicated IVAS mode and bit rate |
| --- | --- | --- | --- |
| 0 | 1 | 0000 | IVAS 13.2 kbps |
| 0 | 1 | 0001 | IVAS 16.4 kbps |
| 0 | 1 | 0010 | IVAS 24.4 kbps |
| 0 | 1 | 0011 | IVAS 32 kbps |
| 0 | 1 | 0100 | IVAS 48 kbps |
| 0 | 1 | 0101 | IVAS 64 kbps |
| 0 | 1 | 0110 | IVAS 80 kbps |
| 0 | 1 | 0111 | IVAS 96 kbps |
| 0 | 1 | 1000 | IVAS 128 kbps |
| 0 | 1 | 1001 | IVAS 160 kbps |
| 0 | 1 | 1010 | IVAS 192 kbps |
| 0 | 1 | 1011 | IVAS 256 kbps |
| 0 | 1 | 1100 | IVAS 384 kbps |
| 0 | 1 | 1101 | IVAS 512 kbps |
| 0 | 1 | 1110 | IVAS-SR |
| 0 | 1 | 1111 | IVAS 5.2 kbps SID |

The ToC also allows signaling the EVS bit rates defined in Tables A.4 and A.5 in [3] when the EVS/IVAS mode bit is set to 1 or when the EVS/IVAS mode bit and the Unused/IVAS indicator bit are set to 0.

NO_DATA and SPEECH_LOST frames for both EVS and IVAS modes are signalled with the bit combinations in Table A.4 in Annex A of [3].

NOTE: 	Received NO_DATA or SPEECH_LOST frames do not relate to either EVS or IVAS modes but simply indicate a non-existent or lost frame.

Special treatment is done in case of signaling of an IVAS split rendering payload. This case is signaled with IVAS bit rate indicator = “1110” and the EVS/IVAS mode bit set to 0 and the Unused/IVAS indicator bit set to 1. In this case an SR-ToC byte follows unconditionally, which indicates the IVAS split rendering bit rate. The structure of the SR-ToC byte shown in Figure A.3.3.3.2-2.

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|0|F|ID | SR-BR |
+-+-+-+-+-+-+-+-+

Figure A.3.3.3.2-2: Structure of SR-ToC byte

F (1 bit):	This bit shall mirror the F bit of the preceding ToC byte. A receiver should ignore this bit and parse the F bit in the preceding ToC byte.

ID (2 bits):	Identifier information identifying the diegetic and non-diegetic streams in a single payload according to Table A.3.3.3.2-2. The ID value 00 – Default indicates a default single stream in the payload, which may be either diegetic or non-diegetic.

SR-BR (4 bits):	Bit rate index as defined in Table A.3.3.3.2-3.

Table A.3.3.3.2-2: ID field in SR-ToC byte.

| ID | Definition |
| --- | --- |
| 00 | Default |
| 01 | Diegetic |
| 10 | Non-diegetic |
| 11 | Reserved |

Table A.3.3.3.2-3: Indicated IVAS split rendering bit rate (SR-BR)

| SR-BR | Indicated IVAS SR bit rate |
| --- | --- |
| 0000 | Reserved |
| 0001 | Reserved |
| 0010 | Reserved |
| 0011 | Reserved |
| 0100 | Reserved |
| 0101 | Reserved |
| 0110 | Reserved |
| 0111 | Reserved |
| 1000 | Reserved |
| 1001 | Reserved |
| 1010 | Reserved |
| 1011 | IVAS SR 256 kbps |
| 1100 | IVAS SR 384 kbps |
| 1101 | IVAS SR 512 kbps |
| 1110 | Reserved |
| 1111 | Reserved |

#### A.3.3.3.3	E (Extra) byte

##### A.3.3.3.3.1	General

The specific E byte structure in the IVAS payload header is shown in Figure A.3.3.3.3.1-1. E bytes contain extra information and shall precede the ToC bytes of the coded frames they relate to. There may be multiple E bytes preceding a ToC byte. After the initial E-byte with the CMR there may be multiple subsequent E bytes preceding ToC bytes. Subsequent E bytes may be extended by another E byte of the same type.  E bytes may precede any ToC byte; E bytes in the current version of this specification are only permitted before the first ToC byte.

The E (Extra) byte structure is shown in Figure A.3.3.3.3.1-1.

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|1|    E-data   |
+-+-+-+-+-+-+-+-+

Figure A.3.3.3.3.1-1: E (Extra) byte structure

Parsing of one payload header byte follows the state machine of Figure A.3.3.3.3.1-2.

Figure A.3.3.3.3.1-2: State Machine for parsing a Payload Header Byte.

##### A.3.3.3.3.2	Initial E-byte (CMR)

If a codec mode request (CMR) is sent in the current RTP packet, the initial E byte follows the structure of the CMR byte as defined in Figure A.4 of [3]. The previously "Reserved" entries of Table A.3 in 3] when the T (Type of Request) field is 111 of Figure A.4 of [3] are replaced according to Table A.3.3.3.2-1.

Table A.3.3.3.2-1: Structure of the CMR byte for T=111

| Code |  | Definition |
| --- | --- | --- |
| T | D | BR |
| 111 | 0000 | IVAS 13.2 |
|  | 0001 | IVAS 16.4 |
|  | 0010 | IVAS 24.4 |
|  | 0011 | IVAS 32 |
|  | 0100 | IVAS 48 |
|  | 0101 | IVAS 64 |
|  | 0110 | IVAS 80 |
|  | 0111 | IVAS 96 |
|  | 1000 | IVAS 128 |
|  | 1001 | IVAS 160 |
|  | 1010 | IVAS 192 |
|  | 1011 | IVAS 256 |
|  | 1100 | IVAS 384 |
|  | 1101 | IVAS 512 |
|  | 1110 | Reserved |
|  | 1111 | NO_REQ |

CMR code-point "NO_REQ" remains as defined in Table A.3 in [3]; it is specified as equivalent to no CMR-value being sent. The receiver of "NO_REQ" shall ignore it.

The resulting byte structure is shown in Figure A.3.3.3.3.2-1.

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|H|1|1|1|  BR   |
+-+-+-+-+-+-+-+-+

Figure A.3.3.3.3.2-1: Initial E byte structure for IVAS (same as EVS CMR byte structure)

BR (4 bit):	IVAS bit rate as indicated in Table A.3.3.3.2-1.

NOTE:	When operating in IVAS Immersive mode, a received EVS CMR (T=000..110) is be interpreted as a request to switch to EVS operation mode. When operating in EVS mode, a received IVAS (Immersive) CMR (T=111) is be interpreted as a request to switch to IVAS Immersive operation mode.

NOTE: 	In a SR session with two streams (diegetic and non-diegetic), a first Initial E byte (CMR) in a payload header is respective the first stream while a second Initial E byte (CMR) is respective the second stream.

##### A.3.3.3.3.3	Subsequent E-bytes

A.3.3.3.3.3.1	General

Subsequent E byte(s) (after the initial E byte) may follow to request bandwidth, coded format, or to indicate the presence of PI data in the payload as described in the following clauses. Reserved bits in the following E byte structures shall be set to 0, unless defined. The common fields in a subsequent E-byte are:

H (1 bit):	Header Type identification bit. For an E byte this bit is always set to 1.

ET (2 bits): 	Type of subsequent E byte (00, 01, 10, 11) as indicated in Table A.3.3.3.3.3-1. The value 11 is reserved and shall not be used.

Table A.3.3.3.3.3-1: ET field in a subsequent E byte

| ET | Definition |
| --- | --- |
| 00 | Bandwidth Request |
| 01 | Format Request |
| 10 | PI indication |
| 11 | Split Renderer Request |

A.3.3.3.3.3.2	Bandwidth Request

Bandwidth requests are defined as shown in Figure A.3.3.3.3.3.2-1 .

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|H| ET| res | BW|
+-+-+-+-+-+-+-+-+

Figure A.3.3.3.3.3.2-1: Subsequent E byte structure for bandwidth request (ET=00)

BW (2 bits): 	Requested bandwidth as indicated in Table A.3.3.3.3.3.2-1.

Table A.3.3.3.3.3.2-1: BW field in a subsequent E byte

| BW | Definition |
| --- | --- |
| 00 | WB |
| 01 | SWB |
| 10 | FB |
| 11 | NO_REQ |

A.3.3.3.3.3.3	Coded Format Request

Coded format requests are defined as shown in Figure A.3.3.3.3.3.3-1

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|H|ET |res| FMT |
+-+-+-+-+-+-+-+-+

Figure A.3.3.3.3.3.3-1: Subsequent E byte structure for coded format request (ET=01)

FMT (3 bits):	Requested coded format as indicated in Table A.3.3.3.3.3.3-1.

Table A.3.3.3.3.3.3-1: FMT field in a subsequent E byte

| FMT | Definition |
| --- | --- |
| 000 | stereo |
| 001 | SBA |
| 010 | MASA |
| 011 | ISM |
| 100 | MC |
| 101 | OMASA |
| 110 | OSBA |
| 111 | NO_REQ |

NOTE:	Mono is not included in Table A.3.3.3.3.3.3-1as mono coding in IVAS is handled by the EVS modes.

A.3.3.3.3.3.4	PI Indication

PI indication to indicate PI data presence in the payload are defined as shown in Figure A.3.3.3.3.3.4-1 .

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|H|ET |   res   |
+-+-+-+-+-+-+-+-+

Figure A.3.3.3.3.3.4-1: Subsequent E byte structure to indicate PI data presence in the payload (ET=10)

A.3.3.3.3.3.5	Split Renderer Configuration Request

Split Renderer configuration requests are defined as shown in Figure A.3.3.3.3.3.5-1. This subsequent E byte is used to request diegetic or non-diegetic support. Diegetic bit D=0 corresponds to requesting a non-headtrackable stereo, 0-DoF binaural, or non-diegetic stream while Diegetic bit D=1 corresponds to requesting a headtrackable diegetic stream with pose correction possibility at the receiver. In addition, bits Y (yaw), P (pitch), R (roll) are used to request pose correction around the corresponding axes or to request disabling pose correction around them.

The following control fields are defined with the following meaning:

D (1 bit):	If the stream is negotiated as a diegetic split rendering stream, the D bit can be used to control if the stream should have diegetic support. D=’1’ requests enabling diegetic support; D=’0’ requests disabling it.

NOTE:	If the stream is negotiated a non-diegetic split rendering, a sender shall not request diegetic support and a receiver shall ignore the D bit and assume it is ‘0’.

Y (1 bit):	Request of pose correction around yaw axis (z-axis). ‘1’ enables pose correction around this axis, ‘0’ disables it.

P (1 bit):	Request of pose correction around pitch axis (y-axis). ‘1’ enables pose correction around this axis, ‘0’ disables it.

R (1 bit):	Request of pose correction around roll axis (x-axis). ‘1’ enables pose correction around this axis, ‘0’ disables it.

NOTE: 	The number of axis around which pose correction is supported may be limited during session setup. A split renderer configuration request shall not exceed this limit.

0 1 2  3  4 5 6 7 
+-+-+-+---+-+-+-+-+
|H|ET |res|D|Y|P|R|
+-+-+-+---+-+-+-+-+

Figure A.3.3.3.3.3.5-1: Subsequent E byte structure for Split Renderer configuration request (ET=11)

## A.3.4	Frame Data

The RTP payload comprises, apart from headers, one or several coded frames, the Frame Data.

The bits are in the same order as produced by the IVAS encoder, where the first bit is placed left-most immediately following the IVAS payload header.

The format supports the transmission of EVS (primary and AMRWB-IO) and IVAS coded frames.

## A.3.5	Processing Information (PI) data

### A.3.5.1	General

The support of PI data is disabled by default and needs to be explicitly enabled using the pi-types parameter as defined in clause A.4.1.

In addition to one or more IVAS frame(s), one or more PI data may be transmitted as part of the PI data section in the IVAS RTP payload. PI data in the forward direction (i.e., from a media sender to a media receiver) is a mechanism to assist the processing of the IVAS frame(s) at the renderer, where the PI data is acquired prior to the rendering of IVAS frame(s). PI data in the reverse direction (i.e., from a media receiver to a media sender) includes sending requests and feedback to control a media sender. Consequently, PI data provides bi-directional signalling between the media sender and the media receiver.

Figure A.3.5.1-1 presents the structure of a PI data section which is formed by the PI-specific header section followed by the PI frame data. The PI header section (see clause A.3.5.a) identifies the different PI data frames (see clause A.3.5x2). The PI data section is included in the IVAS RTP payload as described in clause A.3.3.2 and in figure A.3.3.2-1.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
 | PI header section |  PI frame data  |
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
 \---------------PI data---------------/

Figure A.3.5.1-1: The structure of a PI data section.

### A.3.5.2	PI data header

As presented in clause A.3.5.1 and in figure A.3.5.1-1, a PI data block contains a header section and a frame data section. Each PI header identifies the type and size for the associated PI data frame. Furthermore, the PI header identifies to which audio frame(s) the PI data is associated with through the PI frame marker bits (PM). All the PI header indications are added to the PI header section in a row before the PI data frames.

PI data header indication structure is presented in figure A.3.5.2-1.

0                             1             _
 0  1  2  3  4  5  6  7  8  9  0  1  2  3  4  5
  +--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+
  |PF| PM  |    PI type   |        PI size        |
  +--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

Figure A.3.5.2-1: PI data header.

The header elements are defined as:

PF (1 bit):	If set to 1, the bit indicates that the PI header indication is followed by another PI header indication. If set to 0, the bit indicates that this PI header indication is the last one in this payload and no further PI header indication follows this entry. See Table A.3.5.2-1.

PM (2 bits):	PI frame marker bits indicate the associated audio frame for the PI data frame, see Table A.3.5.2-2

PI type (5 bits):	PI type bits indicate the type for the PI data as indicated in tables A.3.5.5-1 and A.3.5.5-2.

PI size (8 bits):		PI size bits indicate the size for the PI data frame in bytes, see table A.3.5.2-3. A size of zero indicates that there is no PI data frame associated for the PI header indication.

Table A.3.5.2-1: PF bits for a PI header.

| PF bits | Indication |
| --- | --- |
| 0 | Last header indication |
| 1 | Header indication follows |

Table A.3.5.2-2: Marker bits for a PI header byte.

| PM bits | Frame marker indication |
| --- | --- |
| 00 | Reserved |
| 01 | Not last PI header for this frame |
| 10 | Last PI header for this frame |
| 11 | General (applied to all frames) |

The PI headers are listed in the beginning of a PI data section in order. First, the PI headers with marker bits PM=11 (i.e., the generally applied PI data is identified first). The following PI headers are associated with the audio frames in the payload in order. PM=01 indicates that the current PI header/data is not the last one for the current audio frame and more PI headers/data will follow. PM=10 indicates that this PI header/data is the last one that is associated with the current audio frame. The next PI data is then associated with the next audio frame, i.e. the time stamp of the PI frame is increase by 320 ticks. The parsing of a PI header indication is illustrated in a state machine in Figure A.3.5.2-2.

Figure A.3.5.2-2: State machine for parsing a PI header byte.

Table A.3.5.2-3 indicates the values for the “size” bits in the PI header. A size value of zero indicates that there is no associated PI data for this header entry. A value of (1111 1111) indicates that another size byte follows. The size of the PI data is then calculated as the sum of the respective sizes indicated by the “size” bytes, where a size of 255 bytes is used for the “size” byte with (1111 1111) bit sequence. For example, if there are two “size” bytes, a first byte of (1111 1111) and a second byte of (0000 1111, indicating a size of 15 bytes), the size of the PI data is 255 + 15 = 270 bytes. Recursive size indication with subsequent “size” bytes is also possible.

Table A.3.5.2-3: Size bits for PI header.

| PI Size bits | Indicated size in bytes (or other indication) |
| --- | --- |
| 0000 0000 | 0 |
| 0000 0001 | 1 |
| … | … |
| 1111 1110 | 254 |
| 1111 1111 | Another size byte follows |

### A.3.5.3	Media time when IVAS PI data is included in RTP packets

When the IVAS codec is used, then RTP packets may include both PI data and audio data, and the PI data may need to be synchronized with the audio data.

When forward direction PI data is included in the RTP packets, the following applies:

-	The PI data is associated with an audio frame and use the media time of the audio data.

-	If PI data needs to be transmitted and no audio frame is available, e.g., during DTX periods, then a NO_DATA frame is included in the packet containing the PI data. Alternatively, the PI data can be transmitted with the SID frames. See clause A.3.5.4 for more information.

- 	If PI data is not needed to be transmitted when there is no audio frame available, e.g., during DTX periods, then the transmission of the PI data can be delayed until an audio frame is available. If there are multiple PI data frames with the same type available after a delay period, the most recent PI data may be selected for transmission (e.g., the most recent device orientation may be transmitted). The other (older) PI data frames with the same type may be ignored. Depending on the type of the delayed PI data frames, all of the PI data frames with the same type may be transmitted. See clause A.3.5.4 for more information.

-	When receiving an RTP packet with both PI data and several audio frames:

-	the media time of the first audio frame is calculated from the RTP time stamp,

-	the media time of a subsequent audio frames is calculated from the media time of a preceding audio frame by adding 20 ms.

-	PI data does not add to the media time.

-	PI data can be sent in a separate RTP packet from the audio frame and then use the media time calculated from the RTP time stamp.

### A.3.5.4	PI data handling during DTX

No audio frames are transmitted when DTX operation mode is determined by the media sender. Consequently, there is no RTP time stamp available to be associated with the transmitted PI data. If the PI data is obtained during DTX operation and needs to be transmitted as soon as possible, the PI data can be associated with the next transmitted SID frame or with a NO_DATA frame. In these cases, the RTP time stamp for the PI data transmission is obtained from the media time of the respective SID or NO_DATA frames.

If the transmission of the PI data can wait until the DTX period is over, the transmission of the PI data can be delayed until the next audio frame is available. In this case, the RTP timestamp of the transmitted packet is calculated from the media time of the oldest PI data included in the packet with one or more NO_DATA frames included before the first audio frame.

If the transmission of the PI data is delayed, there can be multiple PI data frames of the same type (e.g., of device orientation type) waiting to be transmitted at the end of DTX operation. In this case, the latest or all PI data can be selected to be transmitted (e.g., the latest device orientation).

### A.3.5.5	Supported PI data types

Supported PI types are listed in tables A.3.5.5-1 and A.3.5.5-2 and described in the following subsections. Table A.3.5.5-1 lists PI types for forward direction signalling. Table A.3.5.5-1.1 lists the PI types for reverse direction signalling. Table A.3.5.5-2 lists additional PI types.

Table A.3.5.5-1 : Supported forward direction PI types in an IVAS session.

| Type bits | Forward direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 00000 | SCENE_ORIENTATION | Describes the orientation of a spatial audio scene in unit quaternions. | fsco | 8 | A.3.5.6.1.2 |
| 00001 | DEVICE_ORIENTATION_COMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is compensated in the transmitted audio. | fdoc | 8 | A.3.5.6.1.3 |
| 00010 | DEVICE_ORIENTATION_UNCOMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is not compensated in the transmitted audio. | fdou | 8 | A.3.5.6.1.3 |
| 00011 | ACOUSTIC_ENVIRONMENT | Selects and optionally describes the acoustic environment. | face | 1,5 or 8 | A.3.5.6.2 |
| 00100 | AUDIO_DESCRIPTION | Describes the content in the transmitted audio | faud | 1 to 4 | A.3.5.6.3 |
| 00101 | DISABLE_HEADTRACKING | Disables headtracking at the media receiver. | fdht | 0 | A.3.5.6.1.4 |
| 00110 | ENABLE_HEADTRACKING | Enables headtracking at the media receiver. | feht | 0 | A.3.5.6.1.4 |
| 00111 | ISM_TYPE | Indicates the ISM type (diegetic/non-diegetic). | ftyp | 2 | A.3.5.6.4.3 |
| 01000 | ISM_DISTANCE_ATTENUATION | Describes distance attenuation for all ISMs. | fida | 3 | A.3.5.6.4.4 |
| 01001 | ISM_ID | Indicates ID of each transported ISM. | fiid | 1 + number of ISMs x 1 | A.3.5.6.4.5 |
| 01010 | ISM_GAIN | Describes gain factor for each ISM. | figa | 1 + number of ISMs x 1 | A.3.5.6.4.6 |
| 01011 | ISM_ORIENTATION | Describes an orientation for each ISM. | fiso | 1 + number of ISMs x 8 | A.3.5.6.4.7 |
| 01100 | ISM_POSITION | Describes a position of each ISM. | fipo | 1 + number of ISMs x 6 | A.3.5.6.4.8 |
| 01101 | ISM_DIRECTIVITY | Describes directivity for each ISM. | fidr | 1 + number of ISMs x 2 | A.3.5.6.4.9 |

Table A.3.5.5-1.1 : Supported reverse direction PI types in an IVAS session.

| Type bits | Reverse direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 10000 | PLAYBACK_DEVICE_ORIENTATION | Feedback. Describes the orientation of the playback device in Quaternions. | rpdo | 8 | A.3.5.7.1 |
| 10001 | HEAD_ORIENTATION | Feedback. Describes the head orientation of the listener in Quaternions. | rhor | 8 | A.3.5.7.2 |
| 10010 | LISTENER_POSITION | Feedback. Describes the position of the listener in 3D space. | rlip | 6 | A.3.5.7.3 |
| 10011 | DYNAMIC_AUDIO_SUPPRESSION | Describes receiver’s preference with respect to audio suppression | rdas | 2 | A.3.5.7.4 |
| 10100 | AUDIO_FOCUS_DIRECTION | Describes a direction of interest for the listener in Quaternions. | rafd | 8 | A.3.5.7.5 |

Table A.3.5.5-2: Additional PI types in an IVAS session.

| Type bits | PI type | Description | SDP indication | Size (bytes) |
| --- | --- | --- | --- | --- |
| 01110-01111, 10100-11110 | Reserved | - | - | - |
| 11111 | NO_PI_DATA | Indicates an empty PI data frame. | nopi | 0 |

NO_PI_DATA PI data type can be used to indicate empty PI data sections. The PM marker bits for a NO_PI_DATA PI data type shall be set as PM=10, see table A.3.5.2-2. For example, if an IVAS RTP payload includes multiple audio frames, and some of the audio frames do not have associated PI data, NO_PI_DATA PI type can be used.

### A.3.5.6	Forward direction PI data types

#### A.3.5.6.1	Orientation PI data (forward direction)

##### A.3.5.6.1.1	Orientation data structures

Figure A.3.5.6.1.1-1 below shows PI orientation data structures in quaternions with 16 bits reserved for each component. The quaternion component values range from -1 to 1 according to Q15, in which resolution for a single component is 215-1-1≈0.00003. The represented quaternion is a unit quaternion. Following the IVAS coordinate system in 7.4.3.1, a quaternion of (w=0, x=1, y=0, z=0) represents the frontal direction. The positive x-axis points towards the frontal direction, the positive y-axis points towards the left direction and the positive z-axis points towards the up direction.

NOTE: 	An orientation in Euler convention can be converted to quaternions before transmission, see clause 7.4.3.2 for the conversion operation and clause 7.4.3.1 for Euler angle definitions in the IVAS coordinate system.

0                   1                   2                   3 _
 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1  
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
 |               W               |               X               |
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
 |               Y               |               Z               |
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.3.5.6.1.1-1: PI orientation data as quaternions.

The received orientations can be transmitted to the external orientation handling and processed as stated in clause 7.4.4.

##### A.3.5.6.1.2	Scene orientation

The SCENE_ORIENTATION PI data describes the orientation of the capturer side scene. The frontal direction of the scene points towards the capture frontal direction. See also clause 7.4.2.1 (Scene orientation). In the PI data, the azimuth and elevation values of the scene orientation are transformed into (unit) quaternions. A frontal direction of zero azimuth and zero elevation corresponds to (w=0, x=1, y=0, z=0) in quaternions, see clause 7.4.3.1 for the IVAS coordinate system.

The SCENE_ORIENTATION PI data is applied to the audio frame with the same timestamp. The latest received SCENE_ORIENTATION PI data is used until a new SCENE_ORIENTATION PI data is received.

##### A.3.5.6.1.3	Device orientation

The DEVICE_ORIENTATION_COMPENSATED and DEVICE_ORIENTATION_UNCOMPENSATED PI data describes the orientation of the sender (capture) device. I.e., the orientation indicates the orientation deviation from the frontal capture direction, when the device is used for audio capture. The frontal direction of the device point towards the capture frontal direction, i.e. (w=0, x=1, y=0, z=0) in quaternions, see clause 7.4.3.1 for the IVAS coordinate system.

The orientation of a capturing device can be used to stabilize the captured spatial audio content by, e.g., removing undesirable orientation changes. The sender device may be a non-stationary capturing device (e.g., a mobile phone with multiple microphones or a spatial audio capture microphone array). For example, a caller is holding a mobile phone in their hand or wearing a head-mounted display with spatial audio capturing capabilities. Some of the movements of the caller that affect the spatial audio capture can be undesirable (e.g., hand or head movements). The undesirable movements (or orientations) can be compensated in the transmitted spatial audio content via the DEVICE_ORIENTATION_COMPENSATED PI type. When the device orientation is compensated in the transmitted spatial audio content, the received spatial audio can be rendered to the receiver (in their user space) without experiencing the undesirable orientations or movements. In case the spatial audio content is transmitted without orientation compensation, the compensation can be performed by the receiver device. In this case, DEVICE_ORIENTATION_UNCOMPENSATED PI type can be used to transmit the change in the capture device orientation.

DEVICE_ORIENTATION_COMPENSATED PI data indicates that the transmitted orientation is already compensated in the related transmitted audio.

DEVICE_ORIENTATION_UNCOMPENSATED PI data indicates that the transmitted orientation is not compensated in the related transmitted audio.

The device orientation PI data is applied to the audio frame with the same timestamp. The latest received device orientation PI data is used until a new device orientation PI data is received.

##### A.3.5.6.1.4	Disable and enable headtracking

DISABLE_HEADTRACKING PI frame indicates that the headtracking should not be used for the transmitted audio. I.e., the head orientation of the listener should not be applied when rendering the transmitted audio. This PI type shall be used only for diegetic audio streams. In case the media receiver does not support head tracked rendering during the time of session negotiation, the disable or enable head tracking PI types should not be included in the SDP answer from the media receiver (as defined in clause A.4.1). In case the media receiver is using non head tracked rendering, the disable or enable head tracking PI types in the payload should be ignored by the media receiver.

When disabling the headtracking, the head orientation should be reset to a default pose corresponding to (w=1, x=0, y=0, z=0) in quaternions, see clauses A.3.5x2.2.1.3 and 7.4.2.2 (Listener orientation). If some other orientations (e.g., scene orientation or device orientation) are applied at the rendering, those are not affected by the disable/enable headtracking. No extra PI data section is transmitted with DISABLE_HEADTRACKING PI data (i.e., the PI data frame section is empty), the indication for disabling head tracking is recognised from the PI type alone.

DISABLE_HEADTRACKING PI data should disable the headtracking for the current audio frame and for any future audio frames. The headtracking can be re-enabled by sending ENABLE_HEADTRACKING PI frame to the media receiver. These PI data are recommendations from the media sender and the media receiver may choose to override these recommendations.

The media sender can provide indication to the media receiver to disable headtracking if there is a need to ensure that the spatial audio content is consumed without responding to head movements and to lock the head orientation to a default pose. For example, DISABLE_HEADTRACKING PI data created by the sender can be transmitted to the media receiver, if the transmitted spatial audio content already includes orientation changes and no further orientation changes are desired from the headtracking. DISABLE_HEADTRACKING PI data may also be used to lock a source to a certain orientation irrespective to the head orientation (e.g., for an audio notification or alert).

#### A.3.5.6.2	Acoustic environment PI data

Acoustic environment (AE) PI data frames can be used to transmit room acoustic data. The room acoustic data consist of late reverb parameters and optionally early reflections parameters. The detailed description of room acoustics parameters is provided in clause 7.4.8.

The late reverb parameters include:

-	RT60 – indicating the time that it takes for the reflections to reduce 60 dB in energy level, per frequency band,

-	DSR – diffuse to source signal energy ratio, per frequency band,

-	Pre-delay – delay at which the computation of DSR values was performed, which can be also seen as the threshold point between early reflections and late reverberation phase.

Both RT60 and DSR parameters are specified per frequency band. Pre-defined or custom frequency bands can be used. Pre-delay is a scalar.

The simplified early reflections parameters include:

-	3D rectangular virtual room dimensions,

-	Broadband energy absorption coefficient per wall.

To control acoustic environments runtime, acoustic environment PI data frames can be used. An acoustic environment PI data frame can contain:

-	an acoustic environment identifier alone (7 bits),

-	a compact representation of the acoustic environment containing only late reverb parameters (40 bits),

-	a compact representation of the acoustic environment containing late reverb and simplified early reflections coefficients (64 bits).

The content of the PI frame received is determined by its size.

Acoustic environment can be selected by sending an acoustic environment PI frame containing a seven-bit AE identifier, as illustrated in figure A.3.5.6.2-1. Such an acoustic environment should be available upfront. It can be also provided using an AE PI data frame containing a compact acoustic environment representation. It allows for the following graceful degradation mechanism:

-	full AE representation should be used if available, otherwise:

-	compact AE representation should be used if available, otherwise:

-	default AE definition should be used if none of the above are available.

0 1 2 3 4 5 6 7
 +-+-+-+-+-+-+-+-+
 |0|      ID     |
 +-+-+-+-+-+-+-+-+

Figure A.3.5.6.2-1: Acoustic environment PI data frame (ACOUSTIC_ENVIRONMENT) containing an AE identifier.

Acoustic environment data can also get updated real-time. In case of real-time updates, the AE data can be transmitted as compact packets using RTP protocol.

Compact packets contain a low-resolution representation of the acoustic environment. This facilitates avoiding spikes in transmission and is better suited for repeated transmission allowing for instant synthesis of the new acoustic environment, although with initially reduced accuracy. A compact acoustic environment PI can be provided with or without early reflections coefficients. An AE data frame without early reflections is presented in Figure A.3.5.6.2-2.

0                   1                   2                   3
  0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
 |     ID      | RT60 lo |   DSR lo  | RT60 mi |   DSR mi  | RT60
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
  hi |  DSR hi   |
 +-+-+-+-+-+-+-+-+

Figure A.3.5.6.2-2: Acoustic environment PI data frame (ACOUSTIC_ENVIRONMENT) containing a compact AE representation with late reverb parameters.

An AE data frame with early reflections is presented in Figure A.3.5.6.2-3.

0                   1                   2                   3
  0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
 |     ID      | RT60 lo |   DSR lo  | RT60 mi |   DSR mi  | RT60
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
  hi |  DSR hi   | dim x | dim y | dim z |ab0|ab1|ab2|ab3|ab4|ab5|
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.3.5.6.2-3: Acoustic environment PI data frame (ACOUSTIC_ENVIRONMENT) containing a compact AE representation with late reverb and early reflection parameters.

The data frame including early reflections consists of an acoustic environment ID field (7 bits), three RT60 fields (5 bits each), three DSR fields (6 bits each), three room dimension (dim) fields (4 bits each) and six absorption coefficient (ab) fields summing up to 64 bits size. The RT60 and DSR values are provided for three frequency bands of center frequency {25 Hz, 250 Hz, 2.5 kHz}, which are denoted {lo, mi, hi} in figure A.3.5.6.2-2 respectively. The RT60 fields use 5-bit code n representing duration in seconds according to formula RT60n=11002n3 with n ∈ 0..31. The 5-bit codes and related RT60 durations are also shown in table A.3.5.6.2-1.

Table A.3.5.6.2-1 : 5-bit codes and respective RT60 values

| Code | Value | Code | Value | Code | Value | Code | Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00000 | 0.01 | 01000 | 0.0635 | 10000 | 0.4032 | 11000 | 2.56 |
| 00001 | 0.0126 | 01001 | 0.08 | 10001 | 0.5080 | 11001 | 3.2254 |
| 00010 | 0.0159 | 01010 | 0.1008 | 10010 | 0.64 | 11010 | 4.0637 |
| 00011 | 0.02 | 01011 | 0.1270 | 10011 | 0.8063 | 11011 | 5.12 |
| 00100 | 0.0252 | 01100 | 0.16 | 10100 | 1.0159 | 11100 | 6.4508 |
| 00101 | 0.0317 | 01101 | 0.2016 | 10101 | 1.28 | 11101 | 8.1275 |
| 00110 | 0.04 | 01110 | 0.2540 | 10110 | 1.6127 | 11110 | 10.24 |
| 00111 | 0.0504 | 01111 | 0.32 | 10111 | 2.0319 | 11111 | 12.9016 |

The DSR values are computed as DSRn=-20-n[dB] with n ∈ 0..63 resulting in the range between ‑20 and ‑83 dB as shown in Table A.3.5.6.2-2.

Table A.3.5.6.2-2 : 6-bit codes and respective DSR values

| Code | Value | Code | Value | Code | Value | Code | Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 000000 | -20 | 010000 | -36 | 100000 | -52 | 110000 | -68 |
| 000001 | -21 | 010001 | -37 | 100001 | -53 | 110001 | -69 |
| 000010 | -22 | 010010 | -38 | 100010 | -54 | 110010 | -70 |
| 000011 | -23 | 010011 | -39 | 100011 | -55 | 110011 | -71 |
| 000100 | -24 | 010100 | -40 | 100100 | -56 | 110100 | -72 |
| 000101 | -25 | 010101 | -41 | 100101 | -57 | 110101 | -73 |
| 000110 | -26 | 010110 | -42 | 100110 | -58 | 110110 | -74 |
| 000111 | -27 | 010111 | -43 | 100111 | -59 | 110111 | -75 |
| 001000 | -28 | 011000 | -44 | 101000 | -60 | 111000 | -76 |
| 001001 | -29 | 011001 | -45 | 101001 | -61 | 111001 | -77 |
| 001010 | -30 | 011010 | -46 | 101010 | -62 | 111010 | -78 |
| 001011 | -31 | 011011 | -47 | 101011 | -63 | 111011 | -79 |
| 001100 | -32 | 011100 | -48 | 101100 | -64 | 111100 | -80 |
| 001101 | -33 | 011101 | -49 | 101101 | -65 | 111101 | -81 |
| 001110 | -34 | 011110 | -50 | 101110 | -66 | 111110 | -82 |
| 001111 | -35 | 011111 | -51 | 101111 | -67 | 111111 | -83 |

No pre-delay value is transmitted. It gets computed as one tenth of RT60 of 250 Hz band.

The three room dimensions: length (x), width (y), and height (z) in [m], are computed as dimensionn=122n2  with n ∈ 0..15 as described in Table A.3.5.6.2-3.

Table A.3.5.6.2-3 : 4-bit codes and respective room dimension values

| Code | Value | Code | Value |
| --- | --- | --- | --- |
| 0000 | 0.5 | 1000 | 8 |
| 0001 | 0.707 | 1001 | 11.314 |
| 0010 | 1 | 1010 | 16 |
| 0011 | 1.4141 | 1011 | 22.627 |
| 0100 | 2 | 1100 | 32 |
| 0101 | 2.8282 | 1101 | 45.255 |
| 0110 | 4 | 1110 | 64 |
| 0111 | 5.6568 | 1111 | 90.51 |

The six absorption coefficients, corresponding to the six room surfaces (front, back, left, right, ceiling, floor) are computed as absorption n= 0.08*21.05n with n ∈ 0..3 as shown on Table A.3.5.6.2-4.

Table A.3.5.6.2-4: 2-bit codes and respective absorption coefficient values

| Code | Value |
| --- | --- |
| 00 | 0.0800 |
| 01 | 0.1656 |
| 10 | 0.3430 |
| 11 | 0.7101 |

#### A.3.5.6.3	Audio Description PI data (forward direction)

Audio Description (AD) PI data frames can be used to describe the audio content (e.g. speech/music/general audio) transmitted from sender to receiver.

The size of AD PI data varies from 1 to 4 Bytes and depends on the IVAS format as described in Table A.3.5.6.3-1. Wherein for OSBA and OMASA formats the number of discrete coded objects are as per Table 5.8-1 and Table5.9-1 respectively.

Table A.3.5.6.3-1: Audio Description PI data size

| IVAS format | Size (Bytes) |
| --- | --- |
| stereo | 1 |
| SBA | 1 |
| MASA | 1 |
| ISM | Number of Objects |
| MC | 2 (1 for center channel + 1 for all other channels) |
| OMASA | 1 + Number of Discrete coded Objects |
| OSBA | 1 + Number of Discrete coded Objects |

Each Byte in AD PI data payload is an audio identifier (AID) that is defined as follows.

AID: An 8 bit identifier, as described in figure A.3.5.6.3-1, to specify type of audio that is being transmitted. This identifier contains V, M and A field, as defined in Table A.3.5.6.3-2, Table A.3.5.6.3-3 and Table A.3.5.6.3-4 respectively, which specifies whether audio contains speech, music or ambiance or a combination of these audio types. A value of AID equal to 0 corresponds to an unspecified audio type which indicates that an audio description is not available for the related audio frames. The reserved bits in AID shall be set to 0 and be ignored by a receiver.

0 1 2 3 4 5 6 7
 +-+-+-+-+-+-+-+-+
 |V|M|A|  RES    |
 +-+-+-+-+-+-+-+-+

Figure A.3.5.6.3-1: Audio Identifier.

Table A.3.5.6.3-2: V field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | Non-speech |
| 1 | speech |

Table A.3.5.6.3-3: M field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | Non-Music |
| 1 | Music |

Table A.3.5.6.3-4: A field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | Absence of background Ambiance |
| 1 | Presence of background Ambiance |

In an example scenario for OSBA format, if the discretely coded objects contain only clean speech then the corresponding Audio Identifier byte should have V field set to 1 and M and A field set to 0 whereas if the coded SBA component contain only background ambiance then the corresponding Audio Identifier byte should have A field set to 1 and V and M field set to 0.

#### A.3.5.6.4	ISM specific PI data

##### A.3.5.6.4.1	ISM specific PI data types and structure

ISM related PI data types are listed in table A.3.5.6.4.1-1. Figure A.3.5.6.4.1-1 presents a general PI data structure for ISM related PI types. The data structure begins with an ISM specific PI header (see clause A.3.5.6.4.2) followed by the PI data frames for each ISM (the number of data frames depends on the number of ISMs).

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|  ISM PI header   |     ISM1 PI data       |     ISM2 PI data  |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|       ISM3 PI data      |      ISM4 PI data   |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.1-1: ISM PI data structure with 4 ISM objects.

Table A.3.5.6.4.1-1: ISM related PI data types.

ISM related PI type

ISM_TYPE

ISM_DISTANCE_ATTENUATION

ISM_ID

ISM_GAIN

ISM_ORIENTATION

ISM_POSITION

ISM_DIRECTIVITY

##### A.3.5.6.4.2	ISM specific PI header

Each ISM specific PI data frame is preceded by a header indicating the number of ISMs in the associated IVAS frame as presented in Figure A.3.5.6.4.2-1. The first two bits indicate the number of ISMs (N) according to table A.3.5.6.4.2-1. The last six bits are used for zero padding to force byte alignment.

0 1 2 3 4 5 6 7
+-+-+-+-+-+-+-+-+
| N |0 0 0 0 0 0|
+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.2-1: ISM header indicating the number of ISMs and flags to indicate non-diegetic ISMs.

Table A.3.5.6.4.2-1: N bit values in the ISM header and the indicated number of ISMs.

| N bits | Indicated number of ISMs |
| --- | --- |
| 00 | 1 |
| 01 | 2 |
| 10 | 3 |
| 11 | 4 |

##### A.3.5.6.4.3	ISM type

The ISM_TYPE PI type can be used to indicate if the transported ISMs are diegetic (0) or non-diegetic (1). The PI data structure is presented in figure A.3.5.6.4.3-1. The four bits act as individual flags for each ISM starting from the first ISM. Each flag (X) indicates if the audio object is non-diegetic (1) or not (0). The last four bits are used for zero padding to force byte alignment.

0 1 2 3 4 5 6 7
+-+-+-+-+-+-+-+-+
|X|X|X|X|0 0 0 0|
+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.3-1: ISM_TYPE PI data frame with flags to indicate non-diegetic ISMs.

##### A.3.5.6.4.4	ISM distance attenuation

The ISM_DISTANCE_ATTENUATION PI data frame is presented in figure A.3.5.6.4.4-1. The data frame includes a reference distance (6 bits), maximum distance (6 bits) and a roll-off factor (6 bits), totalling in 3 bytes of data (including zero padding for byte alignment) in accordance with table A.3.5.6.4.4-1 - A.3.5.6.4.4-2. ISM distance attenuation is a rendering configuration applicable for all rendered ISMs. For more information about ISM distance attenuation, see clause 7.2.2.2.7.

0                   1                   2    _
 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
 | Ref dist. | Max dist. |  Roll-off |0 0 0 0 0 0|
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.4-1: ISM_DISTANCE_ATTENUATION PI data frame.

Table A.3.5.6.4.4-1: 6-bit codes and respective Reference distance values (m)

| Code | Value | Code | Value | Code | Value | Code | Value |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 000000 | 0.1 | 010000 | 1.7 | 100000 | 3.3 | 110000 | 4.9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 000001 | 0.2 | 010001 | 1.8 | 100001 | 3.4 | 110001 | 5.0 |
| 000010 | 0.3 | 010010 | 1.9 | 100010 | 3.5 | 110010 | 5.1 |
| 000011 | 0.4 | 010011 | 2.0 | 100011 | 3.6 | 110011 | 5.2 |
| 000100 | 0.5 | 010100 | 2.1 | 100100 | 3.7 | 110100 | 5.3 |
| 000101 | 0.6 | 010101 | 2.2 | 100101 | 3.8 | 110101 | 5.4 |
| 000110 | 0.7 | 010110 | 2.3 | 100110 | 3.9 | 110110 | 5.5 |
| 000111 | 0.8 | 010111 | 2.4 | 100111 | 4.0 | 110111 | 5.6 |
| 001000 | 0.9 | 011000 | 2.5 | 101000 | 4.1 | 111000 | 5.7 |
| 001001 | 1.0 | 011001 | 2.6 | 101001 | 4.2 | 111001 | 5.8 |
| 001010 | 1.1 | 011010 | 2.7 | 101010 | 4.3 | 111010 | 5.9 |
| 001011 | 1.2 | 011011 | 2.8 | 101011 | 4.4 | 111011 | 6.0 |
| 001100 | 1.3 | 011100 | 2.9 | 101100 | 4.5 | 111100 | 6.1 |
| 001101 | 1.4 | 011101 | 3.0 | 101101 | 4.6 | 111101 | 6.2 |
| 001110 | 1.5 | 011110 | 3.1 | 101110 | 4.7 | 111110 | 6.3 |
| 001111 | 1.6 | 011111 | 3.2 | 101111 | 4.8 | 111111 | 6.4 |

Table A.3.5.6.4.4-2: 6-bit codes and respective Maximum distance values (m)

| Code | Value | Code | Value | Code | Value | Code | Value |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 000000 | 1 | 010000 | 17 | 100000 | 33 | 110000 | 49 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 000001 | 2 | 010001 | 18 | 100001 | 34 | 110001 | 50 |
| 000010 | 3 | 010010 | 19 | 100010 | 35 | 110010 | 51 |
| 000011 | 4 | 010011 | 20 | 100011 | 36 | 110011 | 52 |
| 000100 | 5 | 010100 | 21 | 100100 | 37 | 110100 | 53 |
| 000101 | 6 | 010101 | 22 | 100101 | 38 | 110101 | 54 |
| 000110 | 7 | 010110 | 23 | 100110 | 39 | 110110 | 55 |
| 000111 | 8 | 010111 | 24 | 100111 | 40 | 110111 | 56 |
| 001000 | 9 | 011000 | 25 | 101000 | 41 | 111000 | 57 |
| 001001 | 10 | 011001 | 26 | 101001 | 42 | 111001 | 58 |
| 001010 | 11 | 011010 | 27 | 101010 | 43 | 111010 | 59 |
| 001011 | 12 | 011011 | 28 | 101011 | 44 | 111011 | 60 |
| 001100 | 13 | 011100 | 29 | 101100 | 45 | 111100 | 61 |
| 001101 | 14 | 011101 | 30 | 101101 | 46 | 111101 | 62 |
| 001110 | 15 | 011110 | 31 | 101110 | 47 | 111110 | 63 |
| 001111 | 16 | 011111 | 32 | 101111 | 48 | 111111 | 64 |

Table A.3.5.6.4.4-3: 6-bit codes and respective Roll-off factor values

| Code | Value | Code | Value | Code | Value |
| --- | --- | --- | --- | --- | --- |

| 000000 | 0 | 010000 | 1.6 | 100000 | 3.2 |
| --- | --- | --- | --- | --- | --- |
| 000001 | 0.1 | 010001 | 1.7 | 100001 | 3.3 |
| 000010 | 0.2 | 010010 | 1.8 | 100010 | 3.4 |
| 000011 | 0.3 | 010011 | 1.9 | 100011 | 3.5 |
| 000100 | 0.4 | 010100 | 2.0 | 100100 | 3.6 |
| 000101 | 0.5 | 010101 | 2.1 | 100101 | 3.7 |
| 000110 | 0.6 | 010110 | 2.2 | 100110 | 3.8 |
| 000111 | 0.7 | 010111 | 2.3 | 100111 | 3.9 |
| 001000 | 0.8 | 011000 | 2.4 | 101000 | 4.0 |
| 001001 | 0.9 | 011001 | 2.5 | 101001-111111 | reserved |
| 001010 | 1.0 | 011010 | 2.6 |  |  |
| 001011 | 1.1 | 011011 | 2.7 |  |  |
| 001100 | 1.2 | 011100 | 2.8 |  |  |
| 001101 | 1.3 | 011101 | 2.9 |  |  |
| 001110 | 1.4 | 011110 | 3.0 |  |  |
| 001111 | 1.5 | 011111 | 3.1 |  |  |

##### A.3.5.6.4.5	ISM ID

ISM_ID PI data specifies an identity (ID) for the ISMs transported in the associated IVAS frame, as presented in figure A.3.5.6.4.5-1. The PI data includes an identity field (one byte) for each transported ISM, positioned after one another. For example, the ISM ID for the first object is followed by the ISM ID for the second object when the number of ISMs N>1.

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|     ISM ID    |
+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.5-1: ISM_ID PI data frame.

##### A.3.5.6.4.6	ISM gain

ISM_GAIN PI data specifies a gain factor for ISMs transported in the associated IVAS frame, as presented in figure A.3.5.6.4.6-1. The PI data includes ISM gain field (one byte) for each transported ISM, positioned after one another. For example, the ISM gain for the first object is followed by the ISM gain for the second object when the number of ISMs N>1. The 8-bit ISM gains represent a uniform range (in dB) ranging from -96 dB to +3 dB with additional code point for muting (-Inf dB), according to table A.3.5.6.4.6-1.

0 1 2 3 4 5 6 7 
+-+-+-+-+-+-+-+-+
|   ISM gain  |0|
+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.6-1: ISM_GAIN PI data frame.

Table A.3.5.6.4.6-1: 7-bit codes and respective gain values (dB)

| Code | Value | Code | Value |
| --- | --- | --- | --- |
| 0000000 | 0 | ⋮ | ⋮ |
| 0000001 | -1 | 1110001 | -87 |
| 0000010 | -2 | 1110010 | -88 |
| 0000011 | -3 | 1110011 | -89 |
| 0000100 | -4 | 1110100 | -90 |
| 0000101 | -5 | 1110101 | -91 |
| 0000110 | -6 | 1110110 | -92 |
| 0000111 | -7 | 1110111 | -93 |
| 0001000 | -8 | 1111000 | -94 |
| 0001001 | -9 | 1111001 | -95 |
| 0001010 | -10 | 1111010 | -96 |
| 0001011 | -11 | 1111011 | -Inf |
| 0001100 | -12 | 1111100 | +1 |
| 0001101 | -13 | 1111101 | +2 |
| 0001110 | -14 | 1100100 | +3 |
| ⋮ | ⋮ | 1100100-1111111 | reserved |

##### A.3.5.6.4.7	ISM orientation

ISM_ORIENTATION PI data describes the orientation of audio object(s) (ISMs) by orientation data structures in accordance with clause A.3.5.6.1.1, with respect to the scene orientation. The ISM specific header indicates the number of orientations (i.e., the number of objects) in the PI orientation data section. In the orientation data section, the full orientation representation for each audio object is positioned after one another. For example, the orientation data section begins with the quaternion components (w,x,y,z) for the first object, followed by the components for the second object and so on.

##### A.3.5.6.4.8	ISM position

ISM_POSITION PI frame indicates the position(s) of audio object(s) (ISMs) in 3D space. The number of ISM position values (i.e., the number of objects) is indicated in the ISM PI header, see clause A.3.5.6.4.2. Figure A.3.5.7.3-1 shows a general position PI data structure as cartesian coordinates (X, Y, Z) that can be used for ISM_POSITION PI data frames. The position PI data structures for the transported ISMs are positioned after one another. For example, the ISM position for the first object is followed by the ISM position for the second object when the number of ISMs N>1. See clause A.3.5.7.3 for more information about position PI data.

##### A.3.5.6.4.9	ISM directivity

ISM_DIRECTIVITY PI data frame structure is presented in figure A.3.5.6.4.9-1. The ISM orientation indicate the direction (of the inner cone) for an ISM object. The inner cone angle, “Inner ang”, (5 bits) determines the size of the main cone directed to the front direction of the object, see table A.3.5.6.4.9-1. The outer cone angle, “Outer ang”, (5 bits) determines the size of the outer (back) cone, see table A.3.5.6.4.9-1. The gain for the inner cone is determined by the ISM gain, and the outer attenuation gain, “Outer att”, (5 bits) determines the attenuation outside the outer cone, see table A.3.5.6.4.9-2. The total size of an ISM_DIRECTIVITY PI data frame for a single object is 2 bytes, including zero-padding for byte alignment. The ISM directivity PI data structures for the transported ISMs are positioned after one another. For example, the ISM directivity for the first object is followed by the ISM directivity for the second object when the number of ISMs N>1. For more information about ISM directivity, see clause 7.2.2.2.7.

0                   1       _   
  0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
 |Inner ang|Outer ang|Outer att|0|
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.9-1: ISM_DIRECTIVITY PI data frame.

Table A.3.5.6.4.9-1: 5-bit codes and respective Inner or Outer cone angle values (deg)

| Code | Value | Code | Value |
| --- | --- | --- | --- |

| 00000 | 0 | 10000 | 240 |
| --- | --- | --- | --- |
| 00001 | 15 | 10001 | 255 |
| 00010 | 30 | 10010 | 270 |
| 00011 | 45 | 10011 | 285 |
| 00100 | 60 | 10100 | 300 |
| 00101 | 75 | 10101 | 315 |
| 00110 | 90 | 10110 | 330 |
| 00111 | 105 | 10111 | 345 |
| 01000 | 120 | 11000 | 360 |
| 01001 | 135 | 11001-11111 | reserved |
| 01010 | 150 |  |  |
| 01011 | 165 |  |  |
| 01100 | 180 |  |  |
| 01101 | 195 |  |  |
| 01110 | 210 |  |  |
| 01111 | 225 |  |  |

Table A.3.5.6.4.9-2: 5-bit codes and respective Outer attenuation gain values (dB)

| Code | Value | Code | Value |
| --- | --- | --- | --- |

| 00000 | -90 | 010000 | -42 |
| --- | --- | --- | --- |
| 00001 | -87 | 010001 | -39 |
| 00010 | -84 | 010010 | -36 |
| 00011 | -81 | 010011 | -33 |
| 00100 | -78 | 010100 | -30 |
| 00101 | -75 | 010101 | -27 |
| 00110 | -72 | 010110 | -24 |
| 00111 | -69 | 010111 | -21 |
| 01000 | -66 | 011000 | -18 |
| 01001 | -63 | 011001 | -15 |
| 01010 | -60 | 011010 | -12 |
| 01011 | -57 | 011011 | -9 |
| 01100 | -54 | 011100 | -6 |
| 01101 | -51 | 011101 | -3 |
| 01110 | -48 | 011110 | 0 |
| 01111 | -45 | 011111 | reserved |

### A.3.5.7	Reverse direction PI data types

#### A.3.5.7.1	Playback device orientation

PLAYBACK_DEVICE_ORIENTATION PI data describes the orientation of the playback device with respect to the frontal direction of the device. The frontal direction refers to the playback device orientation at the beginning of playback, i.e. (w=0, x=1, y=0, z=0) in quaternions. Playback device orientation describes the deviation from the frontal direction. PLAYBACK_DEVICE_ORIENTATION PI type is used for feedback signalling.

The latest received PLAYBACK_DEVICE_ORIENTATION PI data is used until a new PLAYBACK_DEVICE_ORIENTATION PI data is received.

#### A.3.5.7.2	Head orientation

The HEAD_ORIENTATION PI data describes the listener orientation and follows the description in clause 7.4.2.2 (Listener orientation), i.e., the head frontal direction is (w=0, x=1, y=0, z=0) in quaternions. HEAD_ORIENTATION PI type is used for feedback signalling.

The latest received HEAD_ORIENTATION PI data is used until a new HEAD_ORIENTATION PI data is received.

#### A.3.5.7.3	Listener position

LISTENER_POSITION PI frame indicates the listener position in 3D space in cartesian coordinates as in figure A.3.5.7.3-1. Figure A.3.5.7.3-1 shows a general position PI data structure as cartesian coordinates (X, Y, Z). Each component is a 16 bits signed integer in units of 0.01 metres, which gives a maximum distance of . This gives a range of approximately [-327.68, 327.68] meters. The cartesian position coordinates follow the representation presented in clause 7.4 (Rendering control), where the x-axis points towards front, the y-axis points towards left and the z-axis points towards up.

0                   1                   2                   3
   0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
  +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
  |               X               |               Y               |
  +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
  |               Z               |
  +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.3.5.7.3-1: General position PI data frame with cartesian coordinates.

#### A.3.5.7.4	Dynamic Audio Suppression

The Dynamic Audio Supression (DAS) PI data describes receiver’s preference with respect to the type of audio content (for e.g., Speech only) that should be enhanced and the amount of suppression to be applied to the background noise, where the background noise is defined as the type of audio content that should be suppressed according to the receiver preference.

The size of DAS PI data is 2 bytes and is described in figure A.3.5.7.4-1. The DAS PI data payload contains an Audio Identifier (AID) byte and a Suppression Level Indicator (SLI) byte, as illustrated in Figure A.3.5.7.4-1. The value of AID field shall be non-zero for the audio identifier bits (V, M, A) and the reserved bits in AID shall be set to 0, unless defined. Likewise, there are 4 useable bits of the SLI while the 4 other (reserved) bits shall be set to 0. The AID byte contains V, M and A field, as defined in Table A.3.5.6.3-2, Table A.3.5.6.3-3 and Table A.3.5.6.3-4 respectively, which specifies whether receiver’s preference is speech, music or ambiance or a combination of these audio types. The Suppression Level Indicator, as defined in table A.3.5.7.4-1, allows specifying a desired degree of suppression where audio signal components other than what is specified by the AID field are considered as undesired audio component. The SLI takes values from 0 to 15 wherein the expected amount of audio suppression is proportional (in approximate logarithmic domain) to the indicator value with 0 indicating no audio suppression and 15 indicating maximum audio suppression as offered by the sender side.

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
 |V|M|A|  RES    |  SLI  |  RES  |
 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

+      AID      +      SLI      +

Figure A.3.5.7.4-1: DAS PI data byte

Table A.3.5.7.4-1: Suppression Level Indicator

| SLI Size bits | Value | Description |
| --- | --- | --- |
| 0000 | 0 | No suppression |
| 0001 | 1 | … |
| … | … | … |
| 1110 | 14 | … |
| 1111 | 15 | Maximum suppression offered by the sender side |

#### A.3.5.7.5	Audio focus direction

AUDIO_FOCUS_DIRECTION PI frame indicates a direction of interest from the receiver (i.e., the listener) to the sender side. The focus direction is transmitted similarly as the orientation PI data, see clause A.3.5.6.1.1. The focus direction is relative to the frontal direction (w=0, x=1, y=0, z=0).

### A.3.5.8	PI data flow

Figure A.3.5.8-1 presents an example RTP session data flow between two UEs. UE1 receives audio data and encodes it into an IVAS bitstream which is included in the IVAS RTP payload S12. Forward direction PI data, e.g., scene or device orientations are included in the payload S12 from UE1 to UE2 in the packetization phase. At the receiver (UE2), the received payload S12 is depacketized to obtain the audio bitstream and PI data. Both are used in the decoding and rendering process. Additionally, head and/or external orientations may be fed and used in the decoding and rendering process.

UE2 receives audio data and encodes it into an IVAS bitstream which is included in the IVAS RTP payload S21. Reverse PI data (e.g., the head orientation or the listener or a suppression request) can be included in the payload S21 in the packetization phase. At UE1, the received payload S21 is depacketized to obtain the audio bitstream and the (reverse) PI data. The audio bitstream is decoded and renderer with the IVAS decoder/renderer. The reverse PI data (e.g., the head orientation or the listener in UE2 or a suppression request) is fed to the encoding process.

Figure A.3.5.8-1: PI data flow between two UEs.

CHANGE 2

# A.4	Payload Format Parameters

## A.4.1	IVAS Media Type Registration

The media type for the IVAS codec is to be allocated from the standards tree. This clause defines parameters of the IVAS payload format. This media type registration covers real-time transfer via RTP and non-real-time transfers via stored files. All media type parameters defined in this document shall be supported.

Media type name: audio

Media subtype name: IVAS

Required parameters: none

Optional parameters:

The parameters defined below apply to RTP transfer only:

ptime:	see [32].

maxptime:	see [32].

dtx/dtx-recv: as defined in Annex A of [3].

max-red:	see [36].

channels:	The number of audio channels shall not be present.

NOTE:	The use of the channels parameter as defined in [35] does not permit signaling all IVAS Immersive mode coded formats; formats need to be derived from the cf/cf-send/cf-recv parameters.

ivas-mode-switch:	This parameter defines the mode at the start or update of the session for the send and the receive directions. Permissible values are 0 and 1. If ivas-mode-switch is 0 or not present, IVAS Immersive mode is used. If ivas-mode-switch is 1, depending on the setting of evs-mode-switch, EVS Primary or AMR-WB IO mode is used The mode initially used in the session may later be modified.

cmr:	As defined in Annex A of [3] for the EVS Primary and AMRWB-IO modes. For IVAS Immersive modes the bit rate, bandwidth and format requests are disabled when cmr is -1. The bitrate, bandwidth and format requests are enabled when cmr is 0 or the cmr parameter is not present. When cmr is 1 the bit rate requests using the initial E byte shall be present in every packet (but may be NO_REQ); format and bandwidth requests for IVAS Immersive modes are optional when cmr is 1.

The following parameters are applicable only to IVAS Immersive operation:

NOTE:	IVAS computational complexity and memory demands of depend on the setting of the following parameters for source codec bit rate, audio bandwidth, and coded format; in addition, factors beyond the signaling, such as complexity of a specific implementation and the (rendered) output format may be significant.

ibr:	Specifies the range of source codec bitrate for IVAS Immersive mode in the session, in kilobits per second, for the direction specified by the session directionality attribute or the suffix. The br parameter can either have: a single bitrate (ibr1); or a hyphen-separated pair of two bitrates (ibr1-ibr2). If a single value is included, this bitrate, ibr1, is used. If a hyphen-separated pair of two bitrates is included, ibr1 and ibr2 are used as the minimum bitrate and the maximum bitrate respectively. ibr1 shall be smaller than ibr2. ibr1 and ibr2 have a value from the set in Table 4.2-2 of the present document. If none of these parameters is present, all bitrates consistent with the IVAS codec capabilities are allowed in the session.

ibr-send/ibr-recv: ibr parameter in send or receive direction.

ibw:	Specifies the audio bandwidth for IVAS Immersive modes to be used in the session, for the direction specified by the session directionality attribute or the suffix. ibw has a value from the set: wb, swb, fb, wb-swb, and wb-fb. wb, swb, and fb represent wideband, super-wideband, and fullband respectively, and wb-swb, and wb-fb represent all bandwidths from wideband to super-wideband, and fullband respectively. If none of these parameters is present, all bandwidths consistent with the negotiated bitrate(s) are allowed in the session.

ibw-send/ibw-recv: ibw parameter in send or receive direction.

cf: 	Specifies the IVAS Immersive mode coded-format (cf) transmitted in the IVAS Immersive mode frames in the session. IVAS coded format corresponds to the format represented in the IVAS Immersive mode coded frames, which is generally the input format to the encoder. The cf parameter is a list of supported comma-separated IVAS Immersive mode coded formats in the order of preference, using the identifiers from Table A.4.1-1 of the present document (column "Identifier"). Selection of the format is application-specific and out of scope of this document. EVS frames in the session are in mono format; switching to mono shall be possible.
For SR format, the following applies: While the formats offered by the offererer may be a list containing SR and other formats, the answer shall either exclusively contain SR or a set of the other offered formats excluding SR. A combination of SR with other formats is not permissible.

Table A.4.1-1: IVAS coded-format

| Identifier | Full Name | Clause |
| --- | --- | --- |
| Stereo | Stereo Operation | 4.2.3 |
| SBA | Scene-based Audio (SBA, Ambisonics) Operation | 4.2.4 |
| MASA | Metadata-assisted Spatial Audio (MASA) Operation | 4.2.5 |
| ISM | Objects (Independent Streams with Metadata, ISM) Operation | 4.2.6 |
| MC | Multi-Channel (MC) Operation | 4.2.7 |
| OMASA | Combined Objects and MASA (OMASA) Operation | 4.2.9 |
| OSBA | Combined Objects and SBA (OSBA) Operation | 4.2.8 |
| SR | Split rendering Operation | 7.6 |

Mono is not listed as an IVAS Immersive mode coded-format as EVS is always supported and shall be used for mono.

NOTE:	IVAS payloads are self-contained for all IVAS coded formats except SR and mono, i.e., they require no additional signaling for decoding than the payload size.

cf-send/cf-recv: cf parameter in send or receive direction

pi-types:       Specifies the supported PI data types for the session. The pi-types parameter is a list of supported comma-separated PI data types using the SDP indications listed in tables A.3.5.5-1, A3.5.5-1.1 and A.3.5.5-2. If the pi-types parameter is not present, PI data is not enabled for the session.

pi-types-send/pi-types-recv:		pi-types parameter in send or receive direction.

pi-br:	 		 Specifies the maximum peak bitrate for the PI data section (excluding the E-bytes for indication) for each packet in the session in kilobits per second. Bitrate calculation for PI data shall take the packet interval, i.e. value of ptime into account. The parameter indicates the maximum bitrate for the PI data. If pi-br parameter is not present, a default value of 0 shall be used.

pi-br-send/pi-br-recv:		pi-br parameter in send or receive direction.

sr-dof:       	Specifies the number of degrees of freedom supported in the head-tracked split rendering session. Permissive values are -1, 0, 1, 2, 3. A value of -1 means that respective stream will be a non-diegetic stream in which the pre-renderer does not expect head-tracker data/does not take such data into account during pre-rendering. A value in the range of 0 - 3 means that the pre-renderer expects head-tracker data, conveyed by PI frames or some other mechanism. A value of D > 0 means that metadata is generated and transmitted in the SR bitstream allowing the post-renderer to make pose corrections of the binaural audio in D degrees of freedom. D=1 means support of pose corrections around a single axis only, D=2 means support of pose corrections around a two axes , D=3 means support of pose corrections around all 3 axes. If the first value is greater than -1, it may be followed separated by a comma by a second value respective a second non-diegetic stream. The only permissible value for that stream is -1.    
If sr-dof is not present in a negotiated SR session, it defaults to 3. A positive value of D may additionally be appended with the optional suffix *, indicating high-efficiency rather than high-quality split renderer metadata calculation to be used in the session.

sr-tc:	Specifies the codec format for the binaural transport channels. This parameter must be present in a negotiated split rendering session. Permissible values are ‘LCLD’ and ‘LC3plus’ followed by a an optional value specifying the maximum allowed split rendering bitrate, in kilobits per second, for the SR stream(s) to be used in the session. If the bit rate value is left empty, the maximum of 512 is assumed. Following, the bitrate value, a frame length indicator, in ms, may be provided. If no value is specified, the frame length defaults to 20 ms. The frame length indicator may be 5, 10 or 20; it has effect only for non-diegetic sterams or streams without pose correction metadata (D=-1 or D=0 in sr-dof parameter). Table A.3.3.3.2-2 specifies the bitrates that can be specified as maximum bitrates. When LC3plus is used in a session with split rendering, two further values, ‘fdi’ and ‘bwr’ shall additionally be supplied. ‘fdi’ is specified in [Ref TS 103 634]; it shall be set to "1" or "2", depending on the split rendering configuration. ‘bwr’ is specified in [Ref TS 103 634]; it shall be set to "fb". 
All value fields shall be separated by commas even if left empty.

The following parameters are applicable only to EVS Primary and AMR-WB IO modes:

evs-mode-switch:	as defined in Annex A of [3]. If ivas-mode-switch is 0 or not present, evs-mode-switch should not be present and shall be ignored.

hf-only:	as specified in Annex A of [3] except that the default and only allowed value of hf-only shall be 1 in this payload format. As the only allowed value for this parameter is 1 it is not required to include this parameter.

NOTE:	There is no compact format support in this payload format, contrary to the EVS payload format in Annex A of [3] that enables the compact format by default.

ch-send: 	Shall not be present. The EVS modes in this payload format shall be mono-only

ch-recv: 	Shall not be present. The EVS modes in this payload format shall be mono-only.

The following parameters are applicable only to EVS Primary modes:

br:	as defined in Annex A of [3]

br-send:	as defined in Annex A of [3]

br-recv:	as defined in Annex A of [3]

bw:	as defined in Annex A of [3]

NOTE:	Narrow-band is not supported for IVAS operation

bw-send:	as defined in Annex A of [3]

bw-recv:	as defined in Annex A of [3]

ch-aw-recv:	as defined in Annex A of [3]

The following parameters are applicable only to EVS AMR-WB IO modes:

mode-set:	as defined in Annex A of [3]

mode-change-period:		see [36].

mode-change-capability:	as defined in Annex A of [3]

mode-change-neighbor: 	see [36]

## A.4.2	Mapping media type parameters into SDP

The information carried in the media type specification has a specific mapping to fields in the Session Description Protocol (SDP) [32], which is commonly used to describe RTP sessions. When SDP is used to specify sessions employing the IVAS codec, the mapping is as follows:

-	The media type ("audio") goes in SDP "m=" as the media name.

-	The media subtype (payload format name) goes in SDP "a=rtpmap" as the encoding name.  The RTP clock rate in "a=rtpmap" shall be 16000, and the encoding parameters (number of channels) shall be omitted.

-	The parameters "ptime" and "maxptime" go in the SDP "a=ptime" and "a=maxptime" attributes, respectively.

-	Any remaining parameters go in the SDP "a=fmtp" attribute by copying them directly from the media type parameter string as a semicolon-separated list of parameter=value pairs.

Mapping to fields in SDP is specified in clause 6 of [33].

## A.4.3	Detailed Description of Usage of SDP Parameters

### A.4.3.1	Offer-Answer Model Considerations

The following considerations apply when using SDP Offer-Answer procedures to negotiate the use of IVAS payload in RTP:

hf-only:	Shall not be included in the SDP offer. The answerer shall include this parameter only if it is set to 1 in the SDP offer. If the value in the SDP offer is not equal to 1, the payload type shall be rejected.

ivas-mode-switch:	When ivas-mode-switch is not offered for a payload type, the answerer may include ivas-mode-switch for the payload type in the SDP answer. When ivas-mode-switch is offered for a payload type and the payload type is accepted, the answerer shall not modify or remove ivas-mode-switch for the payload type in the SDP answer.

cmr:	When cmr is not offered for a payload type, the answerer may include cmr for the payload type in the SDP answer. When cmr is offered for a payload type and the payload type is accepted, the answerer shall not modify or remove cmr for the payload type in the SDP answer.

ibr:	When the same bitrate or bitrate range is defined for the send and the receive directions, ibr should be used but ibr-send and ibr-recv may also be used. ibr can be used even if the session is negotiated to be sendonly, recvonly, or inactive. For sendonly session, ibr and ibr-send can be interchangeably used. For recvonly session, ibr and ibr-recv can be interchangeably used. When ibr is not offered for a payload type, the answerer may include ibr for the payload type in the SDP answer. When ibr is offered for a payload type and the payload type is accepted, the answerer shall include ibr in the SDP answer which shall be identical to or a subset of ibr for the payload type in the SDP offer.

ibr-send:	When ibr-send is not offered for a payload type, the answerer may include ibr-recv for the payload type in the SDP answer. When ibr-send is offered for a payload type and the payload type is accepted, the answerer shall include ibr-recv in the SDP answer, and the ibr-recv shall be identical to or a subset of ibr-send for the payload type in the SDP offer.

ibr-recv:	When ibr-recv is not offered for a payload type, the answerer may include ibr-send for the payload type in the SDP answer. When ibr-recv is offered for a payload type and the payload type is accepted, the answerer shall include ibr-send in the SDP answer, and the ibr-send shall be identical to or a subset of ibr-recv for the payload type in the SDP offer.

ibw:	When the same bandwidth or bandwidth range is defined for the send and the receive directions, ibw should be used but ibw-send and ibw-recv may also be used. ibw can be used even if the session is negotiated to be sendonly, recvonly, or inactive. For sendonly session, ibw and ibw-send can be interchangeably used. For recvonly session, ibw and ibw-recv can be interchangeably used. When ibw is not offered for a payload type, the answerer may include ibw for the payload type in the SDP answer. When ibw is offered for a payload type and the payload type is accepted, the answerer shall include ibw in the SDP answer, which shall be identical to or a subset of ibw for the payload type in the SDP offer.

ibw-send:	When ibw-send is not offered for a payload type, the answerer may include ibw-recv for the payload type in the SDP answer. When ibw-send is offered for a payload type and the payload is accepted, the answerer shall include ibw-recv in the SDP answer, and the ibw-recv shall be identical to or a subset of ibw-send for the payload type in the SDP offer.

ibw-recv	When ibw-recv is not offered for a payload type, the answerer may include ibw-send for the payload type in the SDP answer. When ibw-recv is offered for a payload type and the payload is accepted, the answerer shall include ibw-send in the SDP answer, and the ibw-send shall be identical to or a subset of ibw-recv for the payload type in the SDP offer.

cf:	The SDP offer [shall] list at least one but may list several IVAS Immersive mode coded formats. The SDP answer shall include at least one IVAS Immersive mode coded format and should respond with the one most preferred coded format from the list in the SDP offer. If more than one format is present in the SDP answer, the first format shall be used at the start of a session and may only be modified by the adaptation mechanisms present in this specification. When the same IVAS Immersive mode coded formats are defined for the send and the receive directions, cf should be used but cf-send and cf-recv may also be used. For sendonly session, cf and cf-send can be interchangeably used. For recvonly session, cf and cf-recv can be interchangeably used.

cf-send:	When cf-send is offered for a payload type and the payload type is accepted, the answerer shall include cf-recv in the SDP answer, and the cf-recv shall be identical to or a subset of the cf-send parameter for the payload type in the SDP offer.

cf-recv	When cf-recv is offered for a payload type and the payload type is accepted, the answerer shall include cf-send in the SDP answer, and the cf-send shall be identical to or a subset of the cf-recv parameter for the payload type in the SDP offer.

pi-types:       	The SDP offer shall list at least one but may list several supported pi types when pi data is enabled in the offer. When one or more of the offered pi types are supported, the SDP answer shall be identical to or a subset of the pi types listed in the SDP offer. When the same pi types are defined for the send and the receive directions, pi-types should be used but pi-types-send and pi-types-recv may also be used. For sendonly session, pi-types and pi-types-send can be interchangeably used. For recvonly session, pi-types and pi-types-recv can be interchangeably used. The pi types listed in the SDP answer should be supported and applicable for the session. When none of the offered pi-types is supported, the answerer shall not include pi-types in the SDP answer.

pi-types-send:	When pi-types-send is offered in the SDP offer and it is accepted, the answerer shall include pi-types-recv in the SDP answer, and the pi-types-recv shall be identical to or a subset of the pi-types-send parameter in the SDP offer.

pi-types-recv:	When pi-types-recv is offered in the SDP offer and it is accepted, the answerer shall include pi-types-send in the SDP answer, and the pi-types-send shall be identical to or a subset of the pi-types-recv parameter in the SDP offer.

pi-br:	When the same bitrate is defined for the send and the receive directions, pi-br should be used but pi-br-send and pi-br-recv may also be used. pi-br can be used even if the session is negotiated to be sendonly, recvonly, or inactive. For sendonly session, pi-br and pi-br-send can be interchangeably used. For recvonly session, pi-br and pi-br-recv can be interchangeably used. When pi-br is not offered in the SDP offer, the answerer shall not include pi-br in the SDP answer. When pi-br is offered in the SDP offer and it is accepted, the answerer shall include pi-br in the SDP answer which shall be identical or lower than pi-br in the SDP offer.

pi-br-send:	When pi-br-send is offered in the SDP offer and it is accepted, the answerer shall include pi-br-recv in the SDP answer, and the pi-br-recv shall be identical or lower than pi-br-send in the SDP offer.

pi-br-recv:	When pi-br-recv is offered in the SDP offer and it is accepted, the answerer shall include pi-br-send in the SDP answer, and the pi-br-send shall be identical or lower than pi-br-recv in the SDP offer.

Split rendering related offer-answer considerations

cf-recv:	When cf-recv is offered for a payload type (typically by lightweight end device), it must list at least SR as one IVAS Immersive mode coded formats. To accept the offer with split rendering used in the subsequent session, the answer shall contain cf_send with SR as the only IVAS Immersive mode coded format.

cf-send:	When cf-send is offered for a payload type (typically by a pre-rendering node/device other than lightweight end device), it must list at least SR as one IVAS Immersive mode coded formats. To accept the offer with split rendering used in the subsequent session, the answer shall contain cf_recv with SR as the only IVAS Immersive mode coded format.

sr-dof:	When cf-recv or cf-send is offered for a payload type with SR listed, the offer may additionally contain the parameter sr-dof. In that case and if the SR session is accepted, the answerer shall include sr-dof in the SDP answer, and the sr-dof shall be identical or lower than sr-dof in the SDP offer. If the first value in the answer is reduced to -1, the second value shall not be present. The answerer may add but shall not remove a * suffix unless the value is smaller than 1.

sr-tc:	When cf-recv or cf-send is offered for a payload type with SR listed, the offer shall additionally contain the parameter sr-tc. In that case and if the SR session is accepted, the answerer shall include sr-tc in the SDP answer, and the sr-tc parameter shall be identical except for the field specifying the bitrate value. If a bitrate value was specified in the offer, the same or a lower bitrate value out of the set of available bitrates may be used in the answer. If not present, the answer may leave that field open or specify a bitrate.

NOTE: 	As split rendering sessions are typically limited to one direction between two directly connected nodes/end points of an IVAS codec session, only directional parameters shall be used to negotiate an IVAS session with split rendering on that connection. When a SR session is accepted, the SDP answer shall not contain any other session parameters for the direction towards the lightweight end device than the split rendering related parameters listed above, PI data related parameters and cmr. The other direction (from the lightweight device) remains unconstrained with the only exception that directional parameters shall be used to specify the session on it.

The offer-answer considerations for the remaining EVS parameters are as described in TS 26.445 Annex A.3.3.1 [3].

END OF CHANGES