3GPP TSG-SA WG4 Meeting #130	S4-241967

# USA, Orlando, 18 – 22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR |  | rev |  | Current version: | 0.3.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_MediaEnergyGREEN] Solution for KI2 based on existing UE energy-related information measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MediaEnergyGREEN |  |  |  |  |  | Date: |  |  | 2024-11-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add a potential solution for KI#2 Monitoring and measurement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a potential solution for KI#2 based on existing UE energy-related information measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution for KI#2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2 and 7.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

…

[BatteryMgr]	Android BatteryManager API, https://developer.android.com/reference/kotlin/android/os/BatteryManager

[ohos]	Harmony OS Battery information API, https://developer.huawei.com/consumer/en/doc/harmonyos-references/js-apis-battery-info-V5

…

2nd Change (all new text)

## 7.3	Solution #2: Existing UE energy-related information measurement

### 7.3.1	Key issue mapping

This solution partially addresses Key Issue#2 (Monitoring and measureent) described in clause 6.2.

### 7.3.2	Functional description

This solution does not address energy-related information measurement from the network. It addresses only energy-related information measurement from UE. The two main questions raised in the key issue description are about which UE energy related information is collected and which UE entity is collecting.

Currently, 5G system does not have access to this kind of information on UEs. As a component external to the UE or mechanisms requiring debug mode are not a possible solution in the use cases from this study, the solution to collect energy related information is then to use an existing entity allowing to provide this information to 5G system via existing API.

Most UE OS are already providing tools to assess the energy footprint of applications using system-wide metrics. Each UE OS is not providing the same data about energy consumption. But common basic data allowing to fulfil the requirements in this study are available on two of the three main smartphone OS: Android [BatteryMgr] and Harmony OS [ohos]. The common basic data between those two OSs are:

- Charging status: Battery charging state of the current device. Indicates if the battery is being/not being/fully charged.

- Voltage: Battery voltage of the device, in unit of microvolt.

- Now current: Battery current of the device, in unit of mA.

Using OS API providing this information, 5GMS client is able to measure energy consumption during video consumption into Joules by regularly requesting the current as joules = currentInAmps * timeDifference * voltage.

But this method as several main limitations:

- Such public API are not available on iOS (but some iOS applications are able to provide those basic data).

- Energy consumption during charging phases is not included.

### 7.3.3	Procedures

The 5GMS Client regularly collect defined UE energy-related information (Charging status, voltage and now current), compute energy-related information, and expose the information to 5GMS-Aware Application.

### 7.3.4	Impacts on existing services, entities and interfaces

#### 7.3.4.1	5GMS Client

- Collect, compute and report defined energy-related information provided by UE API to 5GMS-Aware Application.

End of Changes