3GPP TSG-SA WG4 Meeting #130	S4-241968

# USA, Orlando, 18 – 22 November 2024

Source:				Fraunhofer IIS

Title:					3GPP Speech/Audio codecs as WebCodec

Document for:		Discussion & Agreement

Agenda Item:		7.7

## 1.	Introduction

In the current draft TR26.858 v0.0.1 [1] an initial discussion on WebCodecs is captured. Since one of the objectives of the FS_ACAPI SI is to identify interfaces for 3GPP’s speech and audio codecs, the source analyzed the APIs of the codecs currently present in Annex A of [1] and match them to the WebCodecs interface. As a result of this draft definitions have been derived that aim to fully specify the WebCodec interface. It is proposed to review those potential definitions and document them in the TR. It should be noted that eAAC+ already has a WebCodec registration (https://www.w3.org/TR/webcodecs-aac-codec-registration/) and is thus not considered in this document.

## 2.	Potential AMR WebCodec Registration

### 2.1	Overview

This registration defines the potential integration of the Adaptive Multi-Rate (AMR) codec into the WebCodecs API, enabling encoding and decoding of narrowband audio at a fixed sample rate of 8 kHz. AMR is widely used in mobile communications for speech coding due to its efficient bitrate usage. This codec supports narrowband speech compression, robust low-bitrate speech transmission, and includes support for frame loss concealment (FLC) to handle network packet loss and corrupted frames.

### 2.2	AMREncoderConfig Dictionary

The AMREncoderConfig dictionary defines configuration options specific to initializing and controlling the AMR encoder.

| Property | Description | Type | Default |
| --- | --- | --- | --- |
| bitRate | Target bitrate in bits per second (e.g., 4750, 5150, 5900, 12200) | integer | 12200 |
| allowDtx | Enables Discontinuous Transmission (DTX) | boolean | false |

### 2.3	AMRDecoderConfig Dictionary

The AMRDecoderConfig dictionary defines configuration options specific to initializing and controlling the AMR decoder.

| Property | Description | Type | Default |
| --- | --- | --- | --- |
| None | The AMR decoder does not require additional configuration options. | N/A | N/A |

The AMR decoder operates at a fixed sample rate of 8 kHz and requires no further configuration.

### 2.4	AMREncoder Interface

The AMREncoder class is a WebCodecs-compatible implementation of the AudioEncoder interface, providing methods to configure, encode, and release resources for the encoder.

#### Constructor

AMREncoder(AudioEncoderInit init)

- Parameters:

  - init: An AudioEncoderInit dictionary with required callbacks:

    - output: Called when the encoder produces an EncodedAudioChunk.

    - error: Called when an error occurs during encoding.

Example:

const encoder = new AMREncoder({
output: (chunk) => {
console.log("Encoded chunk:", chunk);
},
error: (error) => {
console.error("Encoding error:", error);
}
});

#### Properties

| Property | Description | Type |
| --- | --- | --- |
| state | Current state of the encoder ("configured", "closed", etc.) | CodecState |
| encodeQueueSize | Number of encode requests in the queue | unsigned long |
| ondequeue | Event handler for when the encode queue size changes | EventHandler |

#### Methods

##### configure(config: AMREncoderConfig)

Configures the encoder with the specified settings, initializing resources based on bitrate and DTX options.

encoder.configure({
bitRate: 12200,
allowDtx: true
});

- Parameters:

  - config: An instance of AMREncoderConfig.

- Returns:

  - Configures encoder state for subsequent encoding.

##### encode(data: AudioData)

Encodes a single AudioData chunk of PCM audio data into compressed AMR format.

- Parameters:

  - data: An AudioData object containing PCM audio samples.

- Output:

  - Invokes the output callback with an EncodedAudioChunk containing the encoded AMR data.

##### flush()

Flushes any pending frames in the encoder’s buffer and finalizes the encoded output.

await encoder.flush();

- Returns:

  - A Promise that resolves once all pending frames have been processed.

##### reset()

Resets the encoder, discarding all pending input and output.

encoder.reset();

- Returns:

  - Resets the encoder to its initial state.

##### close()

Releases encoder resources by freeing allocated memory.

encoder.close();

- Returns:

  - Frees encoder instance resources.

##### Static Method: isConfigSupported

Checks if a particular configuration is supported by the AMR encoder.

AMREncoder.isConfigSupported({ bitRate: 12200 }).then(support => {
if (support.supported) {
console.log("AMR encoder configuration is supported");
}
});

- Parameters:

  - config: An instance of AMREncoderConfig.

- Returns:

  - A Promise resolving to AudioEncoderSupport information about support for the specified configuration.

### 2.5	AMRDecoder Interface

The AMRDecoder class is a WebCodecs-compatible implementation of the AudioDecoder interface, providing methods to configure, decode, handle frame loss concealment, and release resources for the decoder.

#### Constructor

AMRDecoder(AudioDecoderInit init)

- Parameters:

  - init: An AudioDecoderInit dictionary with required callbacks:

    - output: Called when the decoder produces an AudioData object.

    - error: Called when an error occurs during decoding.

Example:

const decoder = new AMRDecoder({
output: (outputData) => {
console.log("Decoded data:", outputData);
},
error: (error) => {
console.error("Decoding error:", error);
}
});

#### Properties

| Property | Description | Type |
| --- | --- | --- |
| state | Current state of the decoder ("configured", "closed", etc.) | CodecState |
| decodeQueueSize | Number of decode requests in the queue | unsigned long |
| ondequeue | Event handler for when the decode queue size changes | EventHandler |

#### Methods

##### configure(config: AMRDecoderConfig)

Configures the decoder with the specified settings. AMR decoding operates at a fixed sample rate of 8 kHz and does not require additional settings.

decoder.configure({});

- Parameters:

  - config: An empty instance of AMRDecoderConfig.

- Returns:

  - Configures decoder state for subsequent decoding.

##### decode(chunk: EncodedAudioChunk or null)

Decodes a single EncodedAudioChunk of AMR data into raw PCM audio format. For frame loss concealment, the decoder accepts null: - If null is passed, the decoder performs frame loss concealment to generate a replacement frame. - Otherwise, it decodes the provided data as a normal frame.

decoder.decode(null); // Uses FLC for a missing or corrupted frame

- Parameters:

  - chunk: An EncodedAudioChunk object containing compressed AMR data or null to trigger frame loss concealment.

##### flush()

Flushes any pending frames in the decoder’s buffer and finalizes the decoded output.

await decoder.flush();

- Returns:

  - A Promise that resolves once all pending frames have been processed.

##### reset()

Resets the decoder, discarding all pending input and output.

decoder.reset();

- Returns:

  - Resets the decoder to its initial state.

##### close()

Releases decoder resources by freeing allocated memory.

decoder.close();

- Returns:

  - Frees decoder instance resources.

##### Static Method: isConfigSupported

Checks if a particular configuration is supported by the AMR decoder.

AMRDecoder.isConfigSupported({}).then(support => {
if (support.supported) {
console.log("AMR decoder configuration is supported");
}
});

- Parameters:

  - config: An instance of AMRDecoderConfig.

- Returns:

  - A Promise resolving to AudioDecoderSupport information about support for the specified configuration.

## 3.	Potential AMR-WB WebCodec Registration

### 3.1	Overview

This registration defines the potential integration of the 3GPP Adaptive Multi-Rate Wideband (AMR-WB) codec into the WebCodecs API, enabling encoding and decoding of wideband audio at a fixed sample rate of 16 kHz. AMR-WB is widely used in telecommunications for high-quality speech coding. This codec supports wideband speech compression, offering enhanced audio quality over narrowband codecs and providing support for frame loss concealment (FLC) to handle network packet loss and corrupted frames.

### 3.2	AMRWBEncoderConfig Dictionary

The AMRWBEncoderConfig dictionary defines configuration options specific to initializing and controlling the AMR-WB encoder.

| Property | Description | Type | Default |
| --- | --- | --- | --- |
| bitRate | Target bitrate in bits per second (e.g., 6600, 8850, 12650, 23850) | integer | 23850 |
| allowDtx | Enables Discontinuous Transmission (DTX) | boolean | false |

### 3.3	AMRWBDecoderConfig Dictionary

The AMRWBDecoderConfig dictionary defines configuration options specific to initializing and controlling the AMR-WB decoder.

| Property | Description | Type | Default |
| --- | --- | --- | --- |
| None | The AMR-WB decoder does not require additional configuration options. | N/A | N/A |

The AMR-WB decoder operates at a fixed sample rate of 16 kHz and requires no further configuration.

### 3.4	AMRWBEncoder Interface

The AMRWBEncoder class is a WebCodecs-compatible implementation of the AudioEncoder interface, providing methods to configure, encode, and release resources for the encoder.

#### Constructor

AMRWBEncoder(AudioEncoderInit init)

- Parameters:

  - init: An AudioEncoderInit dictionary with required callbacks:

    - output: Called when the encoder produces an EncodedAudioChunk.

    - error: Called when an error occurs during encoding.

Example:

const encoder = new AMRWBEncoder({
output: (chunk) => {
console.log("Encoded chunk:", chunk);
},
error: (error) => {
console.error("Encoding error:", error);
}
});

#### Properties

| Property | Description | Type |
| --- | --- | --- |
| state | Current state of the encoder ("configured", "closed", etc.) | CodecState |
| encodeQueueSize | Number of encode requests in the queue | unsigned long |
| ondequeue | Event handler for when the encode queue size changes | EventHandler |

#### Methods

##### configure(config: AMRWBEncoderConfig)

Configures the encoder with the specified settings, initializing resources based on bitrate and DTX options.

encoder.configure({
bitRate: 23850,
allowDtx: true
});

- Parameters:

  - config: An instance of AMRWBEncoderConfig.

- Returns:

  - Configures encoder state for subsequent encoding.

##### encode(data: AudioData)

Encodes a single AudioData chunk of PCM audio data into compressed AMR-WB format.

- Parameters:

  - data: An AudioData object containing PCM audio samples.

- Output:

  - Invokes the output callback with an EncodedAudioChunk containing the encoded AMR-WB data.

##### flush()

Flushes any pending frames in the encoder’s buffer and finalizes the encoded output.

await encoder.flush();

- Returns:

  - A Promise that resolves once all pending frames have been processed.

##### reset()

Resets the encoder, discarding all pending input and output.

encoder.reset();

- Returns:

  - Resets the encoder to its initial state.

##### close()

Releases encoder resources by freeing allocated memory.

encoder.close();

- Returns:

  - Frees encoder instance resources.

##### Static Method: isConfigSupported

Checks if a particular configuration is supported by the AMR-WB encoder.

AMRWBEncoder.isConfigSupported({ bitRate: 23850 }).then(support => {
if (support.supported) {
console.log("AMR-WB encoder configuration is supported");
}
});

- Parameters:

  - config: An instance of AMRWBEncoderConfig.

- Returns:

  - A Promise resolving to AudioEncoderSupport information about support for the specified configuration.

### 3.5	AMRWBDecoder Interface

The AMRWBDecoder class is a WebCodecs-compatible implementation of the AudioDecoder interface, providing methods to configure, decode, handle frame loss concealment, and release resources for the decoder.

#### Constructor

AMRWBDecoder(AudioDecoderInit init)

- Parameters:

  - init: An AudioDecoderInit dictionary with required callbacks:

    - output: Called when the decoder produces an AudioData object.

    - error: Called when an error occurs during decoding.

Example:

const decoder = new AMRWBDecoder({
output: (outputData) => {
console.log("Decoded data:", outputData);
},
error: (error) => {
console.error("Decoding error:", error);
}
});

#### Properties

| Property | Description | Type |
| --- | --- | --- |
| state | Current state of the decoder ("configured", "closed", etc.) | CodecState |
| decodeQueueSize | Number of decode requests in the queue | unsigned long |
| ondequeue | Event handler for when the decode queue size changes | EventHandler |

#### Methods

##### configure(config: AMRWBDecoderConfig)

Configures the decoder with the specified settings. AMR-WB decoding operates at a fixed sample rate of 16 kHz and does not require additional settings.

decoder.configure({});

- Parameters:

  - config: An empty instance of AMRWBDecoderConfig.

- Returns:

  - Configures decoder state for subsequent decoding.

##### decode(chunk: EncodedAudioChunk or null)

Decodes a single EncodedAudioChunk of AMR-WB data into raw PCM audio format. For frame loss concealment, the decoder accepts null: - If null is passed, the decoder performs frame loss concealment to generate a replacement frame. - Otherwise, it decodes the provided data as a normal frame.

decoder.decode(null); // Uses FLC for a missing or corrupted frame

- Parameters:

  - chunk: An EncodedAudioChunk object containing compressed AMR-WB data or null to trigger frame loss concealment.

##### flush()

Flushes any pending frames in the decoder’s buffer and finalizes the decoded output.

await decoder.flush();

- Returns:

  - A Promise that resolves once all pending frames have been processed.

##### reset()

Resets the decoder, discarding all pending input and output.

decoder.reset();

- Returns:

  - Resets the decoder to its initial state.

##### close()

Releases decoder resources by freeing allocated memory.

decoder.close();

- Returns:

  - Frees decoder instance resources.

##### Static Method: isConfigSupported

Checks if a particular configuration is supported by the AMR-WB decoder.

AMRWBDecoder.isConfigSupported({}).then(support => {
if (support.supported) {
console.log("AMR-WB decoder configuration is supported");
}
});

- Parameters:

  - config: An instance of AMRWBDecoderConfig.

- Returns:

  - A Promise resolving to AudioDecoderSupport information about support for the specified configuration.

### 3.6	MIME Type

The MIME type for the AMR-WB codec within WebCodecs is audio/amr-wb, consistent with existing media standards.

## 4.	Potential AMR-WB+ WebCodec Registration

### 4.1	Overview

This registration defines the potential integration of the 3GPP Adaptive Multi-Rate Wideband Plus (AMR-WB+) codec into the WebCodecs API, enabling encoding and decoding of wideband and super-wideband audio at a fixed sample rate of 16 kHz or higher. AMR-WB+ is designed for high-quality audio and supports both mono and stereo configurations, making it suitable for various audio applications requiring robust performance in low-bitrate environments. This codec includes support for frame loss concealment (FLC) to handle network packet loss and corrupted frames.

### 4.2	AMRWBPlusEncoderConfig Dictionary

The AMRWBPlusEncoderConfig dictionary defines configuration options specific to initializing and controlling the AMR-WB+ encoder.

| Property | Description | Type | Default |
| --- | --- | --- | --- |
| bitRate | Target bitrate in bits per second (e.g., 8000, 16000, 24000) | integer | 24000 |
| stereo | Indicates if the encoder should operate in stereo mode | boolean | false |

### 4.3	AMRWBPlusDecoderConfig Dictionary

The AMRWBPlusDecoderConfig dictionary defines configuration options specific to initializing and controlling the AMR-WB+ decoder.

| Property | Description | Type | Default |
| --- | --- | --- | --- |
| stereo | Indicates if the decoder should operate in stereo mode | boolean | false |

The AMR-WB+ decoder operates at a sample rate of 16 kHz or higher, depending on the configuration, and can be set to decode stereo or mono audio.

### 4.4	AMRWBPlusEncoder Interface

The AMRWBPlusEncoder class is a WebCodecs-compatible implementation of the AudioEncoder interface, providing methods to configure, encode, and release resources for the encoder.

#### Constructor

AMRWBPlusEncoder(AudioEncoderInit init)

- Parameters:

  - init: An AudioEncoderInit dictionary with required callbacks:

    - output: Called when the encoder produces an EncodedAudioChunk.

    - error: Called when an error occurs during encoding.

Example:

const encoder = new AMRWBPlusEncoder({
output: (chunk) => {
console.log("Encoded chunk:", chunk);
},
error: (error) => {
console.error("Encoding error:", error);
}
});

#### Properties

| Property | Description | Type |
| --- | --- | --- |
| state | Current state of the encoder ("configured", "closed", etc.) | CodecState |
| encodeQueueSize | Number of encode requests in the queue | unsigned long |
| ondequeue | Event handler for when the encode queue size changes | EventHandler |

#### Methods

##### configure(config: AMRWBPlusEncoderConfig)

Configures the encoder with the specified settings, including bitrate and stereo options.

encoder.configure({
bitRate: 24000,
stereo: true
});

- Parameters:

  - config: An instance of AMRWBPlusEncoderConfig.

- Returns:

  - Configures encoder state for subsequent encoding.

##### encode(data: AudioData)

Encodes a single AudioData chunk of PCM audio data into compressed AMR-WB+ format.

- Parameters:

  - data: An AudioData object containing PCM audio samples.

- Output:

  - Invokes the output callback with an EncodedAudioChunk containing the encoded AMR-WB+ data.

##### flush()

Flushes any pending frames in the encoder’s buffer and finalizes the encoded output.

await encoder.flush();

- Returns:

  - A Promise that resolves once all pending frames have been processed.

##### reset()

Resets the encoder, discarding all pending input and output.

encoder.reset();

- Returns:

  - Resets the encoder to its initial state.

##### close()

Releases encoder resources by freeing allocated memory.

encoder.close();

- Returns:

  - Frees encoder instance resources.

##### Static Method: isConfigSupported

Checks if a particular configuration is supported by the AMR-WB+ encoder.

AMRWBPlusEncoder.isConfigSupported({ bitRate: 24000, stereo: true }).then(support => {
if (support.supported) {
console.log("AMR-WB+ encoder configuration is supported");
}
});

- Parameters:

  - config: An instance of AMRWBPlusEncoderConfig.

- Returns:

  - A Promise resolving to AudioEncoderSupport information about support for the specified configuration.

### 4.5	AMRWBPlusDecoder Interface

The AMRWBPlusDecoder class is a WebCodecs-compatible implementation of the AudioDecoder interface, providing methods to configure, decode, handle frame loss concealment, and release resources for the decoder.

#### Constructor

AMRWBPlusDecoder(AudioDecoderInit init)

- Parameters:

  - init: An AudioDecoderInit dictionary with required callbacks:

    - output: Called when the decoder produces an AudioData object.

    - error: Called when an error occurs during decoding.

Example:

const decoder = new AMRWBPlusDecoder({
output: (outputData) => {
console.log("Decoded data:", outputData);
},
error: (error) => {
console.error("Decoding error:", error);
}
});

#### Properties

| Property | Description | Type |
| --- | --- | --- |
| state | Current state of the decoder ("configured", "closed", etc.) | CodecState |
| decodeQueueSize | Number of decode requests in the queue | unsigned long |
| ondequeue | Event handler for when the decode queue size changes | EventHandler |

#### Methods

##### configure(config: AMRWBPlusDecoderConfig)

Configures the decoder with the specified settings. AMR-WB+ decoding operates at a sample rate of 16 kHz or higher, depending on the configuration, and can support stereo or mono decoding.

decoder.configure({ stereo: true });

- Parameters:

  - config: An instance of AMRWBPlusDecoderConfig.

- Returns:

  - Configures decoder state for subsequent decoding.

##### decode(chunk: EncodedAudioChunk or null)

Decodes a single EncodedAudioChunk of AMR-WB+ data into raw PCM audio format. For frame loss concealment, the decoder accepts null: - If null is passed, the decoder performs frame loss concealment to generate a replacement frame. - Otherwise, it decodes the provided data as a normal frame.

decoder.decode(null); // Uses FLC for a missing or corrupted frame

- Parameters:

  - chunk: An EncodedAudioChunk object containing compressed AMR-WB+ data or null to trigger frame loss concealment.

##### flush()

Flushes any pending frames in the decoder’s buffer and finalizes the decoded output.

await decoder.flush();

- Returns:

  - A Promise that resolves once all pending frames have been processed.

##### reset()

Resets the decoder, discarding all pending input and output.

decoder.reset();

- Returns:

  - Resets the decoder to its initial state.

##### close()

Releases decoder resources by freeing allocated memory.

decoder.close();

- Returns:

  - Frees decoder instance resources.

##### Static Method: isConfigSupported

Checks if a particular configuration is supported by the AMR-WB+ decoder.

AMRWBPlusDecoder.isConfigSupported({ stereo: true }).then(support => {
if (support.supported) {
console.log("AMR-WB+ decoder configuration is supported");
}
});

- Parameters:

  - config: An instance of AMRWBPlusDecoderConfig.

- Returns:

  - A Promise resolving to AudioDecoderSupport information about support for the specified configuration.

### 4.6	MIME Type

The MIME type for the AMR-WB+ codec within WebCodecs is audio/amr-wb+, consistent with existing media standards.

## 5.	Potential EVS WebCodec registration

### 5.1	Overview

This registration defines the potential integration of the Enhanced Voice Services (EVS) codec into the WebCodecs API, enabling encoding and decoding of wideband (16 kHz) to fullband (48 kHz) audio. EVS provides high-quality audio compression and supports mono output configurations. This codec is well-suited for modern telecommunications applications that require high fidelity, low latency, and resilience to packet loss, with support for frame loss concealment (FLC) to handle network packet loss and corrupted frames.

### 5.2	EVSEncoderConfig Dictionary

The EVSEncoderConfig dictionary defines configuration options specific to initializing and controlling the EVS encoder.

| Property | Description | Type | Default |
| --- | --- | --- | --- |
| bitRate | Target bitrate in bits per second (e.g., 9600, 13200, 24400) | integer | 13200 |
| bandwidth | Audio bandwidth mode (e.g., "NB", "WB", "SWB", "FB") | string | "WB" |
| dtx | Discontinuous Transmission (DTX) interval for adaptive SID (0 for adaptive intervals) | integer | 0 |
| partialCopyOffset | Offset of partial copies for channel-aware mode (bitrate=13200) | integer | 3 |
| fecIndicator | FEC indicator for channel-aware mode (either HI or LO) | string | "HI" |

The encoder uses the sample rate provided in the AudioData object, so no separate sampleRate configuration is needed.

### 5.3	EVSDecoderConfig Dictionary

The EVSDecoderConfig dictionary defines configuration options specific to initializing and controlling the EVS decoder.

| Property | Description | Type | Default |
| --- | --- | --- | --- |
| outputSampleRate | Output sample rate in Hz (8000, 16000, 32000, 48000) | integer | 16000 |

The EVS decoder can be configured for various output sample rates but supports only mono output.

### 5.4	EVSEncoder Interface

The EVSEncoder class is a WebCodecs-compatible implementation of the AudioEncoder interface, providing methods to configure, encode, and release resources for the encoder.

#### Constructor

EVSEncoder(AudioEncoderInit init)

- Parameters:

  - init: An AudioEncoderInit dictionary with required callbacks:

    - output: Called when the encoder produces an EncodedAudioChunk.

    - error: Called when an error occurs during encoding.

Example:

const encoder = new EVSEncoder({
output: (chunk) => {
console.log("Encoded chunk:", chunk);
},
error: (error) => {
console.error("Encoding error:", error);
}
});

#### Properties

| Property | Description | Type |
| --- | --- | --- |
| state | Current state of the encoder ("configured", "closed", etc.) | CodecState |
| encodeQueueSize | Number of encode requests in the queue | unsigned long |
| ondequeue | Event handler for when the encode queue size changes | EventHandler |

#### Methods

##### configure(config: EVSEncoderConfig)

Configures the encoder with the specified settings, including bitrate, bandwidth, DTX interval, partial copy offset, and FEC indicator.

encoder.configure({
bitRate: 13200,
bandwidth: "WB",
dtx: 0,
partialCopyOffset: 3,
fecIndicator: "HI"
});

- Parameters:

  - config: An instance of EVSEncoderConfig.

- Returns:

  - Configures encoder state for subsequent encoding.

##### encode(data: AudioData)

Encodes a single AudioData chunk of PCM audio data into compressed EVS format.

- Parameters:

  - data: An AudioData object containing PCM audio samples.

- Output:

  - Invokes the output callback with an EncodedAudioChunk containing the encoded EVS data.

##### flush()

Flushes any pending frames in the encoder’s buffer and finalizes the encoded output.

await encoder.flush();

- Returns:

  - A Promise that resolves once all pending frames have been processed.

##### reset()

Resets the encoder, discarding all pending input and output.

encoder.reset();

- Returns:

  - Resets the encoder to its initial state.

##### close()

Releases encoder resources by freeing allocated memory.

encoder.close();

- Returns:

  - Frees encoder instance resources.

##### Static Method: isConfigSupported

Checks if a particular configuration is supported by the EVS encoder.

EVSEncoder.isConfigSupported({ bitRate: 13200, bandwidth: "WB", dtx: 0, fecIndicator: "HI" }).then(support => {
if (support.supported) {
console.log("EVS encoder configuration is supported");
}
});

- Parameters:

  - config: An instance of EVSEncoderConfig.

- Returns:

  - A Promise resolving to AudioEncoderSupport information about support for the specified configuration.

### 5.5	EVSDecoder Interface

The EVSDecoder class is a WebCodecs-compatible implementation of the AudioDecoder interface, providing methods to configure, decode, handle frame loss concealment, and release resources for the decoder.

#### Constructor

EVSDecoder(AudioDecoderInit init)

- Parameters:

  - init: An AudioDecoderInit dictionary with required callbacks:

    - output: Called when the decoder produces an AudioData object.

    - error: Called when an error occurs during decoding.

Example:

const decoder = new EVSDecoder({
output: (outputData) => {
console.log("Decoded data:", outputData);
},
error: (error) => {
console.error("Decoding error:", error);
}
});

#### Properties

| Property | Description | Type |
| --- | --- | --- |
| state | Current state of the decoder ("configured", "closed", etc.) | CodecState |
| decodeQueueSize | Number of decode requests in the queue | unsigned long |
| ondequeue | Event handler for when the decode queue size changes | EventHandler |

#### Methods

##### configure(config: EVSDecoderConfig)

Configures the decoder with the specified settings. EVS decoding supports a range of output sample rates.

decoder.configure({ outputSampleRate: 16000 });

- Parameters:

  - config: An instance of EVSDecoderConfig.

- Returns:

  - Configures decoder state for subsequent decoding.

##### decode(chunk: EncodedAudioChunk or null)

Decodes a single EncodedAudioChunk of EVS data into raw PCM audio format. For frame loss concealment, the decoder accepts null: - If null is passed, the decoder performs frame loss concealment to generate a replacement frame. - Otherwise, it decodes the provided data as a normal frame.

decoder.decode(null); // Uses FLC for a missing or corrupted frame

- Parameters:

  - chunk: An EncodedAudioChunk object containing compressed EVS data or null to trigger frame loss concealment.

##### flush()

Flushes any pending frames in the decoder’s buffer and finalizes the decoded output.

await decoder.flush();

- Returns:

  - A Promise that resolves once all pending frames have been processed.

##### reset()

Resets the decoder, discarding all pending input and output.

decoder.reset();

- Returns:

  - Resets the decoder to its initial state.

##### close()

Releases decoder resources by freeing allocated memory.

decoder.close();

- Returns:

  - Frees decoder instance resources.

##### Static Method: isConfigSupported

Checks if a particular configuration is supported by the EVS decoder.

EVSDecoder.isConfigSupported({ outputSampleRate: 16000 }).then(support => {
if (support.supported) {
console.log("EVS decoder configuration is supported");
}
});

- Parameters:

  - config: An instance of EVSDecoderConfig.

- Returns:

  - A Promise resolving to AudioDecoderSupport information about support for the specified configuration.

### 5.6	MIME Type

The MIME type for the EVS codec within WebCodecs is audio/evs, consistent with existing media standards.

## 6.	Summary

The source presented potential draft registrations for many of the 3GPP speech and audio codecs. The source encourages to review those, especially the configurations. Those are again summarized in Tables 1 and 2.

Table 1: Encoder Configuration Elements

| Codec | Property | Description | Type | Default |
| --- | --- | --- | --- | --- |
| AMR | bitRate | Bitrate in bits per second (e.g., 4750, 5150, 12200) | integer | 12200 |
| AMR | allowDtx | Enable Discontinuous Transmission (DTX) | boolean | false |
| AMR-WB | bitRate | Bitrate in bits per second (e.g., 6600, 8850, 23850) | integer | 23850 |
| AMR-WB | allowDtx | Enable Discontinuous Transmission (DTX) | boolean | false |
| AMR-WB+ | bitRate | Bitrate in bits per second (e.g., 8000, 16000, 24000) | integer | 24000 |
| AMR-WB+ | stereo | Enable stereo encoding | boolean | false |
| EVS | bitRate | Bitrate in bits per second (e.g., 9600, 13200, 24400) | integer | 13200 |
| EVS | bandwidth | Bandwidth mode (e.g., "NB", "WB", "SWB", "FB") | string | WB |
| EVS | dtx | DTX interval for adaptive SID (0 for adaptive intervals) | integer | 0 |
| EVS | partialCopyOffset | Offset for partial copies in channel-aware mode | integer | 3 |
| EVS | fecIndicator | FEC indicator for channel-aware mode (either "HI" or "LO") | string | HI |

Table 2: Decoder Configuration Elements

| Codec | Property | Description | Type | Default |
| --- | --- | --- | --- | --- |
| AMR | None | AMR decoder does not require additional config options | N/A | N/A |
| AMR-WB | None | AMR-WB decoder does not require additional config options | N/A | N/A |
| AMR-WB+ | stereo | Enable stereo decoding | boolean | false |
| EVS | outputSampleRate | Output sample rate (8000, 16000, 32000, 48000) | integer | 16000 |

It is then proposed to include the overview tables into the main body of  [1] and the specific full text potential registrations also into the main body or an Annex.

## 7.	References

[1] 3GPP S4-241215, 3GPP TR 26.858, “APIs for 3GPP Speech and Audio Codecs”, v0.0.1