3GPP TSG-SA WG4 Meeting #130 	S4-242014

Orlando, USA, 18th November–22nd November 2024

Title:	[FS-AMD WT 3b]: Further details on application layer considerations with multi-access media delivery

Agenda Item:	8.6

Source:	Samsung Electronics Co Ltd.

Introduction

Document S4aI240193 on the topic of application layer considerations with multi-access media delivery was submitted to Post-SA4#129e meeting and discussed. There were some comments during the presentation of that document. This contribution addresses those comments, and provides an update. This contribution also proposes a summary and conclusions of the study topic.

Background and motivation

Document S4aI240177, and subsequently clause 5.15.6.2 in endorsed candidate baseline CR#13 in S4aI240192, proposed to extend the media stream handling client API exposed by the Media Stream Handler to the 5GMS-Aware Application at reference point M7 and to the Media Session Handler at reference point M11 as follows:

1.	For the 5GMS-Aware Application to configure the following multipath delivery parameters:

-	Enable/disable multipath media delivery.

-	The number of MPQUIC paths or MPTCP subflows in the multipath delivery connection.

-	Add new or remove existing MPQUIC path or MPTCP subflows to/from the multipath delivery connection.

-	Which media application flows are mapped onto which MPQUIC path or MPTCP subflow.

These objectives may be achieved by modifying the Configurations and settings API specified in clause 13.2.4 of TS 26.512 [16].

2.	For the 5GMS-Aware Application to be informed of the following:

-	Connection endpoint information to each of the MPQUIC path or MPTCP subflow.

-	Status information of multipath delivery connection.

These objectives may be achieved by modifying the Dynamic Status Information specified in clause 13.2.6 of TS 26.512 [16].

Document S4aI240193 described the modifications to the Configurations and settings API specified in clause 13.2.4 and Dynamic Status Information specified in clause 13.2.6 of TS 26512 as indicated above to support application configuration. However, there were some comments during the presentation of S4aI240193 (meeting minutes link: https://docs.google.com/document/d/149-Zl8YpuhH-Np_XwznANd73gkz_hlbcN49Ecn1Ft0c/edit?tab=t.0 )

S4aI240193; [FS-AMD WT 3b]: Further details on application layer considerations with multi-access media delivery; Samsung Research America; Prakash Kolan. E-mail Discussion: Revisions: none Presenter: Prakash Kolan Online Discussion: - Prakash presents. - Prakash: I don’t think we are modifying the entry point here, related to Thomas’ comment on 192. - Richard: I prefer to express the possible changes to the spec as a bullet list and not as a spec diff. It feels that it goes too far for a FS. - Richard: On the first table, you are suggesting enabling multipath delivery as a static information but maybe it could per client. This goes probably too fast to the solution. On the second table, it seems like a stage 3 proposal but the element is just a string what should be an object but then you are limited because this is only stage 2 right now. - Prakash: It’s ok with me to just have a bullet list of possible changes. - Thomas: I am lost of what this is about. What would a player do with this? I would like to check with player developers how useful this information is. - Prakash: Both MSH and media player terminate M7. You need to inform the MSH and maybe yes the media player might not do much with this information. Decision: S4aI240193 is noted.

Here are the responses to the above comments:

1. Richard: I prefer to express the possible changes to the spec as a bullet list and not as a spec diff. It feels that it goes too far for a FS.

[Prakash]  Understand, and agree to the proposal. This contribution proposes the update as a simple bulleted list as suggested.

2. Richard: On the first table, you are suggesting enabling multipath delivery as a static information but maybe it could per client. This goes probably too fast to the solution. On the second table, it seems like a stage 3 proposal but the element is just a string what should be an object but then you are limited because this is only stage 2 right now.

[Prakash]  Yes, I am open to defining a detailed structure for the response object, but would like to go with a simple string to start with.

3. Thomas: I am lost of what this is about. What would a player do with this? I would like to check with player developers how useful this information is.

[Prakash]  As responded to during the presentation of the contribution, we need application configuration of transport parameters, and there is only one place in our spec where the configuration happens – over M7 and M11. Hence this proposal of updating the existing API to configure some transport parameters. It is understandable that a Media Player may not know how to use these parameters, but the M7 API terminates at the Media Access Function / Media Stream Handler as well. Hence, it may be prudent to enhance the M7 and M11 API, and have those functions use the configuration as appropriate.

Text proposal

The following textual changes to TR 26.804 CR0013 are proposed based on above discussion. In addition, summary and conclusion text for this study topic is proposed.

# * * * * First change * * * *

### 5.15.6	Candidate solutions

Editor’s Note:	Candidate solutions for identified key issue.

#### 5.15.6.1	Multi-access downlink media streaming using CMMF

This candidate solution includes approaches where a 5GMSd Client accesses and downloads, via reference point M4d, CMMF-encoded media objects [CMMF – reference included within CR adding clause 5.19], and possibly original source media (e.g., MPEG-DASH or HLS media segments), over multiple access networks simultaneously from a single 5GMSd AS. CMMF (discussed in detail within clause 5.19.6.3) enables multi-access capabilities through application-layer implementations of the 5GMSd Client without requiring lower-layer (e.g., network, transport, etc.) multi-access integrations.

In this solution, multiple different CMMF-encoded bitstreams/objects (or representations) of the source media are stored/cached within a single logical 5GMSd AS. A CMMF-enabled 5GMSd Client requests and downloads a different CMMF-encoded representation (stripe) of the required original source media over each of the access networks available to it. These (potentially partially) received CMMF bitstreams/objects are decoded by a CMMF decoder in the 5GMSd Client yielding the required original source content once enough information has been received over all of the available access networks.

Unlike other multi-access technologies such as MPTCP, MPQUIC, ATSSS, etc., the responsibility to set up, request, and steer the delivery of content across each available access network rests with the application layer (e.g., Media Player) in this solution. For example, a Media Player sets up multiple HTTP connections in parallel, each one bound to a different network interface (each assigned with an IP address appropriate to that network). Requests to a single 5GMSd AS for different CMMF-encoded representations (stripes) of the original source media are sent from each of the HTTP connections over the different access networks. The HTTP response from the 5GMSd AS to each of these requests is routed appropriately over the appropriate access network, following standard network-layer/IP routing rules and procedures.

As mentioned above, traffic steering over each access network may be performed by the application layer (e.g., Media Player); and multiple policies can be defined/implemented based on the desired outcome. For example, a best-effort policy may evolve downloading as much CMMF-encoded content from each available access network until the CMMF decoder can successfully decode the required media. Networks that have lower latency, higher bandwidth, etc. will naturally contribute more to the download than those with higher latency, lower bandwidth, etc. Another policy may preference delivery of content from one access network over another. In this case, a schedular may be implemented so that requests of CMMF-encoded content made to the Application Server can be throttled over one access network so that the majority of the download is completed on the other(s).

Integration of CMMF within the 5GMS System is discussed in detail within clause 5.19.6.3.

#### 5.15.6.2	Multi-Access media delivery using ATSSS

To address the gaps identified in clause 5.15.5.2, it is proposed to extend the media stream handling client API exposed by the Media Stream Handler to the 5GMS-Aware Application at reference point M6 and to the Media Session Handler at reference point M11 as follows:

1.	For the 5GMS-Aware Application to configure the following multipath delivery parameters:

-	Enable/disable multipath media delivery.

-	The number of MPQUIC paths or MPTCP subflows in the multipath delivery connection.

-	Add new or remove existing MPQUIC path or MPTCP subflows to/from the multipath delivery connection.

-	Which media application flows are mapped onto which MPQUIC path or MPTCP subflow.

These objectives are achieved by enhancing the parameters to the Configurations and settings API specified in clause 13.2.4 of TS 26.512 [16] as follows:

-	Add a new property (e.g., enableMultipathDelivery of type Boolean) to configure whether the 5GMS-Aware Application intends to use multipath delivery using multiple access network connection endpoints.

-	Add a new property (e.g., pathsForMultipathDelivery of type Integer) to configure the number of paths to be used by the Media Stream Handler for the multipath delivery connection. In the case of MPQUIC based multipath delivery, this property represents the number of MPQUIC paths. In case of MPTCP based multipath delivery, this property represents the number of MPTCP sub flows.

-	Add a new property (e.g., accessNetworkEndpoints of type Array) to configure the set of access network endpoints over which the multipath delivery connection is to be used.

2.	For the 5GMS-Aware Application to be informed of the following:

-	Connection endpoint information to each of the MPQUIC path or MPTCP subflow.

-	Status information of multipath delivery connection.

These objectives are achieved by enhancing the Dynamic Status Information specified in clause 13.2.6 of TS 26.512 [16] as follows:

-	Add a new property (e.g., multipathDeliveryStatus of type string) to convey status information of multipath delivery connection to the 5GMS-Aware Application.

Editor's Note: Detailed formats of the above information are to be developed in conjunction with study progress documented in clause 5.24 of present document.

### 5.15.7	Summary and conclusions

Multi-access media delivery enables media streaming applications to efficiently access content over multiple access networks. The study looked into existing specification related to ATSSS (Access Trafffic Steering Switching and Splitting) architecture in TS 23501 [23501] and TS 23502 [23502] to identify its impact on 5GMS. Topics related to application awareness and influence on multi-access delivery, potential enhancements to dynamic policy feature of 5GMS to support multiple access paths, and network assistance with multi-access delivery were studied. The study documented collaboration scenarios and ATSSS architecture mapping into 5GMS.

It is recommended that:

1.	A brief description on multi-access media delivery based on clause 5.15.1 of present document and ATSSS mapping into 5GMS architecture described in clause 5.15.3.2 of present document be included in TS 26501 [26501] as an informative annex

2.	The changes to the Configuration Settings API and Dynamic Status Information API as described in clause 5.15.6.2 of the present document to allow for application configuration and status information exchange for multi-access media delivery be implemented in TS 26510 [26510]

# * * * * End of changes * * * *