3GPP TSG-SA WG4 Meeting #130						S4-242015

USA, Orlando, 18 – 22 November 2024

Source:	Nokia

Title:	On acoustic echo control testing and on UE type combinations

Agenda item:	7.8

Document for:	DISCUSSION

# Introduction

During the first phase of ATIAS work item, the assessment of the acoustic echo cancelling (AEC) performance of the UEs was not discussed. As the AEC performance of the UE is crucial component for ensuring a proper call quality, it is necessary to include test method(s) for covering such aspects. With immersive capture and playback, the task of attenuating echo will be more complex than before, but at least for initial assessment traditional methods can be utilized with minor adjustments.

In this contribution, a test method for assessing the AEC performance based on Terminal Coupling Loss (TCL) with far-end user transmitting immersive audio is presented. In addition, clarification on test arrangement definition for testing a combination of multiple individual UE-types simultaneously is discussed.

# Discussion

## 2.1 UE type combinations

With immersive audio, it is very likely that multiple devices will be used as combined acoustic interface. While typically sending and receiving interfaces are integrated into the same device, this may not be always the case. For example, UE for immersive voice communication could be a combination of a headset and a mobile phone, where the headset is used only for the audio playback (receiving), and the mobile phone for the audio capture (sending). Typically, testing in sending and receiving directions are done separately, but in certain cases it is necessary to have both sending and receiving interfaces in the same test setup, e.g., when assessing echo. Furthermore, as the recently standardized IVAS codec supports transmission of combined audio formats, both the headset and the mobile phone may be used for a simultaneous capture.

A UE can be a combination of any specified UE type in any direction (send/receive). A simple way to define the test setup for combinations of different UE types is to use already specified setups in 3GPP TS 26.260 [1]. Furthermore, relative positions between individual devices forming the combined UE can be derived from the position relative to a simulated user, i.e., positioning the individual devices according to their respective arrangement for receiving direction testing.

However, when testing a spatial capture of UE comprising multiple different UE types and simultaneous capture, the specified sound source position of individual UE types may conflict. To overcome any ambiguities, common definition for spatial sound source position at 1 meter distance from the horizontal center of the combined UE could be enforced. Height of the sound source should comply with all individual UE type definitions. With user capture testing, the sound source would be naturally HATS. As the Headset UE has slightly quieter playback level definition compared to other UE types, either -4.7 dB Pa or -1.7 dB Pa should be defined as a common playback level to be used with any UE combination.

Two example setups of different device combinations are visualized in Figure 1 and 2. In the first figure, an example test setup for assessing the acoustic echo cancellation of a combined UE comprising Headset UE in receiving direction and Handheld hands-free (HHHF) UE in sending direction is visualized. According to clause 5.4.2.3 in 3GPP TS 26.260, the Headset UE is placed on the HATS in their recommended wearing position. Furthermore, the HATS (wearing the Headset UE) is positioned at 42cm from the center point of the visual display of the HHHF UE, as defined in clause 5.4.2.4.

Figure 1 Example setup for combined Headset and Handheld hands-free UE

In Figure 2, an example setup for testing a simultaneous spatial capture of UE comprising Headset UE and Table-mounted UE in sending direction is illustrated. According to clause 5.4.3.2 the Headset UE is placed on the HATS in their recommended wearing position. Furthermore, HATS is positioned at a distance of 40cm between the Table-mounted UE and lip reference plane and a height of 40cm as defined in clause 5.4.3.4. The sound source in spatial capture is positioned at 1m from the horizontal center of the combined UE. Height is according to both UE type setup definitions.

Figure 2 Example setups for combined Headset and Table-mounted UE

Based on the presented observations, following definition in the brackets is proposed to be documented for possible further integration in the TS 26.260.

[

5.4.2.8 Combination of multiple UE types

The tested UE can be a combination of any specified UE types in any direction (receive/send). If not specified otherwise, the arrangement for testing any combination of UE types shall be according to the arrangements specified for receiving direction testing in clauses 5.4.2.2 – 5.4.2.7 for the individual UE types.

If combined UE type comprises one UE type in sending direction, testing of User capture or Spatial capture shall be done according to the definition of the individual UE type used in sending direction.

If combined UE comprises two or more UE types simultaneously in sending direction:

User capture

The sound source for User Capture is the artificial mouth of the HATS. The playback levels at MRP shall be calibrated to [-4.7 dB Pa].

Spatial capture

The sound source for spatial capture shall be [loudspeaker] positioned at 1 m relative to the horizontal center point between the individual UEs. Height/angle of the sound source shall be according to the individual UE type definitions.

EXAMPLE 1:	If the acoustic echo control is tested for the combination of Headset (RCV) and Handheld hands-free (SND), the setups for receiving testing specified in clause 5.4.2.3 is used for the Headset UE and the setup specified in clause 5.4.2.4 is used for the Handheld hands-free UE.

EXAMPLE 2:	If the sending loudness is tested for the combination of Headset (SND) and Table-mounted (SND) UE with simultaneous capture, the arrangement for receiving testing specified in clause 5.4.2.3 is used for the Headset UE and the arrangement specified in clause 5.4.2.5 is used for the Table-mounted UE. When testing Spatial capture loudness, a loudspeaker is positioned at 1m relative to the horizontal center point between Headset UE and Table-mounted UE at the height of 40cm. When testing User capture loudness, the artificial mouth of the HATS is used with the playback levels at MRP calibrated to -4.7 dB Pa.

]

## 2.2 Acoustic echo control testing

## 2.2.1 Overview

Terminal Coupling Loss (TCL) is a common measure for assessing echo attenuation/leakage perceived by the far-end user. While such echo loss measurement is typically applied for monaural audio, similar approach can be adjusted for testing echo attenuation/leakage with immersive capture and playback. Ideally, the far-end user should perceive as little echo as possible, despite the utilized capture and playback technology.

The overview on how the TCL based AEC testing could be performed with immersive audio is illustrated in Figure 3. A calibrated test signal representing far-end user is transmitted to the tested UE, and the attenuation of the returning signal containing possible echo is recorded at the reference client. Input and output signals are rendered to the same analysis format, and the echo level in analysis signal versus test signal is calculated. While echo doesn’t typically yield any immersive properties, it should be considered whether a mono-format based analysis is sufficient. For example, to assess any immersive issues in the recorded echo signal, stereo-format could be considered for the analysis. Nevertheless, rendering to a common analysis format should be performed with IVAS reference renderer.

Figure 3 Test setup for acoustic echo control

## 2.2.2 Test signal

In the Figure 3, the test signal block comprises calibrated, immersive input test signal in tested IVAS format. To ensure sufficient AEC performance with immersive audio playback/capture, the test signal should contain some level of immersion, e.g., simulated natural conversation comprising different talkers from different source direction. In Figure 4, a generalized virtual positioning pattern for simulated talker positions is visualised. The test signal is divided into 6 sentence long convergence and analysis segments. Each sentence within the segments is positioned to different directions to have comprehensive spatial sound scene. Alternatively, dynamic movement within the test signal sentences, or static directions per measurement could be considered. In addition, ambience or reverberation could be injected to the test signal to increase immersiveness.

Figure 4 Sentence directions in the test signal

Traditionally, a dynamically compressed version of the real speech single-talk sequence according to ITU-R P.501 [2] is applied for the echo loss measurements, to achieve high output level and highly uniform energy across the audio frequency band. In addition, by following the similar principles already defined in TS 26.260, the test signal can be calibrated to a certain loudness, to ensure sufficient output level. Typically, a slightly higher test signal levels are applied for echo measurements.

## 2.2.3 Test description

Based on the presented aspects, a test method description for assessing echo loss with immersive input and output is presented inside the brackets:

[

Test method

The assessment of UE echo loss is illustrated in Figure 3. The calculation of terminal coupling loss (TCL) is based on the attenuation between the rendered input signal and rendered output signal versus frequency bands. For the rendering step, the IVAS renderer [30] shall be used. The following common measurement steps are applicable for all types of UE modes:

Table 1 Source direction of each sentence in test signal

| Source direction per sentence |  |
| --- | --- |
| 1st & 7th | 0° |
| 2nd & 8th | 45° |
| 3rd & 9th | -45° |
| 4th & 10th | 90° |
| 5th & 11th | -90° |
| 6th & 12th | 180° |

- The source signal to be used for the measurements shall be [the compressed British-English single talk sequence described in clause 7.3.3 of Recommendation ITU-T P.501 [14].]

- The test signal sentences shall be [virtually positioned according to clause 5.5.2. The source directions of each sentence shall be according to the Table 1.]

- The input signal shall be calibrated to [-20 LFKS]

- The first 17,0s of the test signal (6 sentences) are discarded from the analysis to allow for convergence of the acoustic echo canceller. The analysis is performed over the remaining length of the test sequence (last 6 sentences).

- The renderer in the reference client shall be configured to [stereo] output.

- The analysis shall be conducted in 1/3-octave band intervals as given by the R.10 series of preferred numbers in ISO 3 [54] for each stereo audio channel. For the calculation, the averaged measured echo level at each frequency band is referred to the averaged, rendered test signal level measured in each frequency band.

- The TCL is calculated according to ITU-T Recommendation G.122 [8], annex B, clause B.4 (trapezoidal rule), but using the frequency range between [300] to [6700] Hz (instead of 300 Hz to 3400 Hz).

]

# Conclusion

In this document, simultaneous testing of UEs comprising multiple individual UE-types is discussed. Furthermore, important aspects on testing AEC with immersive playback and capture are presented.

During the first phase of ATIAS work, the permanent document approach for documenting proposed content for producing the output was utilized. While working with the permanent document had its own issues when, similar approach should be considered for tracking the proposed updates on target specifications. Alternatively, (draft) CR could be established for the same purpose. If either approach is agreed, the presented sections in the brackets are proposed to be documented for further integration into the TS 26.260.

# References

[1] 3GPP TS 26.260, “Objective test methodologies for the evaluation of immersive audio systems”, Release-18.

[2] ITU-T Recommendation P.501, “Test signals for use in telephony and other speech-based applications”, 06/2022.