Source:	Xiaomi

Title:	[FS_AMD] WT#13: Views on conclusions for clause 5.24 QUIC-based Media Delivery

Agenda Item:	8.6

Document for:	Agreement

- Introduction

In FS_AMD, the Work Topic #13 is examining the possible benefits of QUIC-based solutions for enhancing the delivery of segmented media content. In this contribution, we discuss the direction of conclusions for this Work Topic’s study.

- Status of the QUIC ecosystem

QUIC is a transport protocol and as such it cannot be used for media delivery as-is. Instead, an application requires an application-layer protocol built on top of QUIC for consuming segmented content. To make use of the QUIC protocol, the application relies on a specific QUIC implementation. It is a key difference compared with TCP and UDP implementations which were more heterogenous since implemented inside the UE’s OS kernel. For QUIC, the implementation is either provided by the deployment environment (e.g. web browser), the OS libraries on the UE or selected by the application developers among any of the existing QUIC third-party libraries. In certain environments where QUIC is not directly accessible, e.g. web browsers, the WebTransport protocol over HTTP/3 can be used to leverage the QUIC protocol and its features.

- Identified QUIC-based streaming technologies

Based on the input contribution (S4-241960 [1]), we have identified several technologies that can use or benefit from QUIC features for delivery.

The candidate technologies are merely listed for the purpose of inventorying. The presence of a candidate technology in this list does not bear any sense of endorsement or selection. As a matter of fact, this list is not guaranteed to be exhaustive and other possible candidate technologies leveraging QUIC for segmented content delivery could be missing at the time of writing.

For convenience, the candidate technologies as proposed in (S4-241960 [1]) are repeated below:

- MPEG-DASH Part 1 over HTTP/3 (with server push and priority)

- MPEG-DASH Part 6 over WebTransport

- Media over QUIC (MoQ)

- Push-based streaming over WebTransport

- Conclusion

Based on those candidate technologies and the overall capabilities of QUIC, the following conclusions can be drawn.

QUIC as a protocol is widely deployed and supported as demonstrated by clause 5.4 in TR 26.804 and its related CR prepared in FS_AMD. However, when it comes to using QUIC for media delivery, the early deployments of segmented streaming over QUIC were achieved by using the HTTP/3 protocol for MPD and segment requests. This approach is QUIC-agnostic and thus does not take full advantage of the capabilities offered by QUIC. In some cases, as reported by clause 5.4, there may even be some degradations of the performances compared to using HTTP 1.1. When it comes to non QUIC-agnostic approaches, there exist several candidate technologies ranging from early standardization efforts based on QUIC, previous standardization activities based on HTTP/2 and WebSocket that could be reused with QUIC and WebTransport, as well as research prototypes natively built on QUIC and WebTransport. As a result, it appears that there is thus no obvious path for enhancing segmented delivery by leveraging the emergence of the QUIC protocol.

At this stage, there is thus no basis justifying the need for any normative work in Release 19.

However, we believe that it would be worthwhile to study further the following aspects identified in the Work Topic:

- Fragmentation of QUIC implementations (both client and server side) and their performances for segmented media delivery

- Impact of reporting Qlog metrics via the downlink interface (M4) to the UE (bidirectional streams)

- The standardized definitions and their availabilities on the market of QUIC client and server APIs

- The standardized definitions and their availabilities on the market of WebTransport client and server APIs

- Evaluation of the identified candidate technologies when it comes to QoE metrics (start-up delay, stalling event, etc.)

- Conclusion

Based this content of this contribution, we propose the following:

- Identify the agreeable part of clause 4, possibly as a whole

- Start drafting initial conclusions based on the agreeable part of clause 4

References

- S4-241960, [FS_AMD] WT #13: Candidate technologies, Xiaomi, 3GPP TSG-SA WG4 Meeting #130, Orlando, US - 18th November 2024 - 22nd November 2024