3GPP SA4 #130	S4-242048

Orlando, US, November 18 - 22, 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 1.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] Edits of Solution #4 and update on the signaling mechanisms for PDU Set Size correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 11/18/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | One aspect of Key issue #1: Inaccuracy of the PDU Set Size (PSSize) information Comments were made during the SA4 RTC Ad Hoc Meeting on Oct 23, 2024, on the signaling mechanisms other than SDP. There were 2 Editor’s notes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added alternative signaling mechanisms. Addressed the 2 Editor’s notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution space will not be comprehensive, unaddressed editor’s notes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

.

# * * * * 1st change (add a reference) * * * *

[x]		3GPP TS 26.113: "Real-Time Media Communication; Protocols and APIs”.

# * * * * End of 1st change * * * *

# * * * * 2nd change * * * *

## 6.4	Solution #4: Measurement Based Pre-compensation for PDU Set Size Correction

### 6.4.1	Key Issue mapping

This solution maps to Key issue #1.

### 6.4.2	Description

As discussed in 5.1.1, there are multiple reasons for causing the PSSize to be inaccurate. Although the impact of those reasons on the size of a single IP packet seems insignificant, a PDU Set may consist of many IP packets and the aggregate impact can still be significant. If the PSSize for which gNB schedules is less than the actual PSSize, when the last packets arrive it may take gNB one or more slots to schedule them, therefore delaying the delivery of the PDU Set, which is detrimental to low-latency applications such as XR applications. The opposite can also happen, which wastes resources.

Therefore, we have the following observation:

Observation 1: it is important to make the PSSize accurate for low-latency applications.

There are efforts to correct the impact of NAT46/64 on the PSSize [11]. However, these efforts are not able to tackle other causes. Moreover, the list of causes in the introduction clause is not complete – even if we list all possible causes today, novel network protocols that change the PSSize are likely to be deployed in the future. We have the following observation:

Observation 2: A generic solution for correcting the PSSize is preferred.

The UPF is the gateway to the 5G core. A UPF may handle a very large amount of traffic. In fact, some network operators have very few UPFs. Therefore, any solution that requires UPF to take action, such as addition, subtraction and multiplication, is undesirable. We have the following observation:

Observation 3: To reduce the UPF complexity, it is preferred not to require UPF to correct the PSSize.

# * * * * End of 2nd change * * * *

# * * * * 3rd change * * * *

From the results, we observe that:

Observation 5: The proposed pre-compensation based PSSize correction method effectively reduces the PSSize error.

For the ‘NAT46/64 only correction’ method to work, the sender needs to be aware of the presence of NAT46/64. The awareness may be obtained by feedback from the receive on the type of the IP address type seen by the receiver. The feedback may be one time during a session, and can be done by SDP signaling,

Simple WebRTC Application Protocol (SWAP) in TS 26.113 [x], or RTCP.

# * * * * End of 3rd change * * * *