3GPP TSG-SA5 Meeting #158 	S5-246360

Orlando, USA, 18 – 22 November 2024

# Source:	Ericsson, Vodafone, Deutsche Telekom, Telecom Italia

# Title:	Signalling traffic monitoring management NRM stage 3 of the drafted TS28.abc

# Document for:	Approval

Agenda Item:	6.19.22

# 1	Decision/action requested

For approval.

# 2	References

[1]	S5-245981, new WID signalling monitoring

[2]	S5-245336, initial skeleton of draft TS28.abc signalling monitoring

# 3	Rationale

Defines Signalling traffic monitoring management NRM stage 3

# 4	Detailed proposal

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1434 at commit 3f215c739d1d635cac6b232ef232057b97f21e56

*** START OF NEXT CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x5]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[x9]	Management and Orchestration APIs Stage 3 Repository https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA103/

*** START OF NEXT CHANGE ***

## 7.2	YANG Definitions

The present clause contains the YANG definitions for the STM NRM.

The Information Service (IS) of the STM NRM is defined in subclause 6.2.

Mapping rules to produce the YANG definition based on the IS are defined in 3GPP TS 32.160 [x5].

YANG definitions are specified in 3GPP Forge [x9].

Directory: yang-models

Files: _3gpp-5gc-nrm-stmfunction.yang

*** START OF NEXT CHANGE ***

*** yang-models/_3gpp-5gc-nrm-stmfunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-stmfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-stmfunction;

prefix stm3gpp;

import _3gpp-common-top { prefix top3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the STM function defined in 3GPP TS 28.abc.

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.abc

Signalling traffic monitoring management";

revision 2024-11-07 { reference "S5-246360"; }

grouping StmReferenceGrp{

description "This <<dataType>> defines a globally unique identifier, which

uniquely identifies the StmReportingCtrl that is created by STM consumer.

It composes with a consumer identifier followed by a STM identifier.

The consumer identifier is one-byte octet string, and it is assigned by

the operator. The STM identifier is two-byte octet string, and it is

generated by the consumer.";

leaf consumerId {

type string;

mandatory true;

description "A one-byte octet string identifier";

}

leaf stmId {

type string;

mandatory true;

description "A two-byte octet string identifier";

}

}

grouping STMFunctionGrp {

leaf stmTargetUri {

type inet:uri;

description "It specifies the Uniform Resource Identifier (URI) of the

streaming target where the signalling traffic shall be sent. ";

reference "Clause 4.4 of 3GPP TS 32.158";

}

list stmRef {

key "idx";

min-elements 1;

max-elements 1;

description "A globally unique identifier, which uniquely identifies the

StmReportingCtrl.";

leaf idx { type uint32 ; }

uses StmReferenceGrp ;

}

leaf-list networkInterfaceType {

type enumeration {

enum ALL;

enum N4;

enum N5;

enum N7;

enum N8;

enum N10;

enum N11;

enum N12;

enum N13;

enum N14;

enum N15;

enum N22;

enum N58;

enum N59;

enum N80;

enum N81;

}

description "List of network interface type. The value ALL is specified

for the case if all the applicable interface type of the network

function shall be monitored. ";

reference "Clause 4.2.3 of 3GPP TS 23.501 for details on the

allowed values.";

}

leaf-list reportingNF {

type types3gpp:DistinguishedName;

description "List of Network Function Distinguished Name";

}

leaf administrativeState {

default UNLOCKED;

type types3gpp:BasicAdministrativeState ;

description "Enable or disables production of the metrics";

}

leaf operationalState {

config false;

mandatory true;

type types3gpp:OperationalState ;

description "Indicates whether the PerfMetricJob is working.";

}

}

grouping STMFunctionSubTreee {

description "Contains classes that manage Tracing.

Should be used in all  classes (or classes inheriting from)

- SubNnetwork

- ManagedElement

- ManagedFunction

If a YANG module wants to augment these classes/list/groupings they must

augment all user classes!";

list StmReportingCtrl  {

description "This IOC represents the STM Control and Configuration

parameters of a particular STM reporting controlling. It can be

name-contained by SubNetwork, ManagedElement, or ManagedFunction.

stmRef is a globally unique identifier, which uniquely identifies

a StmReportingCtrl object instance.

reportingNFList is the parameter on which network function the

signalling traffic shall be monitored. This parameter is defined by

the STM consumer. This parameter may be omitted if the STM reporting

control object is specified under a ManagedFunction.

networkInterfaceTypeList is the parameter on which network interface

type shall be monitored. This parameter is defined by the STM consumer.

streamTargetUri is the URI of the STM consumer to which the STM report

shall be streamed.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses STMFunctionGrp ;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE ***