3GPP TSG-SA5 Meeting #158	S5-246470

Orlando, USA, 18 - 22 November 2024

# Source:	Nokia

# Title:	Discussion on external management data

# Document for:	Endorsement

Agenda Item:	6.19.10 Data management phase 2

# 1	Decision/action requested

The group is asked to discuss and endorse detailed proposals.

# 2	References

[1]	3GPP TS 28.537 Management capabilities

[2]		S5-246298 Draft CR 28.537 for MADCOL_Ph2, 3GPP SA5#157, Hyderabad, India, October 2024

[3]	3GPP 28.533 Architecture framework

[4]	3GPP TS 28.622 Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

[5]	3GPP 32.156 Fixed Mobile Convergence (FMC) model repertoire

[6]		S5-246471 Input to Draft CR 28.622 External Management Data, 3GPP SA5#158, Orlando, USA, November 2024

# 3	Rationale

External management data has been discussed since Release 17. The agreements achieved so far are summarized in clause 6.4 of TS 28.537 [1][2] comprising the description and requirements of external management data.

So far there is no conclusion on how to include external management data in the 3GPP management system such that it can be used. This aspect is discussed in detail in the following chapter.

# 4	Discussion

According to TS 28.533 [3] management information MnS component type C, i.e. „internal 3GPP“ management data, is associated to managed entities that are described by MnS component type B, i.e. the network resource model, and are exchanged by using the operations described by MnS component type A, see Figure 1.

Figure 1: Management Service Components

In other words, „internal 3GPP“ management data is associated to object instances of information object classes (IOC) that are defined in the network resource model. E.g. each counter (even if vendor-specific) has an associated measured object instance, and each alarm has an associated alarming object instance where the IOC are known in the NRM.

In contrast, external management data does not have such well-defined association to object instances. Nevertheless, the external data obviously has some relationship to the network because otherwise it would not make sense to handle external data in the context of network management at all. Such relationship depends on the implementation and the details of the external data.

Proposal 1: 	External management data needs to have a relationship to the MOIs of the NRM.

Potential data which could be treated as external management data according to the description in TS 28.537[1] are in our understanding:

- Pictures, or videos

- Measurements which are not specified by 3GPP or are not reported by 3GPP specified PM framework e.g. brightness/lumen of sun

- Analytics and/or predictions of various parameters e.g.

  - weather forecasts,

  - traffic forecasts,

  - amount of data to be transmitted in a certain area.

  - UE paths

- Schedule for planned events e.g. concerts, sports events

In the following, it is discussed how external management data could be related to MOIs of the NRM.

Figure 2: Relationship of external management data

External management data, e.g. a weather forecast or a picture, might relate to many network entities and vice versa one network entity might relate to many weather forecasts and pictures. This results in a potential many to many relationship between external management data and instances of managed objects.

Proposal 2: 	The same external management data might be associated to many managed object instances, the same manage object instance might be associated to many different external management data.

Due to the restriction of name containment that “a target instance can not participate in or belong to multiple namespaces” [5] and the “m x n relationship” of external management data, it is obvious that "SubNetwork" IOC is not suitable for associating managed object instances according to external data or vice versa.

Observation 1: 	It is not possible to use "SubNetwork" IOC to group instances of managed object per external data.

Observation 2: 	It is not possible to use "SubNetwork" IOC to group external data per managed object instance.

Considering the list of potential external management data in the context of management system, it seems obvious that external management data is related to a geographic area or location especially since the source of external management data is not aware of the DNs used by the management system to address managed objects.

Proposal 3: 	Notion of geographic area or location plays a key role for external management data.

If data is related to a geographic area, it is related to a set of managed objects, NFs, or NEs. The set of managed objects, NFs, or NEs might change over time e.g. by adapting transmission power or beam direction of antenna (elements), assignment policy etc.

Observation 3: 	The set of managed objects, NFs, or NEs related to the external management data may change over time.

The definition of "ManagedElement", see TS 28.622 [4], states the following:

"A ManagedElement IOC is used to represent a Network Element defined in TS 32.101[1] including virtualization or non-virtualization scenario. […] A ManagedElement contains equipment that may or may not be geographically distributed."

This means that "ManagedElement" MOI is not necessarily related to a single location or area but might present several distributed locations, namely the location(s) or area(s) of the equipment which may be geographically distributed due to virtualization etc. Therefore, it is not suitable to associate with external management data which is related to a certain (closed) geographical area or location.

Observation 4: 	Location is not a good property of "ManagedElement" or other logical functions.

To fulfil the demand of location relationship and to allow the “m x n relationship”, we propose to introduce an association class which may provide associations to the external management data itself and the related network elements or the MOIs of the NRM. Figure 3 visualizes one example implementation of our proposal.

Figure 3: Association class “ExternalData”

Some attributes like "mediaType" or "reportingMethods" are used to describe the meta data of the external management data.

On the one hand, the attribute "geoArea" may provide an implicit association between the geographic area and the external management data element. Internal in the system will be a mapping to the corresponding MOIs of the NRM. One example for this provides MOI "Ext-1" of class "ExternalData" in Figure 3 with value "Munich" for attribute "geoArea".

On the other hand, the attribute "objectInstances" may provide an explicit association between the external management data element and the related MOIs of the NRM. As in Figure 3 MOIs "cell-1", "cell-2" and "cell-3" of the IOC "CellDU" for external management data MOI "Ext-2".

Proposal 4: 	Introduce an IOC allowing implicit and/or explicit association between external management data and related MOIs of the NRM.

The name containment of the proposed IOC needs to be discussed further. In Figure 3 the example MOIs are name contained below "NMS-1" of IOC "SubNetwork". Another potential solution could be that "ExternalData" is name contained by "MnSAgent" which itself is name contained by "SubNetwork".

Potential stage 2 implementation of the proposed IOC is provided in S5-246471 [6].

One advantage of the proposed solution of association class is that the external management data can be associated to the management system without providing the relationship to network elements in the first step. In other words, there might be a MnF which collects all external management data and associates each of them to the same MOI of type "SubNetwork". In a second step associations to the MOIs of the network elements might be added for each external management data element e.g. by a second MnF.

# 5	Detailed proposal

Proposal 1: 	External management data needs to have a relationship to the MOIs of the NRM.

Proposal 2: 	The same management data might be associated to many external management data. The same external management data might be associated to many management data.

Proposal 3: 	Notion of geographic area or location plays a key role for external management data.

Proposal 4: 	Introduce an IOC allowing implicit and/or explicit association between external management data and related MOIs of the NRM.