# 3GPP TSG SA WG5 Meeting #158	S5-246578

# Orlando, Florida, USA 18 - 22 November 2024

# Source:	Huawei

# Title:	New charging solution for DC application usage by duration

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to approve the follow update in TR 28.851.

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR proposes new solution for topic 2 on IMS DC download and usage charging in TR 28.851 [1].

The solution propose to charge the application usage by the duration of the application DC.

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851.

First change

5.2.4.x	Solution #x: DC application usage charging by duration

5.2.4.x.1	General

This solution #x resolves the key issues #1a and #1b for REQ_NG_RTC_CH-DCAPP-02. The IMS charging specified in TS 32.260 [11] can support the duration based charging for DC application usage.

5.2.4.x.2	Description

The application DC may be established and terminated via SIP Re-INVITE message during an IMS DC session. In this case, the IMS charging can support the duration based charging for application DC, with the following details:

-	Triggers: IMS trigger conditions in TS 32.260 [11] Table 5.4.3.1 may apply. For example, the application DC setup may be triggered by a SIP Re-INVITE message for adding application DC. The application DC release may be triggered by end of the call via SIP BYE message, or SDP negotiation via another SIP Re-INVITE message to release the application data channel.

-	Charging Information: IMS charging information in TS 32.260 [11] Table 6.4.2.2.1 may apply. For example, the “SDP Media Component” may hold the application binding information via the "a=3gpp-req-app" attribute when media component is DC. The application binding information contains: the application identification via the "req-app-id" parameter, and the endpoints for the application DC via "app-dc-info" parameter, according to TS 26.114 [8] clause 6.2.13.

End of changes