3GPP TSG-SA5 Meeting #158	S5-246718

Orlando, USA, 18 - 22 November 2024

# Source:	Huawei

# Title:	Discussion paper on potential normative work for SA5 NDT

# Document for:	Endorsement

Agenda Item:	6.19.5

1	Decision/action requested

This contribution is for discussion and endorsement.

2	References

[1]	3GPP TS 28.915

3	Rationale

3.1 			Background

A number of agreed use cases and potential solutions are described in TR 28.915. The implementations of NDT can be very different regarding the potential solutions of NDT in SA5. However, the conclusion is also to develop the new proposed management service to support above scenarios by using the NDT.

This contribution is to provide Huawei opinion on Rel-19 NDT normative work tasks.

3.2 		Overview of studied use cases

Use cases where NDT may provide support include (TR 28.915):

-	Verification:

-	RAN energy saving policy verification (see clause 5.1).

-	Signalling storm configuration verification (see clause 5.2).

-	Emergency preparedness (see clause 5.3).

-	Configuration verification (see clause 5.9).

-	Visualization:

-	Network topology and traffic visualization (see clause 5.8).

-	Prediction:

-	Network failure and risk prediction (see clause 5.4).

-	Simulated data generation:

-	ML model training data generation (see clause 5.6).

New added use cases:

-	Network issue inducement

-	Measuring customer satisfaction with the network services

3.3 		Concept of NDT

Role of NDT in studied UCs is focusing verification which may be based on simulation or not in TR 28.915. When NDT role is to verify the management operations or any changes to the network, the NDT takes a role of a new capability for management scenarios. The management scenarios can be the verification or simulation tasks related to network planning and deployment (including the changes to the network). On the other hand, the NDT may provide an NDT models and reasonably detail to support further enhancement to automation functions. When NDT provide enabling data to automation function, those functions (e.g., MDA) may produce analytics based on those input data provided by an NDT instance. Those analytics, on the other hand, can be used for O&M purpose, for example, an evaluation of network and provide recommendation actions.

Proposal 1: It’s proposed to capture the NDT related concept in Rel-19 normative work.

3.4		Lifecycle management of NDT

Lifecycle management may support the creation, configuration, activation, reporting, de-activation and termination of a NDT instance.

An NDT instance or simulated network is created for a management scenario of NDT (e.g., evaluation of a signalling storm configuration), as described in clause 3.2. During or after creation of NDT, the scope can be set by NDT MnS producer. The possible scope of NDT instance are described as following: subnetworks, duration, geography location, simulation scenario and network functions.

Proposal 2: It’s proposed to capture the NDT LCM in Rel-19 normative work.

3.5		Define an NDT

As an example of signalling storms simulation scenario, an NRM IOC for simulated network or NDT instance that can provide the attributes based on NDT scenarios in 3GPP management system.

Generic part of NDT: it’s generic information which applicable for all NDT scenarios.

- The MOI information related to simulated network or NDT instance

- The scope of NDT (e.g., RAN, core network NFs, deployment location, duration etc.)

- The NDT related management data:

  - The fault information (e.g., NF fault status) related to NDT instance

  - The performance information (e.g., service KPIs) related to the simulated network or NDT

Scenario specific part of NDT: it’s specific information which differentiates according to different NDT scenarios.

The use of NDT model can support visualization of simulated network or NDT instance or provide the enabling data for other automation functions (e.g., MDAF).

Proposal 3: It’s proposed to capture the NDT related management models including combination of generic management framework with scenario specific information in Rel-19 normative work.

3.6		Discussion on relations and support to automation function in SA5

Figure 4.1.2: NDT MnS example related to automation function

The example related to automation function is shown in Figure 4.1.2. NDT MnS producer provide NDT MnS, which may be invoked by MDA.

Proposal 4: It’s proposed to capture the relation between NDT and other automation functions in Rel-19 normative work.

4	Detailed proposal

It’s proposed to keep the following topics in Rel-19 normative work:

Proposal 1: It’s proposed to capture the NDT related concept in Rel-19 normative work.

Proposal 2: It’s proposed to capture the NDT LCM in Rel-19 normative work.

Proposal 3: It’s proposed to capture the NDT related management models including combination of generic management framework and scenario specific modelling information in Rel-19 normative work.

Proposal 4: It’s proposed to capture the relation between NDT and other automation functions in Rel-19 normative work.