3GPP TSG-SA5 Meeting #158	S5-246728

Orlando, USA, 18 - 22 November 2024

# Source:	ZTE Corporation

# Title:	R19 pCR TR 28.858 Add Potential Solution and Evaluation of Managing ML Model Delivery

# Document for:	Approval

Agenda Item:	6.19.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.858: " Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2"

3	Rationale

Add Potential Solution and Evaluation of Managing ML Model Transfer and ML Model Delivery.

4	Detailed proposal

5.4.2 	Managing ML Model Transfer/delivery

5.4.2.1 	Description

The ML Model transfer/delivery (clause 7.2.1.4) in TR 38.843 [5]   addresses number of potential solutions for model delivery/transfer to the UE, including the following solution:

“Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.”

According to RAN, AI/ML model transfer refers to the delivery of an AI/ML model over the air interface while AI/ML model delivery refers to delivery of an AI/ML model from one entity to another.

NOTE: the specific details of the solution 4b identified by RAN as documented in TR 38.843 [5] as one of the potential solutions for model delivery to UE is yet to be investigated by SA5.

5.4.2.2	Use cases

5.4.2.2.1	Relation of ML model Delivery in RAN to ML model loading in SA5

AI/ML model delivery corresponds to the “OAM→RAN→UE scenario” in TR 38.843. As the interaction over air interface between gNB and UE is outside the scope of SA5, what is referred to as “delivery” in RAN corresponds to ML Model Loading in SA5 for which the existing solutions for ML model loading in TS 28.105 could be reused for the model transfer process between OAM and the gNB.

5.4.2.3	Potential Requirements

The requirements documented in TS 28.105 [2], clause 6.4.1.3 are applicable for AI/ML model delivery to gNB. Further clarifications into clause 6.4.1.3 should be further discussed and decided during the normative phase.

5.4.2.4	Possible solutions

Clarification on the relation between ML Model Delivery and ML Model Loading is needed. Add the following sentences to elaborate the relation of between ML Model Delivery in RAN and ML Model Loading in clause 6.4.1 of TS 28.105.

“ML Model Delivery in RAN refers to the delivery of an AI/ML model from one entity to another entity. Interaction over air interface in not the scope of SA5 and what is called ML model delivery in RAN corresponds to ML Model Loading in SA5.”

5.4.2.5	Evaluation

The solution described in clause 5.4.2.4 proposes to clarify the relation between ML Model Delivery and ML Model Loading. The introduced solution aligns with the current standards, which is feasible to be developed further in the normative specifications.