3GPP TSG-SA5 Meeting #158	S5-246855

Orlando, USA, 18 - 22 November 2024

# Source: Ericsson India Private Limited (TSDSI)

# Title: PCR TR 28.858 DP Model Training or multiple Contexts

# Document for: Endorsement

Agenda Item: FS_AIML_MGT_Ph2

# 1	Decision/action requested

endorsement

# 2	References

[1] 		3GPP TR 28.858 “Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2”.

[2]	      3GPP TS 28.105: Artificial Intelligence / Machine Learning (AI/ML) management".

# 3	Rationale

The SA5 specification allows for defining an MLModelCoordinationGroup as specified in TS 28.105 [2], clause 7.2a.2.3, representing a group of ML models that can be jointly trained, tested, and used to perform inference in a coordinated manner. It is also specified that one ML model may depend on one or more other ML models within the same group.

In TR 28.858 [1], clause 5.1.4.2, a use case for “ML model training for multiple contexts” has been agreed upon, with the following requirement:

REQ-ML_CLUSTER-TRAIN-1: The MLT MnS producer should have a capability for an authorized MnS consumer to request training of a cluster of ML models associated to a set of multiple contexts from a previously trained ML model.

A potential solution to address this requirement is described in clause 5.1.4.4:

#### 5.1.4.4	Possible solutions

The  MLTrainingRequest IOC has to be extended with:

- a flag indicating that a cluster of ML models has to be trained.

- a clustering criteria indicating which ML models can be formed the cluster, This may indicate context similarities, e.g., similar geographical location, training data similarities etc. Take traffic congestion analysis as example, two ML model instances for one specific inference type during peak and off-peak hours would have significant geographical similarity in their contexts, these two models can be placed in one cluster.the DN of the ML instance to be used as the baseline to train another ML model with a related context.

- the expectedRunTimeContext can be used to capture the context of new ML models that should be trained.

## Potential issues#1

The requirement REQ-ML_CLUSTER-TRAIN-1 addresses both clustering and training for multiple contexts, but it’s unclear how these two aspects are meant to interact or if they are separate features. Clarifying whether clustering is merely a means to optimize multi-context training or if it serves a different purpose.

## Potential issues#2

The proposed solution in clause 5.1.4.4 suggests enhancements to the MLTrainingRequest IOC to support clustering; however, this functionality may already be achievable through the MLModelCoordinationGroup defined in 28.105 [2].

## Potential issues#3

The solution in clause 5.1.4.4 proposes modifying MLTrainingRequest IOC for clustering. However, similar functionality may already be available through MLModelCoordinationGroup as defined in 28.105[2].

## Conclusion

The above issue needs to be confirmed by SA5.

The solution proposed in TS 28.585 [1] clause 5.1.4.4 can be realized with already specified “ML model joint training” in TS 28.105[2] clause 6.2b.2.6. For the use of a base model to train the MLModelCoordinationGroup solution ,  already specified in the NRM definition for  MLTrainingRequest in TS 28105[2] clause 7.3a.1.2.2, were the use of mLModelRef or mLModelCoordinationGroupRef attributes allows a base model to be used when requesting training.

Considering that ML model algorithm is proprietary and the consumer in fact does not have knowledge about all data that are used for training, the consumer does not have insight on how the producer should handle clustering criteria. Also the consumer cannot decide how training should be performed by the producer.

# 4	Detailed proposal

The group is asked to acknowledge this issue and endorse the proposed solution.

If endorsed, PCRs will be submitted to SA5#158 to :

- update evaluation of use case “ML model training for multiple contexts” [1] Clause 5.1.4 with information that clustering as well as multiple contexts for a group of ML models is already specified in the context of SA5 as MLModelCoordinationGroup [1].