# 3GPP TSG SA WG5 Meeting #158	S5-246995

# Orlando, Florida, USA 18 - 22 November 2024

# Source:	Huawei

# Title:	Evaluation for IMS DC charging topic 2

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to approve the following update in TR 28.851 [1].

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR provides evaluation for topic 2 on IMS DC download and usage charging in TR 28.851 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851 [1].

First change

5.2.5	Evaluation

Solution #1 proposed to charge based on the DC application download. This requires DCSF as a new CTF to collect and report DC application download information to CHF.

Solution #x proposed to charge based on the DC application usage duration. The charging information can reuse the existing IE in IMS charging. The charging principle may be extended to describe the support for duration based charging per DC application.

Solution #y proposed to charge based on the data volume consumed during DC application usage on per IMS DC session basis. The SMF charging can already support this soluion. However, this solution does not distinguish the volume consumed for each application, when the IMS DC session involved more than one application DC.

End of changes