3GPP TSG SA WG5 Meeting #157 	S5-245887

- Hyderabad, India, 14 - 18 October 2024

- Source:	Nokia

- Title:	pCR TR 28.849 Multiple API Providers Solution

- Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

This is a pCR to TR 28.849 introducing a new Solution to Topic#2.

# 2	References

[1]	3GPP TR 28.849 "Study on charging aspects for Common API Framework for Northbound APIs (CAPIF) phase2".

# 3	Rationale

This pCR proposes to introduce a solution for Topic #2 in TR 28.849 [1].

# 4	Detailed proposal

Propose to include the following change into the TR 28.849 [1].

32.290 [4]

First change

#### 6.2.5.x	Solution #2.x Multiple API Provider Service Management Charging

A possible solution for key issue #1.1 covers the requirement REQ-CH_ CAPIF_NEF-01.

The focus on this solution to provide the capability to charge te following service API in case required from multiple Service Providers:

- service API publish, as specified in the TS 23.222 [2] clause 8.3

- service API unpublish, as specified in the TS 23.222 [2] clause 8.4

- service API retrieve, as specified in the TS 23.222 [2] clause 8.5

- service API update, as specified in the TS 23.222 [2] clause 8.6

The following deployment model already depicted in 3GPP TS23.222 [2] clause B.3.2.2  provides a view on the possibility on having multiple API providers

Figure 6.2.5.X-Y - Integrated Deployment of the SCEF and the NEF with CAPIF

The solution is similar to the one already detailed in clause 6.2.5.x. Therefore, the following details can be considered to be on top of what is proposed in the solution. The main point is to consider the possibility to charge muitple API Providers, which could be charged in a different way, and in order not to over complicate a solution, the availability of an ‘Tenant Identifier’ would be the reference of the right API Provider reason its included in the Charging Data Request Message and CDR, which is already available according to TS32.290 [4], clause 6.2a.

End of changes