3GPP TSG-SA5 Meeting #157	S5-246084

Hyderabad, India, 14 - 18 October 2024

- Source:	Nokia

- Title:	pCR 28.572 Add requirements for activation

- Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

# 5	Requirements

General plan activation requirements

Req-PAG-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to activate a planned configuration.

Req-PAG-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of a plan activation.

Req-PAG-3: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to cancel an ongoing plan activation.

Req-PAG-4: The 3GPP management system shall support a capability allowing a MnS consumer to retrieve the result of a plan activation.

Req-PAG-5: The 3GPP management system should allow for multiple sequential activations of the same plan.

Req-PAG-6: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously activated.

Req-PAG-7: The 3GPP management system shall validate a planned configuration automatically when activation of the planned configuration is requested without prior validation.

Requirements related to control parameters for activation

Req-PAC-1: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if activation should be done with the least possible service impact or within the shortest possible time.

Req-PAC-2: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if fallback shall be enabled.

Requirements related to the activation of multiple plans

Req-PAM-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to activate multiple planned configurations with a single request.

Req-PAM-2: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the type of relationship between the plans, for the case where multiple plans are requested to be activated with a single request.

Req-PAM-3: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the behavior if plan conflicts are detected, for the case where multiple plans are requested to be activated with a single request.

End of modifications