3GPP TSG-SA5 Meeting #158	S5-247117

Orlando, USA, 18 - 22 November 2024	(rev of S5-246482)

# Source:	NTT DOCOMO, Rakuten Mobile, Huawei, ZTE Corporation

# Title:	pCR 28.880 Use case #2 updates and evaluation

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

For approval

# 2	References

- 3GPP TR 28.880 v1.1.0: "Study on energy efficiency and energy saving aspects of 5G networks and services (Release 19)".

# 3	Rationale

The present contribution proposed addressing open editor’s notes and improve the readability of some descriptions in TR 28.880 [1].

# 4	Detailed proposal

It proposes to make the following changes to TR 28.880 [1].

1st Change

## 5.2	Use case #2: Alternative option to obtain energy consumption of VNF/VNFC

### 5.2.1	Description

For Energy Consumption (EC) of VNF/VNFCs based on VMs, estimated measurement is supported as defined in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2]. The way to calculate the estimated VM-based VNF/VNFC energy consumption requires obtaining the sum of the vCPU mean usage of all virtual compute resource instances running on the same NFVI Node during the same observation period, all separately provided by NFV-MANO (see clause 7.1.2 of ETSI GS NFV-IFA 027 [6]).

The issue is that there is no easy way to obtain all virtual compute resource instances information running on the same NFVI Node and measure the vCPU, vMemory or vDisk mean usage for all of them. The measurement method as described in clause 7.1.2 of ETSI GS NFV-IFA 027 [6] does not support setting the measurement scope on the same NFVI Node. Some other mechanism to know all the virtual compute resource instances information running on the same NFVI Node is required.

In addition, the mechanisms for obtaining energy consumption (EC) of VNF/VNFC in practical implementations are more complicated than the estimation method currently defined in in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2], which is simply based on vCPU, vMemory, vDisk mean usage or I/O traffic volume. Diverse power measurement models and tools for obtaining the EC are possible depending on the operator deployment scenarios.

Alternative options for obtaining energy consumption of VNF/VNFC in addition to the existing estimation methods defined in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2] are needed.

As described in clause 7.1.18 and clause 7.4.12 of ETSI GS NFV-IFA 027 [6], the actual energy consumption of virtual compute and OS container workload is provided. As an enhancement, this use case is to study:

a)	potential enhancements to existing EC of VM-based VNF/VNFCs estimation method; and

b)	potential extension to EC estimation of containerized VNF/VNFCs.

### 5.2.2		Potential Requirements

REQ-ENHANCEMENT-MEAS-01: The 3GPP management system should have a capability to generate KPIs about the energy consumption of an NF realized by VM-based VNF/VNFCs.

REQ-ENHANCEMENT-MEAS-02: The 3GPP management system should have a capability to generate KPIs about the energy consumption of an NF realized by OS container-based VNF/VNFCs.

### 5.2.3	Potential solutions

#### 5.2.3.1	Potential solution #1: Obtain energy consumption of a NF realized by VM or OS container based VNF/VNFC

##### 5.2.3.1.1	Introduction

In this potential solution, new alternative formula is proposed to calculate and generate KPIs about the energy consumption (EC) of a Network Function (NF). All the options listed only consider that 3GPP management system performs either some calculation based on other existing measurements available to the 3GPP management system, or that it performs the new KPI by means of object mapping.

##### 5.2.3.1.2	Description

The KPI for EC of a 5G Network Function (NF) is defined in clause 6.7.3.1.1 in 3GPP TS 28.554 [2], and it is considered that ECVNF (Energy Consumption of a VNF) is estimated based on vCPU, vMemory, vDisk mean usage ratio or I/O traffic volume.

According to ETSI GS NFV-IFA 027 V5.1.1 [6], energy consumption of a VM or an OS container workload can be also directly provided by the ETSI NFV's NFV-MANO system, which is computed by the NFV-MANO measurement producer based on energy consumption measurements received from NFVI or Container Infrastructure Services (CIS), and the exact method for estimating or measuring the energy consumption in the NFVI or CIS is not defined and left to implementation. It makes it flexible for vendors and operators to implement and/or deploy more advanced solutions to provide better energy consumption estimations. Therefore, it is proposed to modify the current definition of ECNF by using a generic definition of energy consumption of VNF enabling both the current (vCPU, vMemory, vDisk mean usage based or I/O traffic volume) and new (ETSI GS NFV-IFA 027 V5.2.1 [6] based) options to exist.

##### 5.2.3.1.3	Formula of ECNF

Same as currently defined in clauses 6.7.3.1.1 of 3GPP TS 28.554 [2], the formula of ECNF is described as below:

and ECVNF,estimated represents the total energy consumption of a VNF, which is obtained by summing up the energy consumption of its constituent Virtualized Network Function Components (VNFC). The unit of this KPI is J.

##### 5.2.3.1.4	Formula of estimated energy consumption of VNF

Same as currently defined in clauses 6.7.3.1.2 of 3GPP TS 28.554 [2], the formula of ECVNF, estimated is described as below:

and ECVNFC, estimated represents the energy consumption of a VNFC.

When VNFC is realized with a virtual machine, ECVNFC, estimated is obtained by the following three options:

a)	By the existing mechanism which estimates the energy consumption of the VNFC based on its resource usage metric provided by ETSI NFV’s NFV-MANO, which is either:

-	mean vCPU usage, as per clause 6.7.3.1.4 of 3GPP TS 28.554 [2]; or

-	mean vMemory usage as per clause 6.7.3.1.5 of 3GPP TS 28.554 [2]; or

-	mean vDisk usage as per clause 6.7.3.1.6 of 3GPP TS 28.554 [2]; or

-	I/O traffic volume as per clause 6.7.3.1.7 of 3GPP TS 28.554 [2].

b)	Or by collecting the total energy consumption of the virtual resource on which VNFC runs over the collection period provided by ETSI NFV’s NFV-MANO (see clause 7.1.18 of ETSI GS NFV-IFA 027 [5] and [6]) which equals to the energy consumption of VNFC.

c)	Or by collecting the total energy consumption of the VNFC over the collection period provided by ETSI NFV’s NFV-MANO (see clause 7.2.16 of ETSI GS NFV-IFA 027 [6]).

When VNFC is realized with OS containers, ECVNFC, estimated is obtained by the following three options:

a)	By estimating the energy consumption of the VNFC based on the resource usage metric provided by ETSI NFV’s NFV-MANO which is either:

-	mean vCPU usage as per clause 7.4.2 of ETSI GS NFV-IFA 027 [6], or

-	mean memory usage as per clause 7.4.4 of ETSI GS NFV-IFA 027 [6], or

-	mean usage of storage resource as per clause 7.4.10 of ETSI GS NFV-IFA 027 [6], or

-	number of incoming and outgoing bytes as per clause 7.4.6 and 7.4.7 of ETSI GS NFV-IFA 027 [6].

Similar to clause 6.7.3.1.4, 6.7.3.1.5, 6.7.3.1.6 and 6.7.3.1.7 of 3GPP TS 28.554 [2], the energy consumption of the VNFC is estimated as a proportion of the energy consumption of the compute node on which the VNFC runs, which in the case of OS containers is referred to as "container infrastructure services (CIS) instance" as defined in ETSI GR NFV-IFA 029 [3]. This proportion is obtained by dividing the resource mean usage of the VNFC instance realized as an OS container workload, by the sum of the resource mean usage of all VNFC instances running on the same compute node. The formulas for computing this KPI are the same as specified in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2] for the ECVNFC, estimated, while using the corresponding resource usage metrics from ETSI GS NFV-IFA 027 [6] listed above. This option needs to consider the same pre-conditions documented in clause 6.3.2 of TS 28.310 [12]. More detailed formula is provided in clause 5.2.3.1.5.

b)	Or by collecting the energy consumption of the container workload realizes the VNFC over the collection period provided by ETSI NFV’s NFV-MANO (see clause 7.4.12 of ETSI GS NFV-IFA 027 [6]) which equals to the energy consumption of VNFC.

c)	Or by collecting the total energy consumption of the VNFC over the collection period provided by ETSI NFV’s NFV-MANO (see clause 7.2.16 of ETSI GS NFV-IFA 027 [6]).

ECVNF,estimated can also be collected, regardless of its composition based on VNFC realized by virtual machine or OS containers by collecting the total energy consumption of the VNF over the collection period provided by ETSI NFV’s NFV-MANO (see clause 7.2.18 of ETSI GS NFV-IFA 027 [6]).

##### 5.2.3.1.5	Formula of estimated energy consumption of containerized VNFC based on resource usage metrics

The detailed formula of the energy consumption of the containerized VNFC based on resource usage metrics is as follows. The estimated energy consumption of containerized VNFC is obtained by taking the estimated energy consumption of the OS container workload instance on which the VNFC runs.

ECcontainerized VNFC, estimated=ECOS container workload, estimated

The energy consumption of an OS container workload instance X is estimated as a proportion of the energy consumption of the compute node (i.e., a container infrastructure services (CIS) instance) on which the OS container workload instance X runs.

NOTE:	The following formulation assumes the case that OS container workloads are deployed on CIS instances that are bare-metal compute nodes, which can be mapped 1:1 to an NFVI Node.

This proportion can be obtained by considering the following four metrics:

-	mean vCPU usage as per clause 7.4.2 of ETSI GS NFV-IFA 027 [6].

The proportion is obtained by dividing the vCPU mean usage of the OS container workload instance X, by the sum of the vCPU mean usage of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, VCpuUsageMean=VCpuUsageMeanOS container workloadVCpuUsageMean*ECNFVINode, measured

where:

-	VCpuUsageMean is the mean virtual CPU usage measurement for the OS container workload instance during the observation period, provided by NFV-MANO (see clause 7.4.2 of ETSI GS NFV-IFA 027 [6]);

-	OS container workloadVCpuUsageMean is the sum of the mean virtual CPU usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV-MANO;

-	ECNFVINode, measured is the energy consumption of the NFVI Node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

-	mean vMemory usage as per clause 7.4.4 of ETSI GS NFV-IFA 027 [6].

The proportion is obtained by dividing the vMemory mean usage of the OS container workload instance X, by the sum of the vMemory mean usage of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, VMemoryUsageMean=VMemoryUsageMeanOS container workloadVMemoryUsageMean*ECNFVINode, measured

where:

-	VMemoryUsageMean is the mean virtual memory usage measurement for the OS container workload instance during the observation period, provided by NFV-MANO (see clause 7.4.4 of ETSI GS NFV-IFA 027 [6]);

-	OS container workloadVMemoryUsageMean is the sum of the mean virtual memory usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV-MANO;

-	ECNFVINode, measured is the energy consumption of the NFVI Node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

-	mean usage of vStorage Resource as per clause 7.4.10 of ETSI GS NFV-IFA 027 [6].

The proportion is obtained by dividing the vStorage Resource mean usage of the OS container workload instance X, by the sum of the vStorage Resource mean usage of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, VDiskUsageMean

=VStorageResourceUsageMeanOS container workloadVStorageResourceUsageMean*ECNFVINode, measured

where:

-	VStorageResourceUsageMean is the mean virtual storage resource usage measurement for the OS container workload instance during the observation period, provided by NFV-MANO (see clause 7.4.10 of ETSI GS NFV-IFA 027 [6]);

-	OS container workloadVStorageResourceUsageMean is the sum of the mean virtual storage resource usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV-MANO;

-	ECNFVINode, measured is the energy consumption of the NFVI Node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

-	number of incoming bytes and outgoing bytes as per clause 7.4.6 and 7.4.7 of ETSI GS NFV-IFA 027 [6].

The proportion is obtained by dividing the I/O traffic volume of the OS container workload instance X, by the sum of the I/O traffic volume of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, IOTrafficVolume=IOTrafficVolumeOS container workloadIOTrafficVolume*ECNFVINode, measured

where:

-	IOTrafficVolume is the sum of the incoming and outgoing traffic volumes of the OS container workload instance during the observation period, provided by NFV-MANO;

-	Incoming traffic volume is obtained by measuring the number of incoming bytes on OS container workload (VNetByteIncoming - cf. clause 7.4.6 of [6]) during the observation period;

-	Outgoing traffic volume is obtained by measuring the number of outgoing bytes on OS container  workload (VNetByteOutgoing - cf. clause 7.4.7 of [6]) during the observation period.

IOTrafficVolume=VNetByteIncoming+VNetByteOutcoming

-	OS container workloadIOTrafficVolume is the sum of the  incoming and outgoing traffic volumes of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO (see clause 7.4.6 and 7.4.7 of [6]);

-	ECNFVINode, measured is the energy consumption of the NFVI Node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

2nd Change

### 5.2.4	Evaluation of potential solutions

The solution described in clause 5.2.3.1 provides the possibility of 3GPP management system to generate KPI about the energy consumption of an NF realized by VM-based and/or OS container-based VNF/VNFC either based on deriving the KPI from energy consumption associated to VNF/VNFC and/or their virtualised compute/OS container workload resources (as provided by ETSI NFV’s NFV-MANO), or by deriving the KPI based on other resource related metrics like CPU, memory, disk/storage resource or traffic volume. Thus, the solution makes diverse power measurement models and tools for obtaining the EC possible to meet specific operator deployment scenarios and requirements. Furthermore, the option of deriving the KPI based on other resource related metrics like CPU, memory, disk/storage resource or traffic volume and the energy consumption from NFVI Nodes has dependency on the availability of such NFVI Node energy consumption to the 3GPP management system, as also documented in clause 6.3.2 of TS 28.310 [12].

Among the different alternatives describe, those that are based on collecting energy consumption associated to VNF/VNFC and/or their virtualised compute/OS container workload resources (refer to options b and c in respective VM/OS container-based cases) are likely to be more accurate since the sourced energy consumption information is associated at collection and reporting level to virtualised compute/OS container resources by mechanisms available in the infrastructure and its management and orchestration platforms.

The potential solution #1 fulfils the requirements REQ-ENHANCEMENT-MEAS-01 and REQ-ENHANCEMENT-MEAS-02. This is a feasible candidate as input to the normative phase.

3rd Change

## 6.2	Alternative option to obtain energy consumption of VNF/VNFC

The use case, potential requirements and potential solutions for alternatives for obtaining energy consumption of VNF/VNFC are described in clause 5.2.

The solution proposes to document different options for obtaining the energy consumption of VNF/VNFC based on:

- Estimation of energy consumption of VNFC based on resource usage metrics provided by ETSI NFV’s NFV-MANO (already supported by 3GPP TS 28.554 [2] in the case of VM-based VNF/VNFC),

- Collection of energy consumption of the OS container workloads or virtualized resource (VM) realizing the VNFC over a collection period as provided by ETSI NFV’s NFV-MANO,

- Collection of total energy consumption of the VNFC over the collection period as provided by ETSI NFV’s NFV-MANO, and

- Collection of total energy consumption of the VNF over the collection period as provided by ETSI NFV’s NFV-MANO.

The additional options are proposed to be documented in the relevant defined KPIs in the TS 28.554 [2] and TS 28.310 [12]. Where applicable, depending on the specific option, applicable 3GPP management system pre-conditions needs to be considered, like specified in clause 6.3.2 of TS 28.310 [12]. The further normative specification should also consider compatibility and/or alignments with updates that might be pursued to further support cloud-native and OS container-based deployments, where applicable.

It is recommended to specify the additional options for deriving the energy consumption of VNF/VNFC in the 28.554 [2] as part of the normative work.

End of Changes