3GPP TSG-SA5 Meeting #158	S5-247190

Orlando, USA, 18 - 22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0627 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 TS 28.552 Add UC reference for LTM related PM definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-10-28 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, two LTM-related use cases are defined in the current specifications including clause A.125 and A.132. However, the mapping relationship between the use case and the performance measurements is missing and which will makes confusing for using LTM performance measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the title of clause 5.1.1.6.12 Add performance measurements reference to the corresponding use case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of mapping relationships risks incomplete set of measurements for LTM cell switch. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.12.1, A.125, A.132 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

5.1.1.6.12	Distribution of time interval

5.1.1.6.12.1 Distribution of time interval between initiation of Early TA acquisition and initiation of L1/L2 Triggered Mobility (successful scenario)

a)	This measurement provides distribution of the time interval between initiation of Early TA acquisition and initiation of L1/L2 Triggered Mobility (LTM) in successful scenario. The successful scenario reflects here to a scenario when early TA acquisition is successfully completed before the LTM is triggered. The measurement is provided per source and target candidate cell pair and optionally may be also provided per 5QI.

b)	CC

c)	Inter gNB-DU LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of source gNB-DU serving the UE in this point of time (step 18 in Figure 8.2.1.5-1 of TS 38.300 ) with TA value include for the candidate target cell of the candidate target gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 13  in Figure 8.2.1.5-1 of TS 38.300 [49]) which was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU ((step 15 in Figure 8.2.1.5-1 of TS 38.300), and point in time when UE was previously instructed to start early TA acquisition procedure (step 13 in Figure 8.2.1.5-1 of TS 38.300 [49]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the clause 9.2.6 of TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Intra gNB-DU LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of gNB-DU serving the UE in this point of time (step 13 in Figure 8.2.1.4-1 of TS 38.300 [49]) with TA value include for the candidate target cell of the gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 11  in Figure 8.2.1.4-1 of TS 38.300 [49]) which was successfully completed via reception of the TA value in the source cell from candidate target cell internally within the gNB-DU, and point in time when UE was previously instructed to start early TA acquisition procedure (step 11 in Figure 8.2.1.4-1 of TS 38.300 [49]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in gNB-DU per source and target candidate cell pair.

d)	Each measurement is an integer.

e)	The measurement name has the form MM.TAAckLTMSuccDist.Bin.5QI, where Bin indicates a delay range which is vendor specific.

f)	NRCellDU;
NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS

Next change

A.125	Monitoring of TA acquisitions in LTM

LTM (L1/L2 Triggered Mobility) is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB changes UE’s serving cell by a cell switch command signaled via a MAC CE. The cell switch command indicates an LTM candidate cell configuration that the gNB previously prepared and provided to the UE through RRC signalling. Then the UE switches to the target cell according to the cell switch command. The LTM procedure can be used to reduce the mobility latency.

When configured by the network, it is possible to initiate UL TA acquisition procedure to one or multiple cells that are different from the current serving cell. For instance, the network may request the UE to perform early TA acquisition of a candidate cell before a cell switch. The early TA acquisition is triggered by the CFRA in such a way that the UE sends MSG1 towards the candidate cell without monitoring for a response from the candidate target cell as specified in clauses 9.2.3.5 and 9.2.6 in the TS 38.300 [49]. The TA validity is fully hidden for UE and thus cannot be reported via any UE report (nor in RACH report) to the network.

It is therefore recommended to monitor the validity of the Early TA acquisition procedure for LTM with the “Number of successful TA acquisitions for the candidate target LTM cell”, “Number of successful but late TA acquisitions for the candidate target LTM cell” and “Total number of TA acquisition requests for the candidate target LTM cell” measurements. Using the measurements operators may evaluate the probability of the successful early TA acquisition or efficiency of the Early TA acquisition and take appropriate action to optimize the LTM configuration and RACH in the candidate target cells.

The corresponding performance measurements of this use case is defined in clause 5.1.1.6.11.

A.132	Distribution of time interval for L1/L2 Triggered Mobility

LTM (L1/L2 Triggered Mobility) is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB changes UE’s serving cell by a cell switch command signaled via a MAC CE. The cell switch command indicates an LTM candidate cell configuration that the gNB previously prepared and provided to the UE through RRC signalling. Prior to sending the cell switch command it is possible to initiate UL TA acquisition procedure to one or multiple cells that are different from the current serving cell. For instance, the network may request the UE to perform early TA acquisition of a candidate cell before a cell switch. The early TA acquisition is triggered by PDCCH order as specified in clause 9.2.6 of the 3GPP 38.300. The obtained TA value may be then sent within the cell switch command which reduces mobility latency.  Then the UE switches to the target cell according to the cell switch command.

However, a LTM may be initiated even when TA acquisition has been triggered, but not yet completed. Typical reason may be when TA acquisition procedure was triggered too late. As another case a scenario when TA acquisition was successfully but TA value of the LTM candidate cell is evaluated as invalid. Typical reason may be when early TA acquisition is triggered too early.

It is therefore recommended to provide monitoring with the following measurements: “Distribution of time interval between initiation of Early TA acquisition and initiation of L1/L2 Triggered Mobility (successful scenario)”, “Distribution of time interval between initiation and successful completion of Early TA acquisition (successful scenario)”, “Distribution of time interval between initiation of Early TA acquisition and initiation L1/L2 Triggered Mobility (unsuccessful scenario 1)”, “Distribution of time interval between initiation and successful completion of Early TA acquisition (unsuccessful scenario 1)”, “Distribution of time interval between initiation of Early TA acquisition and initiation L1/L2 Triggered Mobility (unsuccessful scenario 2)”,  and “Distribution of time interval between initiation of and successful completion of Early TA acquisition (unsuccessful scenario 2) to identify proactively too late and too early triggered TA acquisition procedure and take preventive actions to avoid them.

The corresponding performance measurements of this use case is defined in clause 5.1.1.6.12.

End of change