3GPP TSG-SA WG6 Meeting #63	S6-244053

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Convida Wireless LLC

Title:	ML model lifecycle management

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, liu.lu@convidawireless.com

1. Introduction

Adds support for ML model lifecycle management to TS 23.482.

2. Reason for Change

Solution #17 in TR 23.700-82 addresses key issues #1 and #3 and was concluded to be considered for normative work with necessary updates. This contribution provide the necessary updates to describe how the ML model lifecycle can be managed by the AIMLE server. The discussion paper S6-244052 describes in more detail the ML model lifecycle management (also known as ML workflow). ML model lifecycle management is a highly iterative process. As such, the AIMLE server may offload the management of such repetitive tasks from the VAL layer.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0.

* * * First Change * * * *

## 8.y	ML model lifecycle management

### 8.y.1	General

ML model lifecycle management is a highly iterative process. To assist with ML model lifecycle management, the AIMLE server may offload the management of the repetitive tasks from the VAL layer. The AIMLE server performs ML model retrieval, selects AIMLE clients as necessary, schedules the AI/ML operations performed by AIMLE clients, performs AIMLE data management with AIMLE clients, configures and manages HFL and VFL training, and aggregation of training results. The AIMLE server may iterate among all the AI/ML operations over many training rounds and manage the AI/ML operations with many AIMLE clients. For each training round, the AIMLE server needs to manage changes with AIMLE clients for the next training round, monitor training performance, and update ML model.

The following clauses specify procedures, information flows, and APIs for ML model lifecycle management.

### 8.y.2	Procedure

Pre-conditions:

1.	The AIMLE service consumer has identified dataset requirements and specific ML model to train.

Figure 8.y.2-1: ML model lifecycle management

- An AIMLE service consumer (e.g. VAL server) sends a ML model lifecycle management request to an AIMLE server. The request includes information as described in Table 8.y.3.1-1.

- The AIMLE server authenticates and authorizes the request. If authorized, the AMILE server assigns an identifier for the subscription.

- The AIMLE server sends a ML model lifecycle management response that includes information in Table 8.y.3.1-3.

- The AIMLE server retrieves the indicated ML model from a ML repository using the ML model information discovery procedure as described in clause 8.11.3.

- The AIMLE server performs AIMLE client selection using the AIMLE client selection procedure as described in clause 8.9, steps 3 and 4.

- The AIMLE server uses the information in the AIML profile and the information provided in step 1 to determine the set of AI/ML operations and tasks for AIMLE clients to perform as part of ML model lifecycle management. The AIMLE server schedules the AI/ML operations to ensure application data is available before HFL/VFL training is started.

- The AIMLE server performs Client data management requests with AIMLE clients as describe in clause 8.x. The Client data management operation collects, prepares, and analyzes application data in preparation for HFL/VFL training.

- The AIMLE server configures AIMLE clients appropriate for HFL or VFL training. If HFL training is requested, the AIMLE server performs HFL training with AIMLE clients as describe in clause 8.12, steps 4 to 7. If VFL is requested, the AIMLE server performs VFL training with AIMLE clients as describe in clause 8.z.

- The AIMLE server aggregates the training results received from each AIMLE clients. During the aggregation, the AIMLE server evaluates the performance metric required for the training and determines whether steps 5 to 8 should be repeated. The determination is based on training requirements provided in the AIML profile. If training is complete, the AIMLE server performs ML model information storage as described in clause 8.11.2.

- The AIMLE server sends a notification to the AIMLE service consumer and include information as described in Table 8.y.3.1-4.

### 8.y.3	Information flows

#### 8.y.3.1	ML model lifecycle management subscription

Table 8.y.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE data management subscription procedure.

Table 8.y.3.1-1: ML model lifecycle management subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AIML profile | M | Requirements for the ML model lifecycle management operation |
| > ML model identifier | M | The identifier for the ML model and initial model parameters to train. |
| > AIMLE client set identifiers | O (NOTE1) | A list of AIMLE client set identifiers to train the ML model. |
| > AIMLE client selection criteria | O (NOTE1) | Selection criteria for finding suitable AIMLE clients for training the ML model. |
| > Number of required AIMLE clients | O (NOTE2) | A minimal number of AIMLE clients required for training the ML model. |
| > Dataset requirements | M | Dataset requirements for training the ML model. A dataset identifier may be provided to identify the dataset or data collection requirements as detailed in Table 8.x.3.1-2 may be provided. |
| > Data preparation requirements | M | Data preparation requirements as detailed in Table 8.x.3.1-3. |
| > Exploratory data analysis requirements | O | Exploratory data analysis requirements as detailed in Table 8.x.3.1-4. |
| > Training requirements | M | Training requirements as detailed in Table 8.y.3.1-2. |
| > Notification output requirements | O | Outputs requirements for notifications sent to the AIMLE service requestor. The notification output requirements can be “all” or any of the optional operational results information elements as detailed in Table 8.y.3.1-4. |
| Notification settings | O | Notification settings for which the AIMLE server provides ML model lifecycle management status: after task completion, after certain job percentage complete, periodically based on date and time, upon error or other triggered events, etc. |
| NOTE1: At least one of the information elements shall be provided. NOTE2: Mandatory if AIMLE client selection criteria is present. |  |  |

Table 8.y.3.1-2: Training requirements

| Information element | Status | Description |
| --- | --- | --- |
| Performance metric | M | Identifies the performance metric to evaluate ML model training. |
| Performance target | O | A target performance that indicates acceptable performance has been reached and training can be stopped. |
| Number of training rounds | M | A minimum number of training rounds for the ML training. |
| Number of data samples | M | A minimum number of data samples for the ML training. |
| Dataset age | O | The age of the dataset required for ML training. |

Table 8.y.3.1-3 shows the response sent by the AIMLE server to the AIMLE service consumer for the ML model lifecycle management subscription procedure.

Table 8.y.3.1-3: ML model lifecycle management subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the ML model lifecycle management operation |
| Subscription identifier | M | An identifier for the subscription. |
| Expiration time | O | Expiration time for the subscription |

Table 8.y.3.1-4 shows the notification sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management subscription procedure.

Table 8.y.3.1-4: ML model lifecycle management notify

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier for the subscription that notification is associated with. |
| Operational status | M | The status for the ML model lifecycle management operation. The status can represent the estimate percentage completion or associated with the notification settings. |
| Operational results | M | The operational result of the ML model training. |
| > Trained ML model | M | The final ML model (and model parameters) after training has been completed. |
| > Performance metric | M | The performance metric for training the ML model. Performance metric can be mean absolute error, mean squared error, accuracy, precision and recall, etc. The performance metric indicates the performance of the ML model. |
| > Number of AIMLE clients | O | The number of AIMLE clients used during ML model training. |
| > Number training rounds | O | The number of training rounds for ML model training. |
| > Number of data samples | O | The number of data samples used for ML model training. |
| > Age of datasets | O | The age of the datasets used for ML model training. |
| > Exploratory data analysis outputs | O | Provides aggregate outputs for exploratory data analysis: dataset identifier, statistical outputs for each feature, outlier and anomaly detection report, and feature correlation information from multivariate analysis. |
| > Elapse time | O | The time that has elapsed for the ML model lifecycle management operation. |
| Timestamp | O | Timestamp of the notification |

* * * End of Changes * * * *