3GPP TSG-SA WG6 Meeting #63	S6-244079

Hyderabad, India, 14th – 18th October 2024	(revision of S6-243116)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0126 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location reporting information obtained from the PLMN operator (5G) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 22.280, there is the location related requirement: [R-6.12-002] The MCX Service shall support conveyance of Location information provided by 3GPP location services. Location information obtained from the network comes with several advantages to the MC operators, as discussed in S6-244077, as: - The MC service server is always aware of the location information of MC users irrespective of possible unforeseen events, such as GPS deactivation intentionally or unintentionally, GPS not working properly, UE is off during a time a trigger should have been sent. - The MC service server is independent from the LMC (including its configuration) for its operations and decision making. - Maintain network resources and offload the radio interface especially during network congestion cases. There are two main mechanisms to obtain the location information from the core network: LCS services and via PCRF/PCF notification as part of the PCC rule mechanism. The LCS network can provide location information either on demand based (immediately) or event-triggered based, according to the procedures defined in 3GPP TS 23.273, e.g., mobile terminating location request related procedures. The type of events related to the event triggered based request are the UE availability (after temporarily being unavailable due to e.g., inaction of the user, IMSI detach, radio connectivity loss), periodic reporting, changing of a predefined geographical area or UE motion once moving more than a predefined distance from a certain location. The location information provided by the LCS report includes, among others, the geographical location of the MC service UE, the time stamp of the location information, and related event based on the requested triggers as mentioned earlier. Furthermore, the MC service server can obtain the Access Network Information Reporting parameters upon subscribing to the PCRF notification. The Access Network Information Reporting includes the user location report (e.g., the cell ID, SAI, PLMN ID) and the UE time zone, as described in 3GPP TS 23.503. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - adding a new reference to 3GPP TS 23.273 - updating the abbreviations in clause 3.3 to include GMLC - adding the reference point Le in the functional model in clause 5.3.1 - updating clause 7.5.1 to include concepts and references to TS 23.273 and TS 23.503 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirement indicated above is not met, and the mechanisms how the MC service server obtains location information from the PLMN operator is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 5.3.1, and 7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[15]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.304: "Proximity based Service (ProSe) in the 5G System (5GS); Stage 2".

[18]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[19]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[20]	3GPP TS 23.479: "UE MBMS APIs for Mission Critical Services".

[21]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[22]	3GPP TS 22.289: "Mobile communication system for railways".

[23]	3GPP TS 23.283: "Mission Critical Communication Interworking with Land Mobile Radio Systems".

[23.273]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

# * * * * Second change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GMLC	Gateway Mobile Location Centre

NPN	Non-Public Network

PNI-NPN	Public Network Integrated Non-Public Network

SNPN	Stand-alone Non-Public Network

NID	Network Identifier

# * * * * Third change * * * *

### 5.3.1	On-network functional model

Each MC service can be represented by an application plane functional model. The corresponding functional model across MC services may be similar but is described by the individual functional entities and reference points that belong to that MC service. Within the application plane for an MC service, a common set of functions as well as reference points is shared across MC services and is referred as the common services core.

Figure 5.3.1-1 shows the common functional model for the application plane for an MC system using 5GS.

Figure 5.3.1-1: Common functional model for application plane for an MC system

The common services core functions and reference points shown in figure 5.3.1-1 are shared across each MC service.

Figure 5.3.1-2 shows the common functional model for the signalling control plane using 5GS.

Figure 5.3.1-2: Common functional model for signalling control plane

In the model shown in figure 5.3.1-2, the SIP core may interact directly with 5GS via the N5 reference point or Rx reference point to control QoS on a per communication flow in accordance with 3GPP TS 23.501 [7].

NOTE 1:	Indirect interaction between SIP core and 5GS Network Exposure Function using N33 reference point is not supported by 3GPP TS 23.002 [8].

Figure 5.3.1-3 shows the relationships between the reference points of the common application plane of an MC service server and the common signalling plane.

Figure 5.3.1-3: Relationships between reference points of the common MC service application plane and signalling control planes

NOTE 2:	Application plane reference point CSC-7 makes use of SIP-2 reference point when the group management servers are connected by a single SIP core. Where they are joined by more than one SIP core, CSC-7 also makes use of the SIP-3 reference point.

NOTE 3:	For simplicity, the HTTP proxy, which provides the interconnection between HTTP-1, HTTP-2 and HTTP-3 reference points, is not shown in figure 5.3.1-3.

NOTE 4:	CSC-5, CSC-9, and CSC-15 make use of SIP-1 and SIP-2 reference points. For simplicity, this mapping relationship is not shown in figure 5.3.1-3.

# * * * * Fourth change * * * *

### 7.5.1	General

In addition to the location management functions defined in 3GPP TS 23.280 [3], the location management system over 5GS shall support the following enhancement:

-	Inter-system RAT changes event triggering and report as described in clause 7.3.3.9.1.

-	Serving and neighbouring 5G NR Cell Global Identifiers (NCGI) as optional Location Information elements in the information flows of 3GPP TS 23.280 [3] clause 10.9.2.

-	The MC service server may obtain MC service UE location information from the LCS services via the LCS server (in specific from GMLC) at the 5GS, either directly via the Le interface, or indirectly via the NEF, as described in 3GPP TS 23.273[23.273]. The identity of the target MC service UE is identified by SUPI, which may contain the IMSI, as defined in 3GPP TS 23.501[7], and shall be shared by the MC service client during the service authorisation procedure if the location information is obtained from the LCS services.

-	The MC service server can subscribe to the PCF related notification to obtain the MC service UE location information as described in 3GPP TS 23.503 [9].

# * * * * End of changes * * * *