3GPP TSG-SA WG6 Meeting #63	S6-244124

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Convida Wireless LLC

Title:	Metaverse Session Management

Spec:	3GPP TS 23.438 V0.0.0

Agenda item:	9.11

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com

1. Introduction

This contribution proposes metaverse session management procedures for managing digital avatar QoS synchronization adapted to the A-DACM architecture.

2. Reason for Change

The conclusion for key issue #5 in TR 23.700-21 specifies solution #10 can be considered for normative work with adaptation to agreed architure. This contribution proposed to adapt solution #10 to the A-DACM architecture to provide QoS control for metaverse sessions involving digital avatars. The resulting synchronization of the digital avatar communications is managed by the SEALDD layer.

Support for metaverse sessions within the A-DACM architecture enables metaverse applications to interact with the A-DACM at a simpler and higher level of abstraction. A metaverse session can be defined to include information for each VAL client and VAL server along with information for each of the digital avatars participating in the metaverse session. Using the metaverse session information, the A-DACM can assist metaverse applications by interacting with other SEAL services on their behalf. As an example, the A-DACM can interact with a SEALDD server to assist with establishing and managing multi-modal SEALDD connections required for the digital avatars participating in the metaverse session.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.438 V0.0.0.

* * *  First Change  * * * *

## 8.X	Metaverse session management

### 8.X.1	Metaverse session create

#### 8.X.1.1	General

Metaverse session creation enables an A-DACM client or VAL server to create an metaverse session for multiple digital avatars to interact with each other.

#### 8.X.1.2	Procedure

Figure 8.X.1.2-1 illustrates the metaverse session creation procedure.

Pre-conditions:

1.	The A-DACM client or VAL server has received information (e.g. URL, IP address/port) related to the A-DACM server.

2.	The A-DACM client or VAL server has received security credentials authorizing it to communicate with the A-DACM server.

3.	For A-DACM client to trigger this procedure, the A-DACM client has received the required information from the VAL client.

4.  The A-DACM server has received information (e.g., URL, IP address/port) related to a SEALDD server.

5. 	Digital avatars that will be part of the metaverse session have been created and are accessible.

Figure 8.X.1.2-1: Metaverse session create

1.	The requestor (e.g., A-DACM client or VAL server) sends a metaverse session create request to the A-DACM server. The request includes a requestor identifier, security credentials, and a list of VAL flows associated with the metaverse session to be created as described in clause 8.X.1.3.1. For each VAL flow, a VAL client and server endpoint is included to associate with a digital avatar participating in the metaverse session. The request may also include multi-modal associations between the VAL flows of the metaverse session and access control rules defining which entities are permitted to discover and access the metaverse session.

2.	Upon receiving the request, the A-DACM server validates if the requestor is authorized for the request. If the requestor is authorized, the A-DACM server creates the metaverse session, assigns a metaverse session identifier to the newly created metaverse session and stores the metaverse session information.

Based on the metaverse session information, the A-DACM server initiates SEALDD policy configuration requests as defined in clause 9.10.2.1 of 3GPP TS 23.433 [Y] to configure SEALDD policy(s) for the Metaverse application flows associated with the metaverse session. The request(s) include an application service identifier, identifiers of the VAL server, VAL client, VAL UE or VAL user, QoS information, and multi-modal information associated with the metaverse session VAL flows.

3.	The A-DACM server sends an metaverse session create response to the requestor and includes information as described in clause 8.X.1.3.1. If the A-DACM server created the metaverse session, the response includes an indication of success and an metaverse session identifier of the newly created metaverse session. Otherwise, the response includes an indication of failure and can include a reason for failure.

#### 8.X.1.3	Information flows

##### 8.X.1.3.1	Metaverse session create

Table 8.X.1.3.1-1 describes information elements for the metaverse session create request from the A-DACM client or VAL server to the A-DACM server.

Table 8.X.1.3.1-1: Metaverse session create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Application service ID | M | The identifier of the metaverse application service associated with the metaverse session (e.g., VAL service ID). |
| Access control rules | O | Access control rules defining which entities are permitted to discover the metaverse session. |
| List of metaverse application flows | M | List of metaverse application flows associated with the metaverse session |
| > VAL server ID | M | Identifier of the VAL server associated with the VAL flow |
| > VAL server endpoint | M | Endpoint information of the VAL server endpoint of the VAL flow. Endpoint information may be specified at the granularity of a VAL server address/port (e.g., FQDN, IP address/port, URL) or at the granularity of a VAL server digital asset / avatar resource (e.g., URI). |
| > VAL client ID | M | Identifier of the VAL client associated with the VAL flow |
| > VAL client endpoint | M | Endpoint information of the VAL client endpoint of the VAL flow. Endpoint information may be specified at the granularity of a VAL client address/port (e.g., FQDN, IP address/port, URL) or at the granularity of a VAL client digital asset / avatar resource (e.g., URI). |
| > QoS information | O | QoS requirements of the VAL flow |
| > Multi-modal information | O | Information regarding multi-modal associations between metaverse session VAL flows |
| >> Multi-modal ID | O | Identifier used to indicate multi-modal associations between the metaverse session VAL flows. VAL flows having the same multi-modal ID have a multi-modal association. |
| >> Synchronization info | O | Multi-modal synchronization information for VAL flows having the same multi-modal ID |

Table 8.X.1.3.1-2 describes information elements for the metaverse session create response.

Table 8.X.1.3.1-2: Metaverse session create response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the metaverse session create request was successful. |
| > Metaverse session ID | O | Created metaverse session identifier. |
| Failure response | O | Indicates that the create metaverse session request has failed. |
| > Cause | O | The cause for the request failure. |

### 8.X.2	Metaverse session update

#### 8.X.2.1	General

A metaverse session update enables an A-DACM client or VAL server to update a metaverse session with the A-DACM server.

#### 8.X.2.2	Procedure

Figure 8.X.2.2-1 illustrates the metaverse session update procedure.

Pre-conditions:

1.	The A-DACM client or VAL server has received information (e.g. URL, IP address/port) related to the A-DACM server.

2.	The A-DACM client or VAL server has received security credentials authorizing it to communicate with the A-DACM server.

3.	For A-DACM client to trigger this procedure, the A-DACM client has received the required information from the VAL client.

4.  The A-DACM server has received information (e.g., URL, IP address/port) related to a SEALDD server.

Figure 8.X.2.2-1: Metaverse session update

1.	The requestor (e.g., A-DACM client or VAL server) sends a metaverse session update request to the A-DACM server. The request includes a requestor identifier, security credentials, the identifier of the metaverse session, and metaverse session information to update as described in clause 8.X.2.3.1.

2.	Upon receiving the request, the A-DACM server validates if the requestor is authorized for the request. If the requestor is authorized and the metaverse session identifier provided in the request is found, the A-DACM server updates the metaverse session information based on information provided in the request.

Based on the metaverse session information, the A-DACM server initiates SEALDD policy configuration update requests as defined in clause 9.10.2.2 of 3GPP TS 23.433 [Y] to update SEALDD policy configuration(s) for the Metaverse application flows associated with the metaverse session. The request(s) include updated SEALDD policy configuration information (e.g., updated application service identifier, identifiers of the VAL server, VAL client, VAL UE or VAL user, QoS information, and multi-modal information associated with the metaverse session VAL flows).

3.	The A-DACM server sends a metaverse session update response to the requestor and includes information as described in clause 8.X.2.3.1. If the A-DACM server updated the metaverse session, the response includes an indication of success and the identifier of the updated metaverse session. Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 8.X.2.3	Information flows

##### 8.X.2.3.1	Metaverse session update

Table 8.X.2.3.1-1 describes information elements for the metaverse session update request from the A-DACM client or VAL server to the A-DACM server.

Table 8.X.2.3.1-1: Metaverse session update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Metaverse session ID | M | The identifier of the metaverse session |
| Updated metaverse session information | M | Updated metaverse session information as described in Table 8.X.1.3.1-1. |

Table 8.X.2.3.1-2 describes information elements for the metaverse session update response from the A-DACM server.

Table 8.X.2.3.1-2: Metaverse session update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the update metaverse session request was successful. |
| > Metaverse session ID | O | Identifier of the updated metaverse session |
| Failure response | O | Indicates that the update metaverse session request has failed. |
| > Cause | O | The cause for the request failure. |

### 8.X.3	Metaverse session delete

#### 8.X.3.1	General

A metaverse session deletion enables a A-DACM client or VAL server to delete a metaverse session with the A-DACM server.

#### 8.X.3.2	Procedure

Figure 8.X.3.2-1 illustrates the metaverse session deletion procedure.

Pre-conditions:

1.	The A-DACM client or VAL server has received information (e.g. URL, IP address/port) related to the A-DACM server.

2.	The A-DACM client or VAL server has received security credentials authorizing it to communicate with the A-DACM server.

3.	For A-DACM client to trigger this procedure, the A-DACM client has received the required information from the VAL client.

4.  The A-DACM server has received information (e.g., URL, IP address/port) related to a SEALDD server.

Figure 8.X.3.2-1: Metaverse session deletion

1.	The requestor (e.g., A-DACM client or VAL server) sends an metaverse session delete request to the A-DACM server. The request includes a requestor identifier, security credentials, the identifier of the metaverse session to delete as described in clause 8.X.3.3.1.

2.	Upon receiving the request, the A-DACM server validates if the requestor is authorized for the request. If the requestor is authorized and the metaverse session identifier provided in the request is found, the A-DACM server deletes the metaverse session information.

Based on the metaverse session information, the A-DACM server initiates SEALDD policy configuration delete requests as defined in clause 9.10.2.3 of 3GPP TS 23.433 [Y] to delete SEALDD policy configuration(s) for the Metaverse application flows associated with the metaverse session.

3.	The A-DACM server sends an metaverse session delete response to the requestor and includes information as described in clause 8.X.3.3.1. If the A-DACM server deleted the metaverse session, the response includes an indication of success and the identifier of deleted metaverse session. Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 8.X.3.3	Information flows

##### 8.X.3.3.1	Metaverse session delete

Table 8.X.3.3.1-1 describes information elements for the metaverse session delete request from the A-DACM client or VAL server to the A-DACM server.

Table 8.X.3.3.1-1: Metaverse session delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Metaverse sessions ID | M | Identifier of the metaverse session to be deleted. |

Table 8.X.3.3.1-2 describes information elements for the metaverse session delete response from the A-DACM server to the A-DACM client or VAL server.

Table 8.X.3.3.1-2: Metaverse session delete response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the delete metaverse session request was successful. |
| > Metaverse session ID | O | Identifier of the deleted metaverse session |
| Failure response | O | Indicates that the delete metaverse session request has failed. |
| > Cause | O | The cause for the request failure. |

### 8.X.4	Metaverse session discovery

#### 8.X.4.1	General

A metaverse session discovery enables an A-DACM client or VAL server to discover metaverse session(s) with the A-DACM server. The discovery of metaverse session(s) is based on matching metaverse session discovery filters provided in the request.

#### 8.X.4.2	Procedure

Figure 8.X.4.2-1 illustrates the procedure for metaverse session discovery following the request-response model.

Pre-conditions:

1.	The A-DACM client or VAL server has received information (e.g. URL, IP address/port) related to the A-DACM server; and

2.	The A-DACM client or VAL server has received security credentials authorizing it to communicate with the A-DACM server.

3.	For A-DACM client to trigger this procedure, the A-DACM client has received the required information from the VAL client.

Figure 8.X.4.2-1: Metaverse session discovery

1.	The requestor (e.g., A-DACM client or VAL server) sends an metaverse session discovery request message to the A-DACM server to obtain information about metaverse sessions of interest. The request includes a requestor identifier, security credentials, and may include metaverse session discovery filters as described in clause 8.X.4.3.1. If the discovery request was triggered by a VAL client on a UE, the metaverse session discovery request may additionally include a VAL client identifier.

2.	Upon receiving the request, the A-DACM server validates if the requestor is authorized for the request. If the requestor is authorized, the A-DACM server identifies metaverse session(s) that satisfies the metaverse session discovery filters. For each matching metaverse session, the A-DACM server checks whether the requestor is authorized to obtain the metaverse session based on the identity and based on the access control rules.

3.	The A-DACM server sends an metaverse session discovery response to the requestor and includes information as described in clause 8.X.4.3.1. If the A-DACM server has identified metaverse session(s), the response includes an indication of success and the identified metaverse sessions. Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 8.X.4.3	Information flows

##### 8.X.4.3.1	Metaverse session discovery request

Table 8.X.4.3.1-1 describes information elements for the metaverse session discovery request from the A-DACM client or VAL server to the A-DACM server.

Table 8.X.4.3.1-1: Metaverse session discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is an A-DACM client. |
| Discovery filters | O | The filters used for determining the metaverse sessions. |
| > List of metaverse session IDs | O | The list of metaverse session identifiers to discover. |
| > List of metaverse application service IDs | O | The list of identifiers (e.g., VAL service ID) of metaverse application services to discover |

Table 8.X.4.3.1-2 describes information elements for the metaverse session discovery response from the A-DACM server.

Table 8.X.4.3.1-2: Metaverse session discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the metaverse session discovery request was successful. |
| List of metaverse sessions | O | List of discovered metaverse sessions. |
| > Metaverse session ID | M | Identifier of the discovered metaverse session. |
| > Metaverse application service ID | M | The identifier of the metaverse application service associated with the metaverse session (e.g., VAL service ID). |
| > List of metaverse application flows | M | List of metaverse application flows associated with the metaverse session as described in Table 8.X.1.3.1-1. |
| Failure response | O | Indicates that the metaverse session discovery request has failed. |
| > Cause | O | The cause for the request failure. |

### 8.X.5	Metaverse session subscription

#### 8.X.5.1	General

A metaverse session subscription enables a A-DACM client or VAL server to subscribe with the A-DACM server to receive notifications related to metaverse sessions.

#### 8.X.5.2	Procedures

##### 8.X.5.2.1	Subscribe

Figure 8.X.5.2.1-1 illustrates the metaverse session subscription procedure.

Pre-conditions:

1.	The A-DACM client or VAL server has received information (e.g. URL, IP address/port) related to the A-DACM server; and

2.	The A-DACM client or VAL server has received security credentials authorizing it to communicate with the A-DACM server.

3.   For A-DACM client to trigger this procedure, the A-DACM client has received the required information from the VAL client.

Figure 8.X.5.2.1-1: Metaverse session subscription

1.	The requestor (e.g., A-DACM client or VAL server) sends an metaverse session subscribe request to the A-DACM server. The request includes a requestor identifier, security credentials, a notification endpoint, and types of metaverse session events for which the requestor is subscribing as described in clause 8.X.5.3.1. The request may include metaverse session event filters and an expiration time.

2.	Upon receiving the request from the requestor, the A-DACM server validates if the requestor is authorized for the request. If the requestor is authorized, the A-DACM server creates the subscription and stores the subscription information. The A-DACM server starts monitoring for metaverse sessions events according to the metaverse session event types and filters defined in the request.

3.	The A-DACM server sends an metaverse session subscribe response to the requestor and includes information as described in clause 8.X.5.3.1. If the A-DACM server has created the subscription, the response includes an indication of success, the subscription identifier and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the metaverse session subscription expires. If the A-DACM server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.X.5.2.2	Notify

Figure 8.X.5.2.2-1 illustrates the metaverse session notify operation between the A-DACM server and an A-DACM client or VAL server.

Pre-conditions:

1.	The A-DACM client or VAL server has subscribed for metaverse session notifications with the A-DACM server.

Figure 8.X.5.2.2-1: Metaverse session notification

1.	An event occurs at the A-DACM server that satisfies trigger conditions for notifying a subscriber (e.g., A-DACM client or VAL server) according to subscribed event types and filters.

2.	The A-DACM server sends an metaverse session notification to the requestor indicating the event. The notification includes information as described in clause 8.X.5.3.2.

##### 8.X.5.2.3	Subscription update

Figure 8.X.5.2.3-1 illustrates the procedure for an A-DACM client or VAL server to update a subscription with the A-DACM server.

Pre-conditions:

1.	The A-DACM client or VAL server has subscribed for metaverse session notifications with the A-DACM server.

Figure 8.X.5.2.3-1: Metaverse session subscription update

1.	The requestor (e.g., A-DACM client or VAL server) sends an metaverse session subscription update request to the A-DACM server. The request includes the requestor identifier, security credentials, a subscription identifier and may include subscribed event information and expiration time as described in clause 8.X.5.3.3.

2.	Upon receiving the request from the requestor, the A-DACM server validates if the requestor is authorized for the request. If the requestor is authorized, the A-DACM server updates the subscription information.

3.	The A-DACM server sends an metaverse session subscription update response to the requestor and includes information as described in clause 8.X.5.3.3. If the A-DACM server has updated the subscription, the response includes an indication of success and may include an expiration time. To maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the metaverse session subscription expires. If the A-DACM server has not updated the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.X.5.2.4	Unsubscribe

Figure 8.X.5.2.4-1 illustrates the procedure for an A-DACM client or VAL server to unsubscribe with the A-DACM server.

Pre-conditions:

1.	The A-DACM client or VAL server has subscribed for metaverse session notifications with the A-DACM server.

Figure 8.X.5.2.4-1: Metaverse session unsubscribe

1.	The requestor (e.g., A-DACM client or VAL server) sends an metaverse session unsubscribe request to the A-DACM server. The request includes the requestor identifier, security credentials and the subscription identifier as described in clause 8.X.5.3.4.

2.	Upon receiving the request from the requestor, the A-DACM server validates if the requestor is authorized for the request. If the requestor is authorized, the A-DACM server cancels the subscription associated with the subscription identifier.

3.	The A-DACM server sends an metaverse session unsubscribe response to the requestor and includes information as described in clause 8.X.5.3.4. If the A-DACM server has canceled the subscription, the response includes an indication of success. If the A-DACM server has not canceled the subscription, the response includes an indication of failure and may include a reason for failure.

#### 8.X.5.3	Information flows

##### 8.X.5.3.1	Metaverse session subscribe

Table 8.X.5.3.1-1 describes information elements for the metaverse session subscribe request from the A-DACM client or VAL server to the A-DACM server.

Table 8.X.5.3.1-1: Metaverse session subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is an A-DACM client. |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| List of event types | M | The types of events for which the requestor is subscribing. Supported event types include: - metaverse session creation - metaverse session update - metaverse session deletion |
| List of event filters | O | The characteristics of metaverse sessions that the A-DACM server considers for triggering a notification. |
| > List of metaverse session IDs | O | Identifiers of the metaverse sessions of interest. |
| > List of metaverse application service IDs | O | Identifiers (e.g., VAL service IDs) of metaverse application services of interest |

Table 8.X.5.3.1-2 describes information elements for the metaverse session subscribe response sent by the A-DACM server.

Table 8.X.5.3.1-2: Metaverse session subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

##### 8.X.5.3.2	Metaverse session notification

Table 8.X.5.3.2-1 describes information elements for the metaverse session notification sent by the A-DACM server.

Table 8.X.5.3.2-1: Metaverse session notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Metaverse session ID | M | Identifier of metaverse session associated with the event. |
| Event type | M | The type of event detected. Supported event types include: - metaverse session creation - metaverse session update - metaverse session deletion |

##### 8.X.5.3.3	Metaverse session subscription update

Table 8.X.5.3.3-1 describes information elements for the metaverse session subscription update request from the A-DACM client or VAL server to the A-DACM server.

Table 8.X.5.3.3-1: Metaverse session subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | Security credentials of the requestor. |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Event type | O | The type of event for which the requestor is subscribing. Supported event types include: - metaverse session creation - metaverse session update - metaverse session deletion |
| Metaverse session filters | O | The characteristics of metaverse sessions that the A-DACM server considers for triggering a notification. |
| > List of metaverse session IDs | O | Identifiers of the metaverse sessions of interest. |
| > List of Metaverse application service IDs | O | Identifiers (e.g., VAL service IDs) of metaverse application services of interest |

Table 8.X.5.3.3-2 describes information elements for the metaverse session subscription update response sent by the A-DACM server.

Table 8.X.5.3.3-2: Metaverse session subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |

##### 8.X.5.3.4	Metaverse session unsubscribe

Table 8.X.5.3.4-1 describes information elements for the metaverse session unsubscribe request from the A-DACM client or VAL server to the A-DACM server.

Table 8.X.5.3.4-1: Metaverse session unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | Security credentials of the requestor |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be unsubscribed |

Table 8.X.5.3.4-2 describes information elements for the metaverse session unsubscribe response sent by the A-DACM server.

Table 8.X.5.3.4-2: Metaverse session unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the unsubscribe request was successful. |
| Failure response | O | Indicates that the unsubscribe request failed. |
| > Cause | O | Indicates the cause of unsubscribe request failure |

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y]	3GPP TS 23.433: “SEAL Data Delivery Enabler for Vertical Applications”.

* * * End of Change * * * *