3GPP TSG-SA WG6 Meeting #63	S6-244135

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0060 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new feature to support tethered device discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in FS_XRApp, the PINAPP is utilized to do the tethered device discovery and management. Additionally, as captured by Editor's note: The specific service and API supporting the PINE discovery needs to be defined in PINAPP. The PINAPP should support tethered device discovery and management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new feature to support tethered device discovery and management, i.e., the PIN client requests to discover other PIN element services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting the PIN client requests to discover other PIN elements results in the tethering link measurement and provisioning services not being supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.8.3.1, new clause: 8.5.8.2.x, 8.5.8.3.x, 8.5.8.3.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.5.8.2.x	PIN client requests to discover other PIN element

Figure 8.5.8.2.x-1 illustrates procedure of PIN client requests to discover other PIN element, based on request/response model.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC and already has application layer connection with PEMC;

4.	Dynamic profile information about the PIN is available at the PEMC and PIN server.

Figure 8.5.8.2.x-1: PIN client requests to discover other PIN element

- The PIN client sends the PIN Management PINE discovery request to PEMC/PIN server. The request includes the security credentials of the PIN client, PIN ID.

- Upon receiving the request, the PEMC/PIN server performs an authorization check to verify whether the PIN client has authorization to discover other PINE(s). If authorized, PEMC/ PIN server sends the PIN Elements List in the Dynamic profile information of the requested PIN to the PIN client, based on the Visibility of PINE.

* * * Next Change * * * *

8.5.8.3	Information flows

8.5.8.3.1	General

The following information flows are specified for PIN creation:

-	PIN Management PINE join into PIN request and response;

-	PIN Management PINE leave from PIN request and response;

-	PIN Management PINE discovery request and response;

8.5.8.3.x		PIN Management PINE discovery request

Table 8.5.8.3.x-1 describes information elements in the PIN Management PINE discovery request from the PINE to PEMC, or from PINE to PIN server.

Table 8.5.8.3.x-1: PIN Management PINE discovery request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the PIN that wants to discover other PINE. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN client ID | M | The PIN client ID of PINE |
| UE Identifier | M | The identifier of the hosting UE (i.e. GPSI or identity token) or the PIN client ID of PEMC |
| PIN server endpoint information | O | Includes URI(s), FQDN(s), IP address(es)) of PIN server. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.2.7. |
| Services that PINE provide | O | Indicate the service that PINE can provide. |

8.5.8.3.y	PIN Management PINE discovery response

Table 8.5.8.3.y-1 describes information elements in the PIN Management PINE discovery response from the PEMC to PINE, or from PIN server to PINE.

Table 8.5.8.3.y-1: PIN Management PINE discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (see NOTE) | Indicates that the PIN Management PINE discover request was successful. |
| > PIN Elements List | M | List of PIN elements that is discoverable within the PIN and their details. |
| >> PINE ID | M | The identifier of the PINE |
| >> Endpoint information | M | Information of each PINE (e.g. URI, FQDN, IP address, port number) used to communicate with the PINE |
| > Services | O | Services offered by the PIN element |
| > Reachability information | O | Reachability information of the PIN element may include the IP address which is internally routable via PEGC (like LAN or Layer 2 gateway) or routable via 5GS. |
| Failure response | O (see NOTE) | Indicates that the PIN Management PINE discover request failed. |
| > Cause | M | Provides the cause for PIN Management PINE discover request failure. |
| NOTE: At least one of the IE shall be present. |  |  |