3GPP TSG-SA WG6 Meeting #63	S6-244138

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0091 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct SEALDD-Uu |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SEALDD-Uu is used in several places. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change SEALDD-Uu to SEALDD-UU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-alignment for reference point name. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 9.6.2.1, 9.6.2.2, 9.6.3.1, 9.6.3.2, 9.6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application connection: Association between the VAL client and the VAL service to provide the VAL service.

Application traffic: a set of signaling messages or a set of media packet constituting a application service flow. E.g., one or more picture(s) may be an application traffic, the audio stream and the video stream are two different application traffic.

Crossflow QoS measurement is an E2E measurement between SEALDD client and SEALDD server that considers multi-modal traffic routes in combination of uplink direction and downlink direction for same application.

SEALDD–S connection: Association between the SEALDD server and the VAL server to achieve data delivery of the application traffic. One application connection may have one SEALDD-S connection, e.g., for all the application traffic flows, or have more SEALDD-S connections, e.g., one for application signaling flow, and the other or application media traffic flow(s). A SEALDD-S connection shall be uniquely identified between the VAL server and the SEALDD server.

SEALDD-S flows: a set of packets have the same attributes, e.g., the same 5-tuple, media type. The application traffic flow may be mapped to one ore more SEALDD-S flow(s). A SEALDD-S flow should be uniquely identified with the  SEALDD-S connection.

SEALDD-UU connection: Association between the SEALDD server and the SEALDD client to achieve the data delivery of the application traffic from the VAL server or the VAL client. One SEALDD-S connection may be associated with one SEALDD-UU connection or multiple SEALDD-UU connections. A SEALDD-UU connection shall be uniquely identified between the SEALDD client and the SEALDD server.

SEALDD-UU flows: a set of packets have the same attributes, e.g., the same 5-tuple, media type, QoS requirements. One SEALDD-S flow(s) may be mapped to one or multiple SEALDD-UU flows. A SEALDD-UU flow should be uniquely identified with the SEALDD-UU connection.

* * * Next Change * * * *

#### 9.6.2.1	SEALDD context transfer

Figure 9.6.2.1-1 describes SEALDD context transfer procedure in pull (step 1a and 2a) or push (step 1b and 2b) operation.

Pre-conditions:

1.	For pull operation, the old SEALDD server endpoint is available in the new SEALDD server

2.	For push operation, the new SEALDD server endpoint is available in the old SEALDD server.

Figure 9.6.2.1-1: SEALDD context transfer

The transferred SEALDD context includes the service subscription information created upon VAL server's interaction for requesting SEALDD transmission service (e.g. step 1 and 2 in Figure 9.2.2.1-1), and also the SEALDD client and SEALDD server communication tunnel management information (e.g. UE IP address) which is created upon SEALDD client's interaction for requesting data transmission (e.g. step 5 and 6 in Figure 9.2.2.1-1). If new SEALDD server supports transportation layer service continuity, additional transport layer context (e.g. TCP/TLS/QUIC) is transferred from old SEALDD server to new SEALDD server. If the new SEALDD Server is not provisioned with the VAL server configured SEALDD policy, then it can pull from the Old SEALDD server via the pull operation. The push operation does not support the transfer of the VAL server configured SEALDD policy between Old and New SEALDD server.

The new SEALDD server, after receiving the SEALDD context from the old SEALDD server, allocates IP address and port for SEALDD-UU user plane communication. The new SEALDD server also sends back to the old SEALDD server with the allocated endpoint information If operation is push. Then the old SEALDD server can request 5GC to perform IP replacement procedure, as defined in clause 6.3.3 of 3GPP 23.548 [8]. The request includes the traffic descriptor of old SEALDD server (i.e., SEALDD-UU address/port), the traffic descriptor of new SEALDD server (i.e., SEALDD-UU address/port) and/or the target DNAI.

Optionally, after receiving the SEALDD context with the SEALDD-UU endpoint from the old SEALDD server, the new SEALDD server can request 5GC to perform IP replacement procedure.

NOTE:	The TCP/TLS/QUIC context transfer is required to support the SEALDD server relocation with IP replacement procedure, as described in clause 9.6.2.2.

To improve the seamless SEALDD relocation, the old SEALDD server may stop the downlink data transmission before pushing the SEALDD context to the new SEALDD server, the context data may include content breakpoint information, as defined in clause 9.6.3.1. After the SEALDD client connects to the new SEALDD server, the new SEALDD server transmits the downlink traffic to the SEALDD client based on content breakpoint information.

* * * Next Change * * * *

#### 9.6.2.2	SEALDD relocation in EDN

Pre-conditions:

1.	VAL Server 1 and 2 are adapted to the EDGEAPP as EAS.

2.	VAL Server 1 and 2 register its associated SEALDD server in EES as described in clause 9.4.3.2.

Figure 9.6.2.2-1: SEALDD support of UE's service continuity

1.	An application client on a UE, acting as a VAL client, establishes a SEALDD-UU flow over SEALDD-UU to send application data to VAL Server 1. SEALDD client and SEALD server 1 maintain SEALDD-UU flow information (e.g., SEALDD-UU flow ID, VAL IDs/addresses, VAL requirements).

2.	Application data between the VAL Client and VAL Server 1 is sent via the SEALDD-UU flow.

3.	The UE moves and generates a mobility event in the 5GC.

4.	The UE's mobility event triggers the execution of an Application Context Relocation (ACR) procedure as described in 3GPP TS 23.558 [10]; or VAL server 1 triggers ACR due to load re-balancing reason. Any of the ACR scenarios detailed in 3GPP TS 23.558 [10] clauses 8.8.2.2-8.8.2.6 may occur. In this step, the first three phases of the ACR procedure are performed, up to ACT. In this step VAL Server 1 acts as EAS 1 and VAL Server 2 as EAS 2, therefore participating in corresponding signalling. VAL server 2 has been selected as SEALDD-enabled server meeting the ACR criteria to be the target EAS. The associated SEALDD server 2 has also been selected and may support transportation layer (e.g. UDP/TCP/QUIC) service continuity.

5a.	Before triggering ACT, VAL Server 1 sends a SEALDD notification of ACR event to SEALDD Server 1.

5b. Before triggering ACT, VAL Server 2 sends a SEALDD notification of ACR event to SEALDD Server 2.

6a.	SEALDD server 1 transfers the SEALDD context to the SEALDD server 2 which serves VAL Server 2 as described in clause 9.6.2.1, with push operation. The SEALDD server 1 can obtain the SEALDD server 2 endpoint for SEALDD-UU user plane to the SEALDD client, as specified in clause 9.6.2.1.

6b. SEALDD server 2 can obtain the SEALDD context from the SEALDD server 1 by using the pull operation, as described in clause 9.6.2.1.

7.	SEALDD server 1 applies the functionality specified in 3GPP TS 23.502 [6] clause 5.2.6.7 for AF traffic influence, providing the N6 routing information for the SEALDD client and SEALDD server 2. The SEALDD server 1 may:

-	If the SEALDD server 2 supports transportation layer service continuity, additionally includes SEALDD IP replacement information (i.e. SEALDD server 1 endpoint and SEALDD server 2 endpoint for SEALDD-UU user plane) in the AF traffic influence. Since the UE is not aware of SEALDD server change, the new SEALDD traffic (due to new VAL traffic sent by VAL client) is sent by UPF towards the new SEALDD server, this handling in UPF is agnostic to the SEALDD server 2. Or,

-	send AF traffic influence with target DNAI of SEALDD server 2, and simultaneous connectivity indicator, to request 5GC to maintain the simultaneous connectivity over source PSA and target PSA with source SEALDD server and target SEALDD server, as described in clause 6.3.4 of 3GPP TS 23.548 [8].

8.	If the SEALDD server 2 has no transportation layer service continuity support, a SEALDD Connection info update notification is sent to SEALDD client, e.g. to update the allocated IP address and port for SEALDD-UU user plane communication. Then the SEALDD client acknowledges the received SEALDD Connection info update notification. After SEALDD client is aware of the new SEALDD-UU IP address and port, it starts to send new SEALDD traffic (received from the VAL client) over the new connection. The new SEALDD server maps the received SEALDD traffic to the application traffic according to the SEALDD traffic descriptor and VAL service ID. The new SEALDD server sends the recovered application traffic to new VAL server. The downlink application traffic sent from the new VAL server to VAL client is processed similarly.

9.	SEALDD Server 1 notifies VAL Server 1 of the completion of the SEALDD-UU flow transfer.

10.	VAL Server 1 (acting as EAS1) and VAL Server 2 (acting as EAS 2) execute the Application Context Transfer (ACT) procedure step corresponding to the pending ACR scenario (3GPP TS 23.558 [10] clauses 8.8.2.2-8.8.2.6), which is out the 3GPP scope.

11.	The post-ACR clean-up phase is executed, as described in the corresponding ACR scenario (3GPP TS 23.558 [10] clauses 8.8.2.2-8.8.2.6).

12.	The application data from the VAL Client is sent via the SEALDD-UU flow (with SEALDD server 2) to VAL Server 2.

NOTE:	The SEALDD-UU client endpoint of the SEALDD-UU flow in this step is maintained the same as in step 1.

* * * Next Change * * * *

#### 9.6.3.1	SEALDD context push request

Table 9.6.3.1-1 describes the information flow from the old SEALDD server to the new SEALDD server to push the SEALDD context.

Table 9.6.3.1-1: SEALDD context push request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identifies the requestor (i.e. old SEALDD server). |
| SEALDD-UU Context | M | Identifies the context related to SEALDD-UU connection, which is created upon SEALDD connection establishment. |
| SEALDD-S Context | M | Identifies the context related to SEALDD-S subscription. |
| Transport layer context | O | Identifies the context related to Transport layer (e.g. TCP/TLS/QUIC) for SEALDD-UU user plane communication. |
| Content breakpoint information | O | The downlink data breakpoint information for pre-provisioned data. |

* * * Next Change * * * *

#### 9.6.3.2	SEALDD context push response

Table 9.6.3.2-1 describes the information flow from the new SEALDD server to the old SEALDD server for responding to the SEALDD context push request.

Table 9.6.3.2-1: SEALDD context push response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M (NOTE) | Success or failure. |
| New SEALDD server endpoint | O | The endpoint (IP address and port) on the new SEALDD for SEALDD-UU user plane communication. Applicable for successful result. |
| NOTE: The result is failed if the new SEALDD server rejects the relocation for SEALDD client. |  |  |

* * * Next Change * * * *

#### 9.6.3.4	SEALDD context pull response

Table 9.6.3.4-1 describes the information flow from the old SEALDD server to the new SEALDD server for responding to the SEALDD context pull request.

Table 9.6.3.4-1: SEALDD context pull response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-UU Context | O (NOTE) | Identifies the context related to SEALDD-UU connection, which is created upon SEALDD connection establishment. |
| SEALDD-S Context | O (NOTE) | Identifies the context related to SEALDD-S subscription. |
| Transport layer context | O (NOTE) | Identifies the context related to Transport layer (e.g. TCP/TLS/QUIC) for SEALDD-UU user plane communication. |
| SEALDD policy | O (NOTE) | Indicates the VAL server configured SEALDD policy. |
| NOTE: These IEs are applicable when the result is success. |  |  |

* * * END of Change * * * *