3GPP TSG-SA WG6 Meeting #63	S6-244168

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Samsung

Title:	Pseudo-CR on enhancements to ML model management procedure

Spec:	3GPP TS 23.482 (v0.2.0)

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath Ramamoorthy (arun.prasath@samsung.com)

1. Introduction

This pCR enhances the ML model management procedure.

2. Reason for Change

As specified in the study, if the required number of AIMLE clients are not returned in the response, the AIMLE clinet may interact with another AIMLE server. Need to add such clarification.

- Added pre-condition for model store and discovery procedure. AIMLE server stores the applicatin specific AIML model upon receiving the request form the VAL server.

- Updated text in discovery porocedure to clarify discoery procedure.

- It is not clear as to how to get consent from the model owner if list of allowed vendors are not present. So, added EN to specify the same.

- Model address where model is actually stored shold be part of response message and not request message.

- Model profile is given by the requestor – no need to send it back.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 (v0.2.0).

* * * First Change * * * *

### 8.11.2	ML model information storage

Figure 8.11.2-1 illustrates the procedure of ML model information storage.

Pre-condition:

1.	The AIMLE server has either received application specific model details from AIML consumer or produced analytics model.

Figure 8.11.2-1: ML model information storage

1.	The AIMLE server sends a ML model information storage request to the ML repository to store an ML model. The ML model included in the request can be one trained by the ML repository consumer or its information can be provisioned to the ML repository consumer. The request contains information elements as described in Table 8.11.4.1-1.

NOTE 1:	The security requirements within the ML model storage requirements are out of scope.

2.	Upon receiving the ML model information storage request, the ML repository verifies if the AIMLE server is authorized to store the ML model identified by Analytics ID and/or list of the allowed vendors provided within the ML model profile attribute. If the AIMLE server is authorized, the ML repository processes the request and records information of the ML model (e.g., by creating a ML model profile). The ML repository sends an ML model information storage response to the AIMLE server with an identifier of the created ML model profile.

### 8.11.3	ML model information discovery

Figure 8.11.3-1 illustrates the procedure of ML model information discovery.

Pre-condition:

1.	AIMLE consumer has requested to discover ML model.

Figure 8.11.3-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. The request contains information elements as described in Table 8.11.4.3-1.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the requestor is authorized to discover the ML model(s) identified by the ML model ID. Further, it verifies whether the rquestor is present in the list of allowed vendors or not. If the ML repository consumer is authorized, the ML repository processes the request and discovers the model based on ML model ID, Analytics ID, Base Model ID, and/or ML model interoperability information.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model. Optionally, the ML repository can initiate a model transfer process to the consumer.

NOTE 1:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details is indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enabler consumer can rely on model repository to get the model details.

NOTE 2:	The model transfer process mentioned in step 2 can use the AI/ML data transfer policies.

Editor’s note: If List of allowed vendors is not present then how to get consent from model owner to share the model to the requestor is FFS.

### 8.11.4	Information flows

#### 8.11.4.1	ML model information storage request

Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information storage.

Table 8.11.4.1-1: ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| ML model | O (NOTE 1) | Provides the trained ML model. |
| ML model information | O (NOTE 1) | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| NOTE 1: At least one of these information elements shall be provided. |  |  |

Table 8.11.4.1-2: ML model information

| Information element | Status | Description |
| --- | --- | --- |
| ML model identifier | M | An identifier for the ML model |
| ML model address | O | Represents the ML model address that can be used by Model repository to download the ML model. |
| Analytics ID | O | Represents analytics ID for which the model can be used. |
| VAL service ID(s) | O | Identify the VAL service ID(s). |
| Domain | O | Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location predication, etc.). |
| Discoverable | O | Indicates whether model is allowed to be discovered by other applications/vendors |
| List of allowed vendors | O (NOTE 1) | Indicates which vendors that are allowed to retrieve the ML model and thereby also are interoperable to the model. |
| ML model interoperability information | O (NOTE 1) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| ML Model phase | O (NOTE 1) | Represents the ML model phase, e.g., in training, trained, re-training, deployed. |
| > Observed accuracy | O (NOTE 2) | Provides accuracy of the model (if ML model is in trained phase). |
| > Observed model execution time | O (NOTE 2) | Provides time to execute the model. |
| ML model storage and discovery requirements | O (NOTE 1) | Represents the requirements for the ML repository for the ML model storage and discovery. |
| > Storage duration | O | Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted. |
| > Security and access requirements | O | Represents the information on security requirements for storing the ML model information and the ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs). |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available. |  |  |

#### 8.11.4.2	ML model information storage response

Table 8.11.4.2-1 describes the information flow from the ML repository to the AIMLE server as a response for the ML model information storage request.

Table 8.11.4.2-1: ML model information storage response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |
| ML model address | O | Represents the ML model address that can be used by Model repository to download the ML model. This IE is included when result IE indicates success. |
| ML model identifier | M | An identifier for the ML model. This IE is included when result IE indicates success. |

* * * End of Change * * * *