3GPP TSG-SA WG6 Meeting #63	S6-244183

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Samsung

Title:	Pseudo-CR on enhancements to spatial map management procedures

Spec:	3GPP TS 23.437 (v0.2.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR proposes enhancements to spatial map management procedures.

2. Reason for Change

Following changes are made:

1) Added IE tables for Create Spatial map, Discover Spatial map, Get Spatial map, Update Spatial map and Delete Spatial map procedures.

2) Solution #15 (in TR 23.700-21) was concluded in FS_Metaverse_App study with following conclusion: “Solution #15 supports spatial map augmented with VAL UE. This solution can be considered to merge with solution #8 in normative phase.”

- Based on the conclusion, the contribution is to merge solution #15 of TR 23.700-21 in to “create spatial map” procedure. The contribution considers to augment VAL users into spatial map as extra layer information.

3) As studied in solution #2 and solution # 8 of TR 23.700-21, spatial anchors can be embedded into spatial map. The contribution considers to augment spatial anchors into spatial map as extra layer information.

4) Resolving EN: “Editor’s Note: The enhancements to the result IE to include "in-progress" response will be considered.”

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 (v0.2.0).

* * * First Change * * * *

## 9.3	Spatial map management procedures

### 9.3.1	Create spatial map

Figure 9.3.1-1 depicts the procedure for creating a spatial map. For the request from VAL server or SEAL SM client, as a spatial map consumer, SEAL SM server creates a spatial map. A spatial map can be structured in layers where each layer signifies a specific aspect of the spatial information (e.g. space, objects).

Figure 9.3.1-1: Create spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to create a spatial map. The request includes requestor ID, security credentials, three-dimensional area of interest, information to be included in the spatial map such as access control rules defining which entities are permitted to discover and access the spatial map. The request may include layering information parameters (e.g. layer 1-basic space, layer 2-objects). The request may include augmented layer Information (e.g. VAL users, Spatial anchors, etc.) to be embedded into the spatial map that are available in the area of the interested related to the spatial map.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. The SEAL SM server produces a requested spatial map using the layering information and using the processed sensor data.

a)	If the augmented layer Information indicates to include VAL users, the SEAL SM server fetches the list of VAL UEs in the area of interest related to the spatial map (as specified in the request message), from SEAL LM server using clause 9.3.10 of 3GPP TS 23.434 [4] and/or from NEF as specified in 3GPP TS 23.502 [5]. The SEAL SM server identifies the VAL UEs information related to the VAL service(s) in the request message from the VAL server or SEAL SM client.

b)	If the augmented layer Information indicates to include spatial anchors within area of interest, the SEAL SM server fetches the list of spatial anchors as specified in clause 8.3.4.

NOTE:	For the current release, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of SA6 scope.

3)	The SEAL SM server sends response message to the requestor with produced spatial map ID and if spatial map is already produced the response message includes information which includes three-dimensional space defined by the spatial map, stationary or moving objects with attributes of them. If the SEAL SM server does not have sufficient sensor data to produce spatial map and the spatial map is still being produced, the result indicates "in-progress" response and expected time when the spatial map will be produced and employed for localization services. Otherwise, the response includes an indication of failure and may include a reason for failure. If response indicates "in-progress", the VAL server (or SEAL SM client) subscribes to receive notification when spatial map is ready for the utilization (e.g. for localization service) as specified in clause 9.3.6.

#### 9.3.1.1	Information flows for Create spatial map

Table 9.3.1.1-1 shows the produce spatial map request sent by VAL server (or SEAL SM client) to SEAL SM server to create a spatial map.

Table 9.3.1.1-1: Create spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL user) |
| Security credentials | M | The security credentials of the requestor. |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Requested information | O | Information to be included in the spatial map (e.g. layers information, access control rules) |
| > list of layers | O | Indicates list of all layers to be included in the spatial map. |
| > augmented layer Information | O | List of required augmented layer information to add as extra layer of information. This IE can contain multiple values; Possible values are: - VAL_USERS; - SPATIAL_ANCHORS; |

Editor’s note:	Details about layers is FFS.

Table 9.3.1.1-2 shows the create spatial map response sent by SEAL SM server to VAL server (or SEAL SM client) as the response to the create spatial map request.

Table 9.3.1.1-2: Create spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result (NOTE 1) | M | Result of the producing spatial map (e.g., success, in-progress or fail), including cause of failure if needed |
| List of spatial maps (NOTE 2) | O | Details of the spatial map. Each spatial map contains information as specified in Table 9.3.1.1-3. |
| Expected time | O | Expected time when the spatial map is employed for localization service. |
| Failure cause | O | Indicates reason for the failure. |
| NOTE 1: The result IE contains in-progress when the SEAL SM server does not have sufficient sensor data to produce spatial map. NOTE 2: This IE is mandatory when result IE indicates success or in-progress. |  |  |

Table 9.3.1.1-3: Spatial map information

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID (NOTE 1) | M | ID of the produced spatial map |
| Map information (NOTE 2) | O | Details of the map |
| > Spatial map media | M | Spatial map media as specified by SA4. May contain location from where the media can be downloaded. |
| > area of interest | M | Three-dimentional area covered with spatial map |
| > Update frequency | O | The frequency at which a spatial map is updated. |
| > List of layers | O | Includes list of layers specifying spatial map information |
| >> Layer ID | M | Identification of the layer |
| >> Layer name | O | Name of the layer |
| >> Layer status | O | Indicates whether to hide or show the layer. If this IE is not present, the layer will be shown by default. |
| >> area | O | Area covered by this layer (This area is same or subset of the area of interest of the spatial map). |
| >> List of stationary objects | O | List of stationary objects for the layer (if available). Each object contains information as specified in Table 9.3.1.1-4 |
| >> List of moving objects | O | List of moving objects for the layer (if available). Each object contains information as specified in Table 9.3.1.1-4 |
| >> associated spatial map | O | Detailed spatial map associated with this layer. For example, if the layer is for a particular floor of the building, the detailed spatial map of that particular floor can be associated with this layer. |
| NOTE 1: This IE is mandatory when result IE indicates success or in-progress. NOTE 2: This IE is added only when result IE indicates success. |  |  |

Table 9.3.1.1-4: Object details

| Information element | Status | Description |
| --- | --- | --- |
| Object Id | O | Identify of the object |
| Position | O | Current three-dimensional position of the object in the spatial map. For moving objects, it includes current position. |
| Time (NOTE 1) | O | For moving object, time when last position was updated |
| Speed (NOTE 1) | O | Speed of the object |
| Direction (NOTE 1) | O | Direction of the object in 3D space |
| NOTE 1: This IE may included for moving objects only. This IE is not included for stationary objects. |  |  |

### 9.3.2	Discover spatial map

Figure 9.3.2-1 depicts the procedure for an authorized spatial map consumer (VAL server or SEAL SM client) to discover spatial map(s). The service is provided by SEAL SM server.

Figure 9.3.2-1: Discover spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to discover spatial map(s). The request includes requestor ID, security credentials, three-dimensional area of interest or localization information to discover spatial maps in the area, optionally discovery filters, i.e., a set of characteristics to search the matching spatial maps when multiple spatial maps are allowed in the area of interest.

NOTE:	How the consumer identifies which three-dimensional area of interest to get the spatial map is not in the scope of this document.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then the server finds spatial map(s) which matches requested discovery requirements.

3)	The SEAL SM server sends the discover spatial map response message to the requestor with spatial map ID(s) when matching spatial map is found, otherwise it sends failure indication with reason.

#### 9.3.2.1	Information flows for Discover spatial map

Table 9.3.2.1-1 shows the discover spatial map request sent by VAL server (or SEAL SM client) to SEAL SM server to discover spatial maps.

Table 9.3.2.1-1: Discover spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User) |
| Security credentials | M | The security credentials of the requestor. |
| Filtering criteria | M | Criteria to discover spatial map(s) |
| > Area of interest | O | Three-dimensional area information to discover spatial maps |
| > Localization information | O | A known location to discover spatial maps in the three dimensional space, including an orientation |
| > Layer ID | O | Identification of the layer |

Table 9.3.2.1-2 shows the discover spatial map response sent by SEAL SM server to VAL server (or SEAL SM client) as the response to the discover spatial map request.

Table 7.8.4.3-2: Discover spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| List of spatial maps (NOTE 2) | O | Details of the spatial map. Each spatial map contains information as specified in Table 9.3.1.1-3. |
| Failure cause | O | Indicates reason for the failure. |

### 9.3.3	Get spatial map information

Figure 9.3.3-1 depicts the procedure for an authorized spatial map consumer (VAL server or SEAL SM client) to get the spatial map. The service is provided by SEAL SM server.

Figure 9.3.2-1: Get spatial map information procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to get the spatial map information. The request includes requestor ID, security credentials, spatial map ID to get the information for it.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then the server collects the requested spatial map information.

3)	The SEAL SM server sends the get spatial map information response message to the requestor with requested spatial map information when successful, otherwise it sends failure indication with reason.

#### 9.3.3.1	Information flows for Get spatial map information

Table 9.3.3.1-1 shows the get spatial map request sent by VAL server (or SEAL SM client) to SEAL SM server to get spatial maps.

Table 9.3.3.1-1: Get spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User) |
| Security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | Identity of the spatial map |
| > Layer visibility criteria | O | Indicates which layer to show or hide per spatial map. If this IE is not present, all layers will be shown. |
| >> Layer IDs | O | Layer ID of the layer to hide |

Table 9.3.3.1-2 shows the get spatial map response sent by SEAL SM server to VAL server (or SEAL SM client) as the response to the get spatial map request.

Table 7.8.4.3-2: Get spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| Spatial map information | O | Details of the spatial map. It contains information as specified in Table 9.3.1.1-3. |
| Failure cause | O | Indicates reason for the failure. |

### 9.3.4	Update spatial map

Figure 9.3.4-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SM server updates the spatial map as requested.

Figure 9.3.4-1: Update spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to update a spatial map. The request includes requestor ID, security credentials, spatial map ID, information on what to update such as updated access control rules and updated spatial map coverage area, etc.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client) and validates the request. The SEAL SM server updates the spatial map as requested.

3)	The SEAL SSM server sends response message to the requestor with result of the request and updated spatial map information with the spatial map ID.

#### 9.3.4.1	Information flows for Update spatial map

Table 9.3.4.1-1 shows the update spatial map request sent by VAL server (or SEAL SM client) to SEAL SM server to update a spatial map.

Table 9.3.4.1-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL user) |
| Security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | Identity of the spatial map |
| Area of interest | O | Three-dimensional area information to produce the spatial map |
| Requested information | O | Information to be included in the spatial map (e.g. layers information, access control rules) |
| > layers to add | O | Specifies new layers to add into the spatial map |
| >> list of Layers | O | Indicates list of all layers to be included in the spatial map. |
| >> augmented layer Information | O | List of required augmented layer Information to add as extra layer of information. It can contain multiple values; Possible values are: VAL_USERS; SPATIAL_ANCHORS; |
| > layers to remove | O | Specifies existing layers to remove from the spatial map |
| >> Layer ID | O | Identity of the layer to remove |

Table 9.3.4.1-2 shows the update spatial map response sent by SEAL SM server to VAL server (or SEAL SM client) as the response to the update spatial map request.

Table 9.3.4.1-2: Update spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result (NOTE 1) | M | Result of the updating spatial map (e.g., success, in-progress or fail), including cause of failure if needed |
| Spatial map information | O | Details of the spatial map. It contains information as specified in Table 9.3.1.1-3. |
| Failure cause | O | Indicates reason for the failure. |
| NOTE 1: The result IE contains in-progress when the SEAL SM server does not have sufficient sensor data to produce spatial map. |  |  |

### 9.3.5	Delete spatial map

Figure 9.3.5-1 depicts the procedure to delete the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SM server deletes the spatial map as requested.

Figure 9.3.5-1: Delete spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to delete the spatial map. The request includes requestor ID, security credentials, a spatial map ID to delete and optionally the reason to delete.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client) and validates the request. The SEAL SM server deletes the spatial map as requested.

3)	The SEAL SM server sends response message to the requestor with the result of success or failure.

#### 9.3.5.1	Information flows for Delete spatial map

Table 9.3.5.1-1 shows the delete spatial map request sent by VAL server (or SEAL SM client) to SEAL SM server to delete spatial maps.

Table 9.3.5.1-1: Delete spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User) |
| Security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | Identity of the spatial map to delete |

Table 9.3.5.1-2 shows the delete spatial map response sent by SEAL SM server to VAL server (or SEAL SM client) as the response to the delete spatial map request.

Table 7.8.4.3-2: Delete spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| Failure cause | O | Indicates reason for the failure. |

* * * End of Change * * * *