3GPP TSG-SA6 Meeting #63	S6-244202

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0205 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requesting of bulk resource owner authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (Ulanqab) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Solution#12 & #13 of 3GPP TR 23.700-22 were concluded to be considered for the normative work for addressing the communication between resource owner and Authorization function (CCF) over CAPIF-8. This CR proposes the implement them 3GPP TS 23.222. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a new clause (8.x) regarding obtaining bulk authorization combining both solution#12 & #13. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The required number of interactions between the CCF (Authorization Function) and resource owner via the ROF over CAPIF-8 will be needlessly high. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.x	API invoker obtaining bulk authorization from a resource owner

### 8.x.1	General

Using the procedure described in clause 8.31 the CAPIF may authorize an API invoker to invoke a service API based on the authorization information from the resource owner given before the API invocation. This clause describes an extension to that procedure to enable the CAPIF to obtain authorization information from the resource owner relating to multiple API invokers and service APIs in a single interaction.

Clause 8.x.3 shows the procedure for obtaining bulk authorization information from a resource owner via their resource owner function.

### 8.x.2	Information flows

Editor’s note:	The security aspects of this procedure are to be specified in TS 33.122 [12].

### 8.x.3	Procedure

Figure 8.x.3-1 illustrates the procedure for API invoker obtaining bulk authorization from resource owner.

Pre-conditions:

1.	The resource owner function can communicate with the CCF (Authorization function).

2.	Access to an API exposing function offered service API requires obtaining authorization from a resource owner.

3. 	The API invoker is successfully onboarded as described in clause 8.1.

Figure 8.x.3-1: Procedure for API invoker obtaining bulk authorization from resource owner

1.	The CCF is configured for requesting bulk authorization from the resource owner, see table 8.x.3-1.

2.	An API invoker sends obtain service API authorization request to the CAPIF core function for obtaining permission to access a service API by including its API invoker identity information and any information required for authentication of the API invoker. The API invoker includes in the request information about the purpose and the targeted resources and operations to be performed on those resources.

3.	The CCF validates the authentication of the API invoker (using authentication information) and checks whether it is permitted to access the requested service API by checking whether a consent record is already in place for that API invoker. If the consent for accessing the requested resources for the signalled purpose is already granted, the procedure continues with step 5.

NOTE 1:	Whether the CCF is allowed to retain consent records depends on local regulations.

4. 	If authorization information is not available regarding whether the API invoker is permitted to access the requested service API (i.e., there is no applicable consent record available at the CCF) the bulk resource owner consent configuration (table 8.x.3-1) is assessed.

If a consent trigger criteria duration has been specified as part of the bulk resource owner consent configuration, interactions with the resource owner are only initiated after the duration timer has expired. The resource owner specific duration timer is started by the first API invoker for which a consent record is not available for accessing the requested service API. It is reset once an interaction with the resource owner is initiated. Whilst the duration timer is running, further obtain service API authorization requests may be received from different API invokers for the same service API and cached. Such requests do not reset the timer.

If no duration timer is specified or the duration timer expires, the authorization function determines the authorization by interacting with the resource owner via the ROF. Based on the configured bulk resource owner consent configuration at the CCF and any cached “obtain service API authorization requests”, the scope of the request towards the ROF can be expanded to include additional API invokers and or service APIs based on the assumption that there is an association between the requesting API invoker and the API exposing functions and API invokers listed in the CCF configuration.

Editor’s Note: The detailed procedure to obtain the resource owner's authorization information is in scope of SA3. However, it is recommended that the resource owner is provided with at least the option to only grant authorization for only the API invoker initiating the authorization request and then only for the specific service being requested.

NOTE 2:	The details of the interaction between RO and ROF for capturing the consent are considered out-of-scope of 3GPP.

5.	Based on the RO provided consent, the CCF sends an API invoker specific service API authorization response to the API invoker that initially triggered the bulk resource owner consent information requested and to each API invoker for which a obtain service API authorization request was received (for the same service API) whilst the bulk resource owner consent configuration duration timer was running, i.e., the cached requests.

6.	An API invoker that receives the service API authorization response can send service API invocation requests to the API exposing function with their API invoker specific resource owner authorization information.

7.	The API exposing function will send a service API invocation response once it has successfully checked that the API invoker is authorized to invoke that service API based on the authorization information.

Table 8.x.3-1: Bulk resource owner consent configuration

| Information element | Status | Description |
| --- | --- | --- |
| List of API invoker associations (NOTE) | O | Lists of API invokers with their associated service APIs |
| > API invoker identity information list | M | List of API invoker identities |
| > Service API identification list | M | List of AEF service API identities. |
| List of consent trigger criteria (NOTE) | O | Criteria for triggering requesting of resource owner authorization |
| > Duration | O | Initiate request towards RO via ROF after the given duration (in seconds) after receiving a obtain service authorization request from an API invoker |
| > Resource owner identifier | O | Can only be included if a duration is included. Indicates that the consent trigger criteria is that to be applied to the specific resource owner identified by their GPSI, i.e., no other consent trigger criteria shall be applied for that resource owner. |
| NOTE: At least one of these attributes shall be present |  |  |

* * * End of Changes * * * *