3GPP TSG-SA6 Meeting #63	S6-244240

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0337 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for sidelink positioning management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A new feature is added to SEAL LMS capabilties based on concluded solution #7 of TR 23.700-72. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper introduces the new capabilities to clause 9.3.x about the SL positioning enablement. This includes the procedure and information flows for the new feature based on concluded solution #7 of TR 23.700-72. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The capability based on the concluded solution will not be added. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses: 9.3.x (new), 9.3.x.1 (new). 9.3.2.X (new), 9.3.2.Y (new), 9.3.2.Z (new), 9.3.2.XY (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.3.x	Support for sidelink positioning / ranging management

#### 9.3.x.1	Procedure of SL positioning /ranging management service

The procedure provides a mechanism for configuring and provisioning information on the entities to be used and their responsibilities for a SL positioning / ranging service. In this procedure, an SL positioning enabler capability at LMS is authorized / delegated by SL positioning server to support the provisioning/delivery of ranging services in an area.

Figure 9.3.x.1-1 shows a high-level procedure with the details.

Figure 9.3.x.1-1: Support for SL positioning procedure

1.	The VAL server (SL Positioning / Ranging Server) sends a subscription request to SEAL LMS to provision and manage ranging operation and configurations. This includes monitoring, discovering of VAL UEs assisting the operation, and delivering the configuration for a target service area, or for a target set of VAL UEs to serve as reference and target.

2.	The SEAL LMS authorizes the VAL subscription request.

3.	The SEAL LMS sends a subscription response to the requestor VAL Server.

4.	The SEAL LMS discovers all the SEAL LMCs in the target service area, and categorizes them by their capabilities as target UE, Reference UEs, or even Located UEs (if Reference UE location is known or for out of coverage scenarios). The identification of UEs in a target area happens via existing SEAL LMS procedures (as in clauses 9.3.10 and 9.3.12).

5.	The SEAL LMS after getting all UE information at the target area, calculates the relative distances among them, and may request/receive HDMaps (e.g., from other VAL servers) for the target area.

Then, SEAL LMS evaluates the need of reference UE (based the information in step 4), and if reference UE is needed (e.g. in NLoS situation), the SEAL LMS selects the appropriate VAL UE to serve as reference and configures the reference VAL UE with configuration information (thresholds, reporting configurations).

NOTE: The selection of UE as reference UEs can be based on their capabilities and the selection criteria specified by the VAL server.

6.	The SEAL LMS sends a SL positioning management notify message to the VAL server to indicate the reference UE and the feasibility of SL positioning based on step 5.

7a.	The SEAL LMS configures the SEAL LMCs with all the configuration information based on step 5 and in particular the expected responsibility in relation ranging/SL positioning service.

7b.	Optionally, the SEAL LMC interacts with its VAL client at the involved VAL UE (of client, reference, target) to send the SL positioning configuration information.

* * * Second Change * * * *

#### 9.3.2.X	SL positioning management subscription request

Table 9.3.2.X-1 describes the information flow from the VAL server to the location management server for SL positioning management subscription request.

Table 9.3.2.X-1: SL positioning management subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server. |
| VAL service ID | M | Identity of the VAL service (SL positioning service) for which the location information is subscribed. |
| VAL service area | M | The geographical area /zone or topological area (e.g. cells) for which the subscription applies. |
| VAL UE/user Identities list | M | List of VAL users or VAL UEs participating at the SL positioning service (reference and target VAL UEs). |
| VAL preference / policies | O | The VAL policies or preferences related to the communication among the VAL UEs (e.g. transmission mode, direct vs network-assisted SL positioning). Such policies may also include the SL positioning parameters to be reported (e.g. distance, direction or both), their frequency, and the permission/exposure level to the VAL UEs in the target service area. |
| VAL UE SL positioning capabilities | M | The capabilities of the VAL UEs of interest (if provided) with respect to ranging/SL positioning capabilities. |
| List of Client UE IDs and addresses | O | The identities of the client UEs which are allowed to get ranging information for other UEs. |
| > capability exposure level | O | The permissions / exposure level for the client UEs (what is allowed to be exposed to them and if some anonymization is needed). |
| Reporting configuration | M | It indicates the threshold and interval time between consecutive reports. |
| > Triggering criteria | O | If reporting is expected to happen based on certain conditions in future instance, this IE provides information on the conditions to trigger some reporting (e.g. high load scenarios, feasibility of direct links, high number of re-Tx). |

#### 9.3.2.Y	SL positioning management subscription response

Table 9.3.2.Y-1 describes the information flow from the location management server to the VAL server for SL positioning management subscription response.

Table 9.3.2.Y-1: SL positioning management subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |
| Subscription identity | O (see NOTE) | If subscription is successful, identifies the subscription |
| NOTE: This IE shall be present if subscription status is set to success. |  |  |

#### 9.3.2.Z	SL positioning management notification

Table 9.3.2.Z-1 describes the information flow from the location management server to the VAL server.

Table 9.3.2.Z-1: SL positioning management notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription. |
| Ranging configuration parameters | M | The configuration information for the SL positioning service. |
| > Reference and target UE ID and address | M | Identifies the list of UEs to serve as reference and target UEs. |
| >> VAL UE ranging capabilities | M | The capabilities of the VAL UEs of interest with respect to ranging/SL positioning capabilities. This may include the reserved resources and VAL UE priorities for ranging service. |
| >> SL configuration policy parameters | O | The access or transmission policy parameters for the SL positioning service. |
| > List of Client UE IDs and addresses | M | The identities of the client UEs which are allowed to get ranging information for other UEs. |

#### 9.3.2.XY	SL positioning configuration command

Table 9.3.2.XY-1 describes the information flow from the location management server to the location management clients or from LMC to VAL client.

Table 9.3.2.XY-1: SL positioning configuration command

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requestor SEAL server or the subscriber VAL server |
| VAL service ID | M | Identity of the VAL service (SL positioning service) |
| VAL service area | M | The geographical area /zone or topological area (e.g. cells) for which the subscription applies. |
| VAL UE/user Identities list | M | List of VAL users or VAL UEs participating at the SL positioning service (reference and target VAL UEs). |
| Ranging configuration parameters | M | The configuration information for the SL positioning service. |
| > Reference and target UE ID and address | M | Identifies the list of UEs to serve as reference and target UEs. |
| >> VAL UE ranging capabilities | M | The capabilities of the VAL UEs of interest with respect to ranging/SL positioning capabilities. This may include the reserved resources and VAL UE priorities for ranging service. |
| >> SL configuration policy parameters | O | The access or transmission policy parameters for the SL positioning service. |
| > List of Client UE IDs and addresses | M | The identities of the client UEs which are allowed to get ranging information for other UEs. |
| >> capability exposure level | O | The permissions / exposure level for the client UEs (what is allowed to be exposed to them and if some anonymization is needed). |
| > allowable transmission modes | O | Whether unicast or groupcast or broadcast is expected for the SL positioning service. |
| Reporting configuration | M | Indicates the threshold and interval time between consecutive reports. |
| >Triggering criteria | O | If reporting is expected to happen based on certain conditions in future instance, this IE provides information on the conditions to trigger some reporting (e.g. high load scenarios, feasibility of direct links, high number of re-Tx). |