3GPP TSG-SA WG6 Meeting #63	S6-244243

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Lenovo

Title:	Support for FL group management

Spec:	3GPP TS 23.482 v0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper provides a new feature on FL member grouping based on the concluded solution in TR 23.700-82.

2. Reason for Change

This paper provides a new feature on FL member grouping based on the concluded solution in TR 23.700-82. This paper introduces the new feature in clause 6.x and describes the procedure and information flows in clause 8.x.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.2.0.

* * * First Change * * * *

## 6.x	Support for FL member grouping

This functionality covers the AIMLE capability to enable the group management of the entities serving as FL clients at the application enablement layer. Such group management is about the creation, monitoring and update of the FL member groups based on the AI/ML operations, which are based on 1) the analytics event/service by ADAES or 2) the VAL requirement for FL support services.

* * * Next Change * * * *

## 8.x	Support for FL member grouping

### 8.x.1	General

This clause provides the procedure for the grouping of the FL members using AIMLE. Such grouping for the given FL process, applicable to a specific VAL request or ML model ID or ADAE analytics ID.

### 8.x.2	Procedure

In this procedure, the AIMLE support capability is described for grouping the FL members, where the grouping is tailored to a specific ML task (VAL triggered task or analytics event/ID).

Figure 8.x.2-1 illustrates the procedure for supporting the FL member grouping.

Pre-conditions:

1.	VAL server is connected to AIMLE Server.

2.	The candidate/selected FL member has registered to the FL member registry based on the capability in clause 8.4.

Figure 8.x.2-1 Procedure for FL member grouping

1.	The VAL server sends an FL process support request to the AIMLE server for supporting an FL process.

2.	The AIMLE Server based on the request, determines the need for creating a group consisting of the needed FL members for a given ML task (i.e., an ML model training/inference job ID). The FL aggregator can be either the VAL server or the AIMLE server himself, based on the request.

3.	The AIMLE Server fetches the FL candidate or available FL members for the given ML task (i.e., an ML model training/inference job ID) from the ML repository.

4.	The AIMLE Server determines by configuring the group based on the selected FL members by the FL server/aggregator. The criteria for determining the group members can be the capabilities of the FL participants, or whether the candidate participants are fixed or mobile nodes and their availabilities, the proximity of the participants among them.

5.	The AIMLE Server notifies the candidate FL members (including AIMLE client if the candidate is VAL UEs) about the group ID and the group member identities for the ML model ID / analytics ID (based on the request in step 1.

6.	The AIMLE server sends a FL process support response to the VAL request indicating the group creation and the group information.

### 8.x.3	Information flows

#### 8.x.3.1	FL process support request

Table 8.x.3.1-1 shows the request sent by the VAL server to AIMLE server for the FL member grouping procedure.

Table 8.x.3.1-1: FL process support request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| VAL service identity | M (NOTE) | The identity of the VAL service for which the request applies. |
| ML model ID | M (NOTE) | The model ID for which the request applies |
| ADAE analytics ID | M (NOTE) | The ADAES analytics ID (as specified in TS 23.436) for which the FL grouping is to be used, in case when FL process is used for a given analytics task. |
| ML task information | O | Information related to the ML task / job for which the FL grouping is used. Such task can be an FL training task or FL inference task. This information may include an ML task ID which may be an FL process ID or correlation ID. |
| ML model profile | O | The ML model profile for which the FL grouping is to be used. |
| List of candidate/selected FL member IDs | O | The list of candidate or selected FL member identities (if known by the VAL server) which are to be used in the grouping. If the FL member is a VAL UE, this is equivalent to the VAL UE ID. |
| > FL member type | O | The type of FL members (FL client, FL server) |
| > FL member status | O | The status (selected, candidate) of the FL member. |
| NOTE: At least one of these information elements shall be present |  |  |

#### 8.x.3.2	FL process support response

Table 8.x.3.2-1 shows the response sent by the AIMLE server to the VAL server for the FL member grouping procedure.

Table 8.x.3.2-1: FL process support response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the FL process support request was successful. |
| > FL group identifier(s) | O | Identifies the AIMLE-created FL group for the FL process and ML task (FL training or inference). |
| > List of FL member IDs / addresses | O | Provides the ID and address of the FL members which are part of the FL group. |
| >> FL member information | O | Information on the FL members such as availability, constraints, role/type. |
| Failure response | O (NOTE) | Indicates that the FL process support request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: At least one of these information elements shall be present |  |  |

#### 8.x.3.3	FL grouping indication

Table 8.x.3.3-1 shows the notification sent by the AIMLE server to the FL members (AIMLE clients, VAL servers) for the FL member grouping procedure.

Table 8.x.3.3-1: FL grouping indication

| Information element | Status | Description |
| --- | --- | --- |
| VAL service identity | M (NOTE) | The identity of the VAL service for which the grouping indication applies. |
| ML model ID | M (NOTE) | The model ID for which the indication applies |
| ADAE analytics ID | M (NOTE) | The ADAES analytics ID (as specified in TS 23.436) for which the FL grouping is to be used, in case when FL process is used for a given analytics task. |
| FL group identifier(s) | M | Identifies the AIMLE-created FL group for the FL process. |
| > List of FL member IDs / addresses | O | Provides the ID and address of the FL members which are part of the FL group. |
| >> FL member information | O | Information on the FL members such as availability, constraints, role/type. |
| NOTE: At least one of these information elements shall be present |  |  |

* * * End of Change * * * *