3GPP TSG-SA WG6 Meeting #63	S6-244252

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-21 | CR | 0001 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for permission control of digital assets |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | 2024-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the descriptions in clause 6.2 and clause 7.7, DACM has avatar management functions, including: - Avatar profile creation; - Avatar upload; - Avatar linking to user/subscriber; - Avatar download; - Avatar update/modification; - Avatar delete; which leads to the inference that DACM has the same management functions for digital assets. Therefore, the core function of DACM is to manage digital assets and ensure the security of access to these assets. The types of digital assets are diverse, and some of them is non-sensitive. Considering the need to promote non-sensitive information digital assets, DAPM is added to provide related services. The key function of DAPM is to publish digital assets and provide subscription digital assets service, assisting DACM in achieving distribution of permissions to multiple consumers. These functionalities have not been implemented by DACM. The reasons for the separate establishment of DACM and DAPM are as follows: 1.The architecture is more flexible, allowing different roles to take on the construction of different functionalities. DAPM may not have the capability to store digital assets, but it can provide channels for the subscription of digital assets. DAPM assists DACM in completing the authorization and distribution permissions. 2.It is advantageous for the evolution of future service. Different operators may each construct a DAPM. Digital assets stored in DACM can be simultaneously publicized across multiple DAPMs, facilitating the promotion and utilization of digital assets. 3.As a channel of distribution, DAPM may connet to different DACM serving different metaverses. User may discover and get permission of usage right, e.g. download, of digital assets from different metaverses from the DAPM. The DAPM may expand the user of the digital asset and help the cross-metaverses access. In summary, to meet the needs for sharing and widespread use of digital assets, while achieving secure management of digital assets, DACM and DAPM work together to achieve the distribution and control of the digital asset usage rights. This contribution provides a architecture for permission control of digital assets, describs the functions and services of DAPM. Based on the existing Avatar download procedures, a solution is provided, including procedures of publishing, subscription, and download digital assets. NOTE: The skeleton of TS23.438 is not available yet, therefore this architecture update and related procedures are proposed to TR23.700-21 firstly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution updates architecture option#4 to remove the Editor Note and provides solution to solve KI#9 (Permission control of digital assets). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Permission control of digital assets in KI#9 will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.2.1, 6.4.4(new), 6.4.5(new), 7.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.4	Option #4: Architecture to support Metaverse services with permission control

6.4.1	Application enablement architecture

There are many types of digital assets. Some digital assets may not contain sensitive information. After being created, they are expected to be widely used. Although the management of API through CAPIF realizes the management of resource usage rights. The current structure does not consider some special scenarios related to digital assets. for example the transfer of ownership or the transfer of ownership during the purchase, the expiration and renewal of digital certificate for usage right etc. are not supported by current version of CAPIF yet. In this case, we must consider both the security of digital assets and sharing of digital asset. Therefore, an architecture is proposed to support permission control of digital assets.

Figure 6.4.1-1 illustrates a potential architecture to support Metaverse services.

Figure 6.4.1-1: Architecture to support Metaverse services

DAPM function provides functionality to the VAL server(s) over the PM-V reference point. DAMP function communicates with A-DACM function over CM-PM reference point. The DAPM function communicate with the UE DAPM function over the PM-C reference point. DAPM supports to provide the service by using CAPIF architecture.

Editor’s Note: Whether DAPM is a new functionality or enhanced other SEAL/CAPIF server is FFS.

Figure 6.4.1-2: Service-based architecture to support Metaverse services

The DAPM function exposes the APIs over the Sdapm interface to the VAL layer and SEAL layer. The VAL functions (VAL servers) and the other SEAL servers consume the services by interacting on the Sdapm interface of the A-DAPM function.

NOTE:	In a certain implementation, the A-DAPM can be implemented separately or implemented together with a SEAL function.

* * * Next Change * * * *

6.4.2	Functional Elements

6.4.2.1	DAPM function

The DAPM function provides the services to VAL layer for the permission management of multiple operation rights of digital assets. The main capabilities as following:

-	Publish the digital asset;

-	Create, store and search subscription records of digital assets;

-	Verify the authenticity and validity of the subscription records;

* * * Next Change * * * *

6.4.4	Service-based interface

6.4.4.1	Sdapm

This interface supports the interactions between VAL function (VAL servers) and DAPM function. The VAL function consumes the services supported by the A-DAPM function as specified in clause 6.4.2.1.

6.4.5	Services

6.4.5.1	DAPM services

The services provided by DAPM function are as following:

-	obtain owner’s commision to publish and promote the digital asset;

-	create, store, search, validate the subscription records of digital asset.

* * * Next Change * * * *

7.x	Solution #x: Support for permission control of digital assets

7.x.1	Solution description

This solution maps to KI#9, achieves permission control on digital assets in the case of digital assets sharing. VAL user uses VAL server to upload a digital asset to A-DACM function. DAPM is to publish digital assets and provide subscription digital assets service. DACM and DAPM work together to achieve the permision distribution. The architectural impacts and procedures are described in clause 7.x.2 and clause 7.x.3 respectively.

Note: The scope of the permission in this paper is download permission.

7.x.2	Architecture Impacts

The solution utilizes the architecture specified in clause 6.4, Option#4. In the architecture DAPM function exposes the APIs to the Vertical Application Layer for service providing.

7.x.3	Procedures

7.X.3.1	Digital asset publishing

Since not all digital assets have publishing requirements, owners need to initiate and upload the digital asset information to DAPM, and make commission to DAPM to promote.

Figure 7.x.3.1-1 illustrates the high level flow for digital asset publishing procedure.

Figure 7.x.3.1-1: Procedure for digital asset publishing

1.	The owner using VAL server or DAPM client sends the Publish Asset Request to DAPM function with the details such as asset ID, asset usage policies (e.g. how to use).

2.	The DAPM function checks the authorization of the VAL server to perform the operation. If successful, then DAPM function publishes the asset.

3.	The DAPM function sends a Publish Asset Response to the VAL server indicating success or failure of the operation.

7.X.3.2	Digital asset subscription

Figure 7.x.3.2-1 illustrates the high level flow for digital asset subscription procedure.

Precondition: The subscriber discovered the digital asset on DAPM and want to download.

Figure 7.x.3.2-1: Procedure for digital asset subscription

1.	The comsumer discovers the digital asset on the DAPM funtion and using VAL server sends the Subscription Request to DAPM function, including digital asset ID, subscriber ID(e.g. GPSI/External ID of the UE etc).

2.	DAPM performs the nessensary authentication and authorization precedures, and processes the subscription request. The DAPM function stores the subscription record. The information of the record includes subscriber ID, digital asset ID, expiration date.

3.	The DAPM function sends Subscription Response to the VAL server indicating success or failure of the operation, including subcription record identifier(e.g. subcription record ID or token of the subcription record).

7.x.3.3	Digital asset download ( enhance Avatar download precedure)

Figure 7.x.3.3-1 illustrates the high level flow for avatar download procedure.

Pre-condition: the consumer has subscribed the digital asset on DAPM and obtain the subcription record identifier.

Figure 7.x.3.3-1: Procedure for digital asset download

1.	Follow step 1 in clause 7.7.1.5 procedure for avatar download. The VAL server sends a Download Avatar Request to the A-DACM function with the Avatar ID(digital asset ID), GPSI/External ID of the UE, Filter/Adaptation information (e.g. VAL information, media adaptations), subcription record identifier.

2.	If the  subcription record identifier is included in the Download Avatar Request, the A-DACM sends Subscription Confirmation Resquest to DAPM function, including subcription record identifier. Otherwise the procedure in clause 7.7.1.5 applies.

3.	DAPM verify the authenticity and validaty of the subscription. DAPM sends Order Subscription Response to A-DACM, including result for whether the subscription is valid, digital asset ID of the valid subscription, subscriber ID of the valid subscription(if the order is valid).

4.	The A-DACM function authorizates the VAL server to perform the operation, based on digital asset ID of the valid subscription, subscriber ID of the valid subscription, the Avatar ID, GPSI/External ID of the UE within the request etc. After authorization, the A-DACM function performs the media adaptation as per the additional filters/adaptation information in the request on the avatar object/media. The adapted avatar media/object is used in the further response to the VAL server.

5.	Same with step 3 in clause 7.7.1.5 procedure for avatar download. The A-DACM function sends a response to the VAL server indicating success or failure of the operation. If success, the avatar object/media is included in the response.

* * * End of Change * * * *