3GPP TSG-SA WG6 Meeting #62	S6-244284

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0237 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the ENs in 23.281 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are still several Editor’s note in the R18 23.281 to be cleanup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resove the editor’s NOTE per discussion paper S6-244305. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s Notes in frozen specifications impact the quality of the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.3.8, 7.1.2.3.1.1.4, 7.2.3.4.2, 7.2.3.5.2, 7.2.3.5.3, 7.5.2.4, 7.7.2.8, 7.16.3, 7.19.2.3, 7.19.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The EN in cl. 7.19.3.1.3 23.281 v18.8.0 is already remove by other CR. So in R19 mirror, the cl. 7.19.3.1.3 is not changed. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ 1st changes *************************/

6.2.2.3.8	MC gateway server

The MC gateway server provides support for MCVideo interconnection services with a partner MCVideo system in a different trust domain whilst providing topology hiding. It acts as a proxy for one or more MCVideo servers in the partner MCVideo system without needing to expose the MCVideo servers in the primary MCVideo system outside the trusted domain of the primary MCVideo system. It may be a role of the MCVideo server described in subclause 6.2.2.3.2 of the present document.

The MC gateway server is responsible for relaying call control and floor control signalling messages, and media between MCVideo servers within the MCVideo system and the interconnected MCVideo system.

# /************************ Next changes *************************/

7.1.2.3.1.1.4	Late entry pre-arranged group call

Procedures in figure 7.1.2.3.1.1.4-1 are the signalling control plane procedures for the MCVideo server requesting a newly affiliated member or a member coming back from out of coverage to join an ongoing MCVideo group call.

Pre-conditions:

1.	MCVideo group is previously defined on the group management server with MCVideo users affiliated to that group. All members of the group belong to the same MC system.

2.	It is assumed that MCVideo users on MCVideo client 2 to MCVideo client n are on an ongoing call.

3.	Optionally, the MCVideo client 1 may have activated functional alias to be used.

4.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.1.2.3.1.1.4-1: Late entry pre-arranged group call

1.	MCVideo server determines that MCVideo client 1 which is newly affiliated or coming back from out of coverage has to be invited to join an ongoing group call (late entry).

2.	The MCVideo checks the activation status of functional alias of the MCVideo 1. MCVideo server generates group call request including the information such as MCVideo service identifier (possible for the SIP core to route the request to the MCVideo server), MC service group ID of the group invited to join, offer one or more media types and sends towards the MCVideo client 1 via SIP core.

3.	MCVideo user at MCVideo client 1 is notified about the incoming group call.

4.	Upon MCVideo user at MCVideo client 1 accepting the incoming group call request, MCVideo client 1 sends the group call response including the selected media types to the MCVideo server through the signalling path. If the incoming group call request is rejected by the MCVideo client 1, the MCVideo server should not resend the group call request

5.	MCVideo client 1 is successfully added to the ongoing group call and MCVideo users at MCVideo client 1 to MCVideo client n may be notified about the MCVIDEO client 1 joining the group call.

6.	A notification with the information of media transmissions in the group call is sent to MCVideo client1.

# /************************ Next changes *************************/

7.2.3.4.2	Procedure

Figure 7.2.3.4.2-1 describes procedures to establish an off-network automatic commencement private communication between MCVideo user A MCVideo user B.

Pre-conditions:

1.	MCVideo user A has initiated automatic commencement private communication with MCVideo user B.

2.	MCVideo user B has indicated MCVideo client B as the designated MCVideo client for MCVideo private communications.

3.	MCVideo client A and MCVideo client B are members of the same ProSe Discovery group and are ProSe 1:1 direct communication capable.

4.	MCVideo client A has discovered MCVideo client B in proximity, associated with MCVideo user B, using ProSe Discovery procedures.

NOTE:	How MCVideo client A determines that MCVideo client B is the designated MCVideo client of MCVideo user B for private communications is implementation specific.

Figure 7.2.3.4.2-1: Off-network automatic commencement private communication

1.	The MCVideo client A sends the Private communication request towards MCVideo client associated with MCVideo user B. The Private communication request contains an indication for automatic commencement and the SDP offer.

2.	On receiving a Private communication request with an indication for automatic commencement, the MCVideo client checks if it is the designated MCVideo client for off-network MCVideo private communications.

3a.	The designated MCVideo client B automatically accepts the Private communication request, and sends a Private communication answer response, indicating the successful receipt of communication request to the MCVideo client A. The Private communication answer response contains the SDP answer.

3b.	The designated MCVideo client B notifies the MCVideo user B about the incoming Private communication request.

NOTE 1:	Step 3a and step 3b can occur in any order.

4.	The MCVideo client A and the MCVideo client B establish the media plane for Private Communication.

NOTE 2:	If a MCVideo client fails to establish the communication, the MCVideo client should send a Private communication failed response indicating the failure reason to the appropriate MCVideo client.

5.	MCVideo media is transmitted from the MCVideo client A to the MCVideo client B and is presented to the MCVideo user B.

NOTE 3:	If a Private communication failed response is received by a MCVideo client, before or after establishing the media session, if already established, the session is terminated and the MCVideo user is notified about the failure and its reason, if any.

# /************************ Next changes *************************/

7.2.3.5.2	Procedure – Communication accepted

Figure 7.2.3.5.2-1 describes procedures to establish an off-network manual commencement private communication between MCVideo user A MCVideo user B, which is accepted by the MCVideo user B.

Pre-conditions:

1.	MCVideo user A has initiated manual commencement private communication with MCVideo user B.

2.	MCVideo user B has indicated MCVideo client B as the designated MCVideo client for MCVideo private communications, if the MCVideo user B has signed on to the MCVideo service with multiple MCVideo clients.

3.	MCVideo client A and MCVideo client B are members of the same ProSe Discovery group and are ProSe 1:1 direct communication capable.

4.	MCVideo client A has discovered MCVideo clients B in proximity, associated with MCVideo user B, using ProSe Discovery procedures.

NOTE:	How MCVideo client A determines that MCVideo client B is the designated MCVideo client of MCVideo user B for private communications is implementation specific.

Figure 7.2.3.5.2-1: Off-network manual commencement private communication – Accepted

1.	The MCVideo client A sends the Private communication request towards MCVideo client associated with MCVideo user B. The Private communication request contains an indication for manual commencement and the SDP offer.

2.	On receiving a Private communication request, the MCVideo client checks if it is the designated MCVideo client for off-network MCVideo private communications.

3a.	The designated MCVideo client B presents the incoming Private communication request to the MCVideo user B.

3b.	The MCVideo client B sends back a Private communication ringing response to the MCVideo client A.

NOTE 1:	Step 3a and step 3b can occur in any order.

4.	The MCVideo client A notifies the ringing status to the MCVideo user A.

5.	The MCVideo user B accepts the Private communication request.

6.	The MCVideo client B sends the Private communication answer response to the MCVideo client A. The Private communication response contains the SDP answer.

7.	The MCVideo client A and the MCVideo client B establish the media plane for Private communication.

NOTE 2:	If a MCVideo client fails to establish the communication, the MCVideo client should send a Private communication failed response indicating the failure reason to the appropriate MCVideo client.

8.	MCVideo media is transmitted from the MCVideo client A to the MCVideo client B and is presented to the MCVideo user B.

NOTE 3:	If a Private communication failed response is received by a MCVideo client, before or after establishing the media session, if already established, the session is terminated and the MCVideo user is notified about the failure and its reason, if any.

7.2.3.5.3	Procedure – Communication rejected/ignored

Figure 7.2.3.5.3-1 describes procedures to initiate an off-network manual commencement private communication between MCVideo user A MCVideo user B, which is rejected or ignored by the MCVideo user B.

Pre-conditions:

1.	MCVideo user A has initiated manual commencement private communication with MCVideo user B.

2.	MCVideo user B has indicated MCVideo client B as the designated MCVideo client for MCVideo Private Communications, if the MCVideo user B has signed on to the MCVideo service with multiple MCVideo clients.

3.	MCVideo client A and MCVideo client B are members of the same ProSe Discovery group and are ProSe 1:1 direct communication capable.

4.	MCVideo client A has discovered MCVideo clients B in proximity, associated with MCVideo user B, using ProSe Discovery procedures.

NOTE:	How MCVideo client A determines that MCVideo client B is the designated MCVideo client of MCVideo user B for private communications is implementation specific.

Figure 7.2.3.5.3-1: Off-network manual commencement private communication – Rejected or Ignored

1.	The MCVideo client A sends the private communication request towards MCVideo client associated with MCVideo user B. The private communication request contains an indication for manual commencement and the SDP offer.

2.	On receiving a Private communication request, the MCVideo client checks if it is the designated MCVideo client for off-network MCVideo private communications.

3a.	The designated MCVideo client B presents the incoming Private communication request to the MCVideo user B.

3b.	The MCVideo client B sends back a Private communication ringing response to the MCVideo client A.

NOTE 1:	Step 3a and step 3b can occur in any order.

4.	The MCVideo client A notifies the ringing status to the MCVideo user A.

5.	The MCVideo user B rejects (or ignores) the Private communication request.

NOTE 2:	The MCVideo client B determines that the MCVideo user B has ignored the Private communication request, if the MCVideo user B does not respond within a configured time limit.

6.	The MCVideo client B sends a Private communication reject response to the MCVideo client A. The Private communication reject response may indicate that the MCVideo user B has ignored the Private communication request.

7.	The MCVideo client A notifies the MCVideo user A that the Private communication request was rejected (or ignored).

# /************************ Next changes *************************/

7.5.2.4	Retrieve MCVideo capabilities information by the MCVideo client

The procedure for retrieval of MCVideo capabilities information by the MCVideo client is described in figure 7.5.2.4-1. This procedure is applicable to query the MCVideo clients belonging to the same MCVideo system as the requesting MCVideo user.

Pre-conditions:

-	The MCVideo server has received MCVideo capabilities information from MCVideo clients and has stored this information;

-	The MCVideo user is authorized to access the MCVideo capabilities information from the MCVideo server.

-	The requesting MCVideo user is within the same MCVideo system as the requested MCVideo users and the requested MCVideo groups.

Figure 7.5.2.4-1: Retrieve MCVideo capabilities information by the MCVideo client

1.	The MCVideo client requests the MCVideo capabilities information by specifying some criteria (e.g. parameters, MCVideo ID).

2.	The MCVideo server checks whether the user of the MCVideo client is authorized to retrieve the requested MCVideo capabilities information.

3.	If authorized, the MCVideo server provides the requested MCVideo capabilities information to the MCVideo client.

NOTE:	The requesting MCVideo user and the MCVideo users or the affiliated groups to be requested are in different MCVideo systems are not specified in the present document.

# /************************ Next changes *************************/

7.7.2.8	Transmission override (revoke self)

This subclause is applicable in the single arbitrator approach.

NOTE:	Transmission override in the self arbitration approach is implementation specific.

Figure 7.7.2.8-1 describes procedures for transmission participants when an MCVideo client authorized to override, requests for transmission permission.

The MCVideo client has detected presence of a transmission arbitrator e.g., by receiving responses to transmission arbitration control message. The MCVideo client sends a transmission request and waits for a response.

Pre-conditions:

1.	An off-network group communication has been established.

2.	Maximum simultaneous transmissions limit has been reached.

Figure 7.7.2.8-1: Transmission override

1.	MCVideo client 2 sends a transmission request message to the MCVideo group.

2.	As the configured limit of maximum simultaneous transmissions is already reached, MCVideo client 1, being a transmission arbitrator, checks the override policy.

3.	If MCVideo client 2 is authorized to override (based on e.g., transmission priority), MCVideo client 1 may send a transmission revoked message to the MCVideo group. Transmission revoked message indicates the MCVideo client 1, the current arbitrator, whose permission is revoked.

4.	MCVideo client 1 stops transmission of video and MCVideo user at MCVideo client 1 may be notified that the transmission permission has been revoked.

5.	MCVideo client 1 sends a transmission granted message to the MCVideo group. The transmission granted message indicates MCVideo client 2 as the intended recipient and MCVideo client 2 as the subsequent transmission arbitrator.

6a.	MCVideo client 2 sends a transmission arbitration taken message to the MCVideo group.

6b.	MCVideo user at MCVideo client 2 may be notified that the video can now be transmitted.

NOTE:	Step 6a and step 6b can occur in any order.

7.	MCVideo client 1, upon receiving the transmission arbitration taken message releases transmission arbitration.

8.	MCVideo client 2 transmits video to the MCVideo group.

# /************************ Next changes *************************/

7.16.3	MCVideo client query procedure

Figure 7.16.3-1 describes the procedure for MCVideo client query in on-network. This procedure is applicable to query the MCVideo clients belonging to the same MCVideo system as the requesting MCVideo user.

Pre-conditions:

1.	The video capabilities have been collected at MCVideo server during the update MCVideo capabilities information at the MCVideo server procedure in subclause 7.5.2.3.

Figure 7.16.3-1: MCVideo client query procedure

1.	MCVideo client A sends a MCVideo client query request to the MCVideo server with certain search criteria.

2.	MCVideo server checks whether the MCVideo user of MCVideo client A has the authorization to perform MCVideo client query. If success, MCVideo server retrieves the MCVideo clients that fulfils the search criteria. If MCVideo group ID list is included in the MCVideo client query request, MCVideo server retrieves the affiliated group members that fulfil the query criteria in each affiliated MCVideo group in the received MCVideo group ID list.

3.	MCVideo server returns the MCVideo client query response to the MCVideo client A with the retrieved MCVideo clients.

NOTE:	The requesting MCVideo user and the MCVideo users or the affiliated groups to be requested are in different MCVideo systems are not specified in the present document.

# /************************ Next changes *************************/

7.19.2.3	Ad hoc group call request (MCVideo server – MCVideo server)

Table 7.19.2.3-1 describes the information flow ad hoc group call request between the MCVideo servers.

Table 7.19.2.3-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID | M | The MCVideo group ID to be associated with the ad hoc group call |
| MCVideo ID list (see NOTE 1, NOTE 3) | O | MCVideo IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Media parameters of MCVideo server |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCVideo imminent peril call |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCVideo emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: This element is included only when the originating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. |  |  |

# /************************ Next changes *************************/

7.19.2.6	Ad hoc group call response (MCVideo server – MCVideo server)

Table 7.19.2.6-1 describes the information flow ad hoc group call response between the MCVideo servers.

Table 7.19.2.6-1: Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the target MCVideo user |
| Functional alias | O | The functional alias of the target MCVideo user |
| MCVideo ad hoc group ID | M | The MCVideo group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group call request (success or failure) |