3GPP TSG-SA6 Meeting #64	S6-245027

Orlando, USA, 18th - 22nd Oct. 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0532 | rev | 4 | Current version: | 19.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Amendments to location reporting configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HOME OFFICE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Control Rooms and Dispatchers have requested the flexibility to change only the emergency reporting or non-emergerncy reporting segments of the default location reporting configuration with a temporary reporting configuration. For this, each of the emregency and non-emegerncy reporting segments should contian the full set of location reporting IEs. Thus we propose to include two separate IEs for ‘Minimum time between successive reports’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Currently the IEs for location reporting configuration has a common parameter for ‘Minimum time between successive reports’ for both emeregency and non-emergency reporting. This CR proposes to have two separate IEs for this update rate for emeregency and non-emergency reporting. Changes to sections 10.9.2.1 and 10.9.2.4 of TS 23.280. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The flexible and quick processes to change the location reporting configuration by the Control Rooms and Dispatchers of emergency services cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.1 and 10.9.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.9.2.1	Location reporting configuration

Table 10.9.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration.

Table 10.9.2.1-1: Location reporting configuration

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user to which the location reporting configuration is targeted. |
| Requested non-emergency location information | O (see NOTE 1) | Identifies what location information is requested, except for emergency or imminent peril calls or emergency alerts |
| Requested emergency location information | O (see NOTE 1) | Identifies what location information is requested, for emergency or imminent peril calls or emergency alerts |
| Triggering criteria in non- emergency cases | O (see NOTE 1) | Identifies when the location management client will send the location report in non-emergency cases |
| Handling criteria in not reporting location information cases | O | Identifies when the location management client will store location information (e.g. never, off-network, IOPS) |
| Triggering criteria in not reporting location information cases | O (see NOTE 2) | Identifies the causes when the location management client will generate location information |
| Minimum time between consecutive reports for emergency cases | O (see NOTE 1) | Identifies the location update rate for emergency cases. Defaults to 0 if absent and 0 for, imminent peril calls |
| Minimum time between consecutive reports for non-emeregency cases | O (see NOTE 1) | Identifies the location update rate for non-emergency cases. |
| Triggering criteria in emergency cases | O (see NOTE 1) | Identifies when the location management client will send the location report in emergency cases |
| NOTE 1: If none of the information elements is present, this represents a cancellation for location reporting based on Triggering criteria in emergency and non-emergency cases, if configured. NOTE 2: If not present, location information is generated based on Triggering criteria in emergency and non-emergency cases, if configured. |  |  |

* * * Next Change * * * *

#### 10.9.2.4	Location reporting trigger

Table 10.9.2.4-1 describes the information flow from the location management client to the location management server for triggering a location reporting procedure.

Table 10.9.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M (see NOTE 1) | Identity of the requesting authorized MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias | O | Functional alias that corresponds to the requesting MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID | O (see NOTE 1) (see NOTE 3) | Identity of the requested MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias | O (see NOTE 3) | Functional alias that corresponds to the requested MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID). |
| Immediate Report Indicator | O (see NOTE 2) (see NOTE 4) | Indicates whether an immediate location report is required |
| Requested non-emergency location information | O (see NOTE 2) (see NOTE 4) | Identifies what location information is requested, except for emergency or imminent peril calls or emergency alerts |
| Requested emergency location information | O (see NOTE 2) (see NOTE 4) | Identifies what location information is requested, for emergency or imminent peril calls or emergency alerts |
| Triggering criteria in non- emergency cases | O (see NOTE 2) (see NOTE 4) | Identifies when the client will send the location report in non-emergency cases |
| Triggering criteria in emergency cases | O (see NOTE 2) (see NOTE 4) | Identifies when the client will send the location report in emergency cases |
| Minimum time between consecutive reports for emergency cases | O (see NOTE 2) (see NOTE 4) | Identifies the location update rate for emergency cases. Defaults to 0 for imminent peril calls . |
| Minimum time between consecutive reports for non-emergency cases | O (see NOTE 2) (see NOTE 4) | Identifies the location update rate for non-emergency cases. |
| NOTE 1: The identity of the requesting MC service user and the requested MC service user shall belong to the same MC service. E.g. if requesting MC service user is using a MCPTT ID, then the requested MC service user identity shall be an MCPTT ID. NOTE 2: At least one of these rows shall be present. NOTE 3: Either the MC service ID or the functional alias must be present. NOTE 4: The Location reporting trigger provides a temporary location reporting configuration. This configuration could contain IEs for emergency reporting, for non-emergency reporting or for both cases. If this configuration represents only 1 case (emergency reporting or non-emergency reporting) reporting for the other case shall happen as per the relevant parameters in the default configuration. (see Table A.8-1). |  |  |

* * * End of Changes * * * *