3GPP TSG-SA WG6 Meeting #64	S6-245047

Orlando, USA, 18th – 22nd November 2024	(revision of S6-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0622 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Including UE capabilities towards the partner MC system during migration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR updates the existing migration service authorisation procedure to allow the migrated MC service client to share necessary information with the partner MC system. Such information is related to the L4S capability of the MC service UE for L4S support, and its IMSI in case location information are obtained from the PLMN operator. The CR is aligned with the updates done in the agreed CRs S6-244722 and S6-244413 where the capabilities are shared with the MC service server of the primary MC system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Including the IMSI abbreviation Updating step 13 of the procedure in clause 10.6.3.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L4S feature and location reporting from PLMN operator are not supported due to migration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3 and 10.6.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

APN	Access Point Name

BM-SC	Broadcast Multicast Service Centre

CMS	Configuration Management Server

CSC	Common Services Core

CSCF	Call Server Control Function

DPF	Direct Provisioning Function

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EPC	Evolved Packet Core

EPS	Evolved Packet System

FEC	Forward Error Correction

GBR	Guaranteed Bit Rate

GCS AS	Group Communication Service Application Server

GCSE_LTE	Group Communication Service Enabler over LTE

GMS	Group Management Server

GRUU	Globally Routable User agent URI

HSS	Home Subscriber Server

HTTP	Hyper Text Transfer Protocol

I-CSCF	Interrogating CSCF

IARI	IMS Application Reference Identifier

ICE	Interactive Connectivity Establishment

ICSI	IMS Communication Service Identifier

IdMS	Identity Management Server

IM CN	IP Multimedia Core Network

IMPI	IP Multimedia Private Identity

IMPU	IP Multimedia PUblic identity

IMS	IP Multimedia Subsystem

IMSI	International Mobile Subscriber Identity

KMS	Key Management Server

LCS	Location Services

LMC	Location Management Client

LMS	Location Management Server

MBMS	Multimedia Broadcast and Multicast Service

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MC	Mission Critical

MC ID	Mission Critical user identity

MCPTT AS	MCPTT Application Server

MCPTT ID	MCPTT user identity

NAT	Network Address Translation

P-CSCF	Proxy CSCF

PLMN	Public Land Mobile Network

ProSe	Proximity-based Services

PSI	Public Service Identity

QoS	Quality of Service

RAN	Radio Access Network

RF	Radio Frequency

ROHC	RObust Header Compression

S-CSCF	Serving CSCF

SIP	Session Initiated Protocol

SSL	Secure Sockets Layer

TLS	Transport Layer Security

TMGI	Temporary Mobile Group Identity

UDC	User Data Convergence

UDR	User Data Repository

USB	Universal Serial Bus

URI	Uniform Resource Identifier

WLAN	Wireless Local Area Network

For the purposes of the present document, the abbreviations given in 3GPP TS 22.280 [3] apply

MCData

MCPTT

MCVideo

# * * * * Second change * * * *

##### 10.6.3.3.1	Migration service authorization procedure

The procedure for service authorization for migration to a partner MC system and subsequent service authorization to migration partner MC system is shown in figure 10.6.3.3.1-1.

Pre-conditions

-	The MC service user wishes to migrate to the partner MC system.

-	The MC service client has been configured with an MC service user profile that contains the necessary parameters needed for connectivity with the partner MC system.

-	A user authentication process has taken place which has supplied the necessary credentials to the MC service client to permit service authorization to take place in the partner MC system.

Figure 10.6.3.3.1-1: Service authorization for migration to partner MC system and subsequent service authorization

1.	The MC service client requests migration service authorization with the partner MC service server indicating the selected MC service user profile to be used during migrated MC service. The MC service client provides both the MC service ID provided during user authentication in the partner MC system, and the MC service ID of the MC service user in the primary MC system of the MC service user.

NOTE 1:	The migrating MC service client also provides authentication credentials which are specified in 3GPP TS 33.180 [25].

2.	The partner MC service server performs an initial authorization check to verify that the MC service user is permitted to migrate to the partner MC system from the primary MC system of the MC service user.

NOTE 2:	The criteria for the initial authorization check is outside the scope of the present document, but for example could be based on a pre-configured list of users who are expected to request migrated service authorization.

3.	The partner MC service server identifies the primary MC system of the MC service user of the MC service client by use of the MC service ID of the MC service user in the primary MC system of the MC service user, which was presented by the MC service client in step 1, and sends a migration service authorization request to the gateway server in the partner MC system.

4.	The partner MC system gateway server identifies the primary MC system of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to the gateway server of the primary MC system.

5.	The gateway server in the primary MC system of the MC service user identifies the primary MC service server of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to that MC service server.

6.	The primary MC service server of the MC service user performs an authorization check, to verify that migration is permitted to that partner MC system by this MC service user using the indicated MC service user profile.

7.	The primary MC service server marks the MC service user as having migrated, and records the partner MC system as the migrated MC system. The primary MC service server also stores other necessary information related to the migrated MC service user (e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system).

8.	The primary MC service server sends a migration service authorization response to the gateway server in the primary MC system.

9.	The gateway server in the primary MC system sends the migration service authorization response to the gateway server in the partner MC system.

10.	The gateway server in the partner MC system sends the migration service authorization response to the partner MC service server.

11.	The partner MC service server stores the necessary information related to the migrated MC service user (e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system). The migration status of the MC service user allows proper communication redirection back to the primary MC system once the migrated MC service user is no longer migrated on this partner MC system.

NOTE 3:	The stored necessary information both in the primary MC system and partner MC system related to the migrated MC service user (e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system) is used for proper communication redirection only after successful completion of MC service authorization at partner MC system (i.e. migrated MC system).

12.	The partner MC service server sends the migration service authorization response to the MC service client, confirming that successful migration and service authorization has taken place.

NOTE 4:	If topology hiding is not used, the migration service authorization request and migration service authorization response messages are sent between the MC service servers in the primary and partner MC system without the need to be sent via an MC gateway server.

13.	The MC service user successfully completes the MC service authorization with the MC service server within the partner MC system to which the MC service user is migrating into. For L4S capabilities defined in IETF RFC 9330[9330], the MC service client shall include the MC service UE’s capabilities related to L4S towards the partner MC service server. Furthermore, if location information is obtained from the PLMN operator via the LCS, the MC service client shall share the MC service UE’s IMSI towards the partner MC service server.

14.	The partner MC service server of partner MC system notifies the primary MC system of the MC service user of the MC service client by sending the MC service authorization notification.

# * * * * End of change * * * *