3GPP TSG-SA WG6 Meeting #63	S6-245050

Orlando, USA, 18th – 22nd November 2024	(revision of S6-24xxxx)

Source:	Ericsson

Title:	Pseudo-CR on clauses 10.2.3, 10.3.3 and 10.6.1.3.2

Spec:	3GPP TR 23.700-09 v 0.3.0

Agenda item:	8.8

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

This pCR address KI#1 with the focus on clauses 10.2.3, 10.3.3 and 10.6.1.3.2.

2. Reason for Change

This pCR looks into having clauses 10.2.3.10.3.3 and 10.6.1.3.2 of TS 23.180 agnostic to the network.

3. Conclusions

This pCR updates clauses 10.2.3, 10.3.3 and 10.6.1.3.2.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-09 v 0.3.0

* * * First Change * * * *

## 6.x	Solution #X: Clauses 10.2.3, 10.3.3 and 10.6.1.3.2

## 6.x.1	General

This solution addresses key issue #1 with the focus on clauses 10.2.3, 10.3.3 and 10.6.1.3.2 to be written in a generic way to be a generic IOPS solution.

### 6.x.2	Solution

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution.

#### 6.x.2.1	clause 10.2.3

### 10.2.3	Procedure

After an MC service user is authenticated on the IOPS MC connectivity function, the IOPS discovery procedure is initiated by the MC users requesting support of the IP connectivity functionality to the IOPS MC connectivity function. If the IOPS MC connectivity function indicates the support of the IP connectivity functionality, the MC service users can send an IOPS discovery request.

The procedure for requesting support of the IP connectivity functionality by the IOPS connectivity client to the IOPS MC connectivity function is described in figure 10.2.3-1.

NOTE 1:	The procedure for requesting support of the IP connectivity functionality is only required when the IOPS connectivity client does not contain information about the support of this functionality by the serving IOPS MC connectivity function.

Pre-conditions:

-	The MC service user is authenticated on the IOPS MC connectivity function.

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The IOPS connectivity client does not contain information about the support of the IP connectivity functionality by the serving IOPS MC connectivity function

Figure 10.2.3-1 IP connectivity functionality request in the IOPS mode of operation

1.	The IOPS connectivity client requests to the IOPS MC connectivity function the support of the IP connectivity functionality.

2.	The IOPS MC connectivity function indicates to the IOPS connectivity client if the IP connectivity functionality is supported or not for the MC user.

The procedure for the discovery of MC users in the IOPS mode of operation is described in figure 10.2.3-2. The IOPS discovery is initiated by the MC users to support MC services based on the IP connectivity functionality.

As part of the IOPS discovery procedure, an MC user can request higher priority from the IOPS MC connectivity function by including a priority state indication within the IOPS discovery request. Based on this indication, the IOPS MC connectivity function can decide to handle with higher priority the transactions related to the MC user, e.g. the distribution of packets related to any MC communication from or to the corresponding MC user, subscriptions, and notifications. When an MC user intends to establish an IOPS emergency (group or private) call, the MC user can request a priority state from the IOPS MC connectivity function.

The MC user's priority state remains unchanged until the MC user explicitly changes it (i.e. by sending a new IOPS discovery request) or the discovery status of the MC user changes to not-discovered on the IOPS MC connectivity function.

Pre-conditions:

-	The MC service user is authenticated on the IOPS MC connectivity function.

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The IOPS MC connectivity function has indicated to the IOPS connectivity client the support of the IP connectivity functionality.

Figure 10.2.3-2 User discovery in the IOPS mode of operation

1.	The MC user sends an IOPS discovery request to the IOPS MC connectivity function. The request includes providing connectivity information for the support of MC services based on the IP connectivity functionality.

2.	The IOPS MC connectivity function stores the information received from the MC user and registers the user's connectivity status as discovered.

3.	The IOPS MC connectivity function provides a response to the IOPS connectivity client indicating the success or failure of the discovery of the requesting MC user.

NOTE 2:	The MC user shall send a new IOPS discovery request to update or modify information elements.

NOTE 3:	The IOPS MC connectivity function shall send a new IOPS discovery response to the IOPS connectivity client for the case that the periodic IOPS discovery request has not been received yet or when the IOPS discovery request periodicity needs to be changed. The IOPS MC connectivity function can verify the availability and IP connectivity information of the discovered MC user within the IOPS MC system based on the reception of periodic IOPS discovery requests.

#### 6.x.2.2	Clause 10.3.3

### 10.3.3	Procedures

The procedure for the IP connectivity subscription of MC users in the IOPS mode of operation is described in figure 10.3.3-1.

Pre-conditions:

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The MC service user is discovered by the IOPS MC connectivity function to support MC services based on the IP connectivity functionality.

-	The MC service user has decided to subscribe to receive connectivity information of other discovered MC users and/or IOPS groups on the IOPS MC connectivity function.

Figure 10.3.3-1 IP connectivity subscription in the IOPS mode of operation

1.	The IOPS connectivity client subscribes to receive connectivity information of other discovered MC users and/or groups on the IOPS MC connectivity function.

2.	The IOPS MC connectivity function checks if a subscription for the MC service ID(s) and/or IOPS group(s) included in the request can be activated for the requesting MC user.

NOTE:	A subscription cannot be activated if an MC user associated to the requested MC service ID has indicated in its IOPS discovery request that its connectivity information is not available to other users on the IOPS MC connectivity function. Besides, a subscription may not be activated if a requested IOPS group does not belong to the list of IOPS groups the requesting MC user has included in its IOPS discovery request.

3.	The IOPS MC connectivity function indicates to the IOPS connectivity client the subscription status associated to the request.

The procedure for the IP connectivity notification of MC users in the IOPS mode of operation is described in figure 10.3.3-2.

Pre-conditions:

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The MC service user has an active IP connectivity subscription associated to one or multiple MC service IDs.

-	The IOPS MC connectivity function has available information related to an IP connectivity subscription associated to one or multiple MC service IDs.

Figure 10.3.3-2 IP connectivity notification in the IOPS mode of operation

1.	The IOPS MC connectivity function notifies to the IOPS connectivity client the connectivity information related to an active IP connectivity subscription associated to one or multiple MC service IDs. The connectivity information can be retrieved by the MC service client to establish an MC service communication based on the IP connectivity functionality.

2.	The IOPS connectivity client provides an IP connectivity notify response to the IOPS MC connectivity function.

The procedure for the IP connectivity group notification of MC users in the IOPS mode of operation is described in figure 10.3.3-3.

Pre-conditions:

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The MC service user has an active IP connectivity subscription associated to one or multiple IOPS MC service group IDs.

-	The IOPS MC connectivity function has available information related to an IP connectivity subscription associated to one or multiple IOPS MC service group IDs.

Figure 10.3.3-3 IP connectivity group notification in the IOPS mode of operation

1.	The IOPS MC connectivity function notifies to the IOPS connectivity client the group connectivity information related to an active IP connectivity group subscription associated to one or multiple IOPS MC service group IDs. The group connectivity information can be retrieved by the MC service client to establish an MC service group communication based on the IP connectivity functionality.

2.	The IOPS connectivity client provides an IP connectivity group notify response to the IOPS MC connectivity function.

#### 6.x.2.3	Clause 10.6.1.3.2

##### 10.6.1.3.2	Procedure

The procedure in figure 10.6.1.3.2-1 describes the case where an MCData user is initiating an IOPS one-to-one MCData communication for sending standalone SDS data over signalling control plane to another MCData user, with or without disposition request. Standalone refers to sending unidirectional data in one transaction.

Pre-conditions:

-	MCData user profile used for the IOPS mode of operation is pre-provisioned in the MCData UEs.

-	MCData users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality.

-	The MCData users are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality.

-	MCData clients have retrieved connectivity information from target MCData users.

Figure 10.6.1.3.2-1: IOPS one-to-one standalone SDS using signalling control plane based on the IP connectivity functionality

1.	The MCData user at MCData client 1 would like to initiate an IOPS SDS data transfer with the MCData user at MCData client 2 based on the IP connectivity functionality. The MCData client 1 checks whether the MCData user 1 is authorized to send an IOPS MCData standalone data request.

2.	The MCData client 1 retrieves the connectivity information of the target MCData user from the IOPS connectivity client 1 (not shown in figure) and sends an IOPS MCData standalone data request towards the MCData client 2. The MCData client 1 transmits the session packets carrying the IOPS MCData standalone data request to the IOPS MC connectivity function for distribution to the corresponding target MCData UE 2's IP address.

The IOPS MCData standalone data request contains the data payload, i.e. the SDS content. The request also contains a conversation identifier for message thread indication and may contain a disposition request if indicated by the user at MCData client 1.

3.	The IOPS MC connectivity function receives the session packets addressing the MCData UE 2's IP address. The IOPS MC connectivity function checks if the MCData UE 2's IP address corresponds to a discovered MC user in order to distribute the received session packets. If it does, the IOPS MC connectivity function distributes the received session packets to the target MCData client over unicast transmissions.

4.	The IOPS MC connectivity function distributes the session packets carrying the IOPS MCData standalone data request to the MCData client 2.

5.	Upon the receipt of the IOPS MCData standalone data request, the MCData client 2 checks whether any policy is to be asserted to limit certain types of message or content to certain members due to, for example, location or user privilege. If the policy assertion is positive and the payload is for MCData user consumption (e.g. it is not application data, not command instructions, etc.) then the MCData client 2 notifies the target MCData user.

The actions taken when the payload contains application data or command instructions are based on the payload content. Payload content received by MCData client 2 which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client 2 to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

NOTE:	If the policy assertion was negative, the MCData client 2 sends an appropriate notification to MCData client 1.

6.	If MCData data disposition was indicated (for delivery, read or both) within the request sent by the MCData client 1, then the receiving MCData client 2 initiates the corresponding IOPS MCData data disposition notification(s) towards the MCData client 1, i.e. addressing the MCData UE 1's IP address.

7.	The IOPS MC connectivity function receives the session packets addressing the MCData UE 1's IP address. The IOPS MC connectivity function checks if the MCData UE 1's IP address corresponds to a discovered MC user in order to distribute the received session packets. If it does, the IOPS MC connectivity function distributes the received session packets to the target MCData client over unicast transmissions.

8.	The IOPS MC connectivity function distributes the session packets carrying the IOPS MCData data disposition notification to the MCData client 1.

### 6.x.4	Other affected clauses

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.x.5	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 into an access generic IOPS solution.

* * * End of Changes * * * *